DMP220 -- Graphics Dump to CGP-220

Copyright 1983  Michael M Rubenstein

DMP220 dumps the TRS-80 Model 100

screen, complete with graphics, to a

TRS-80 CGP-220 printer.  DMP220 is

designed to be incorporated in BASIC

programs.

Two versions are available -- a BASIC

version and a machine language version.

Including BASIC Version in Programs

A BASIC program using DMP220 must

1.	not have any line numbers

greater than 64999

2.	not use the variables HH%, II%,

JJ%, KK%, or LL%

DMP220 must be an ASCII file in RAM,

on cassette (CSAVEd with the "A"

option), or read in from the RS232 or

modem interface.  To combine with your

BASIC program, use one of the commands

MERGE "DMP220"

MERGE "CAS:DMP220"

MERGE "COM:xxxxE"  (xxxx is the

RS232 spec)

MERGE "MDM:xxxE"   (xxx is the

modem spec)

as appropriate.

Using BASIC version

The initialization routine must be

executed exactly once, before any

use of the screen print routine, by

GOSUB 65000

To print the screen, use the statement

GOSUB 65100

Installing Machine Language Version

The machine language version is

distributed in hex format.  To install

it, first load DMP220.HEX onto the

model 100 (as a .DO file).  Then run

HEXLDR.  Respond to the request for

file name with

DMP220

There will be a pause for several

seconds while HEXLDR creates the

DMP220.CO.

Using Machine Language Version

To use the machine language version,

you must first reserve space for it

with the statement

CLEAR 256,61936

(of course the first number is the

string space required and may need to

be changed for your application.)

Then load the .CO file with

LOADM "DMP220"

Now at any point, the screen may be

printed by

CALL 61936

When done, the space used by DMP220

may be recovered by

CLEAR 256,MAXRAM

<End of Document>