******Documentation for 'disk.100'******       copyright Dieter F. Barth

free usable for all m100sig members

not for commercial use

What it does :

Disk.100 transferes .do- and .ba- files from and to a desktop computer

via rs 232 using the utility program 'pip' in the cp/m dos.

All keyboard input and all display including file directory of disk is

done on m100.

Disk.100 not only transferes datas it also lets you operate the desktop

from m100.

Disk.100 resides as .ba file on 2,777 bytes and uses an other 1145

bytes if it runs.

How it works :

Disk.100 is used since about six months together with a 'AMPRO little

board computer' using 'pip' of cp/m dos without any problems.

Disk.100 should also work with any other computer using cp/m dos.

It should be possible to modify Disk.100 for use with ms dos after changing

the specific comands in lines 35,39,53,62 and 69.

Disk.100 transfers with the 19,200 baud if your desktop can't handle that

change variable d in line 1.

How to install :

Load Disk.100 and save it as

DISK.BA

DON'T USE AN OTHER NAME IT WILL NOT WORK!

Each diskette you want to use with Disk.100 MUST contain the cp/m system

and the cp/m utility 'pip'.

If you have a coldstart program you can define one of the function keys

with

"Disk"+chr$(24)+"run"+chr$(34)+"disk"+chr$(13)

and you will have access to the

program with ONE keystroke.

You also should open a file with the name 'year' containing '86'.

Functions :

- start up

After start up Disk.100 shows the program title on the screen and tries

to load 'pip' in the desktop.

If not already done now is the time to turn on your desktop and feed it

with a diskette.

After loading 'pip' the screen clears and you will see '*' in the upper

corner of your screen and there will be a new definition of the m100

function keys.

From now on your desktop will accept all 'pip' comands from m100. You

can leave 'pip' just by pushing <cr> and the system will return with 'A>'

on the m100 screen and cp/m of your desktop is waiting for cp/m comands.

If you read a file and want to stop the display just push 'break'.

Push 'break' again and the display will continue.

IMPORTANT:

Disk.100 can't be stopped with ctrlc or shift break.

If something goes wrong the system jumps to an 'error trap' showing error

and error line with a terrible sound.

If the program should hang up then push the 'reset button' ONCE.

You will be back in the main menu without coldstart.

- f1 = prev

f1 acts like 'prev' in the TELCOM program and shows you the previous 8

lines of the screen.

- f2 = down

This routine loads a .txt file from disk as .do file to m100 and starts

displaying all .txt files of the diskette on the m100 screen and

finishes display with

'filename: ?'

Input the filename WITHOUT extention '.txt' and the system answers with

'downloading...'

The disk file display can be finished anytime by pushing any key. This

might be useful if there are lots of files on disk but you already know

the name of the file you want to download.

During the downloading a counter in the lower right corner shows you

something is happening.

After downloading the .txt file as .do file in m100 the system returns

again with

'filename :?'

If there is no other file to load then just push <cr> that brings you

back to disk.100 menu.

- f3 = up

This routine transferes a .do file of m100 as .txt file to disk and

starts with displaying all m100 .do files in a simular way as menu does.

The file you want to upload is selected by using the arrow keys.

Each selected file displays the name and file length.

Push <cr> and the system will answer with 'uploading...' and the counter

in the right lower corner tells you something is happening.

After the operation is finished the system lets you select an other

file to upload.

Pushing 'esc' brings you back to the disk.100 menu.

IMPORTANT !

If you want to transfer one file to different disks you can do that but

you have to send a ctrlc to the desktop after changing the disk.

-f4 = Cdsk

This routine lets you upload cas: files directly to disk.

You can prepare a file called 'casfil' with the name of the cas: file in

each line.

Disk.100 will read this file and upload all files mentioned in 'casfil'

automaticly.

If there is no 'casfil' disk.100 will answer with

'filename :?'

and ask you for input.

For leaving this routine just push <cr> without putting in a name.

-f5 = echo

This function key acts like ECHO in TELCOM and everything on the m100

screen will also be printed.

-f6 = load

This routine loads a .bas file on disk as .ba file to m100 and starts

with a display of all .bas files available on disk.

'Load' acts like 'down' but you wan't see a running counter and the

transfer seems to be slow because of the conversion from ASCII to basic.

After downloading the system saves the file as .ba file and returns with

the disk.100 menu.

-f7 = save

This routine loads a .ba file from m100 to disk as .bas file and starts

with a display of all .ba files in m100.

Files are selected by using the arrow keys and the screen shows the

length of the selected file.

<cr> starts uploading the file and 'esc' lets you leave the routine without

transfer. After transfer the system returns with the disk.100 menu.

-f8 = menu

Pushing f8 brings you back to m100 main menu setting maxfiles=1 and

stringspace=256. It also makes sure the year in date$ is correct.