DOCS FOR "OPTEST.CO"

==========================

Dave Newman  [76656,2526]

DaMN Software, Uninc.

946 South 12th St.

Lincoln, NE  68508

(402) 477-0347

--------------------------

OPTEST.CO is a machine language program written for the purpose of

testing your Model 100's execution of the 80C85's "illegal"

instructions.  The mnemonics I have used are very similar to the ones

suggested by Ken Barbier ("The secret life of the 8085",

Microcomputing, Sept. 1980), with some minor changes to conform to my

assembler's format.  If you would like more information on these

"illegal" instructions, read that article or the file called

"OPCODE.DOC" from DL8.  However, you need not know how they work to run

this program.

I am interested in the results that you get, as I am trying to decide

whether or not to "market" several programs written using these

undocumented instructions.  Please let me know by MAIL or message how

they worked for you.

LOADING AND SAVING "OPTEST.CO"

The code resides at 54000 to 54988 decimal (D2F0 to D6CC hex) to allow

for any other ML program you may have in memory.  If everything works,

nothing higher than that will be affected.  If your machine chokes on

these instructions, then nothing will save your files!  Be sure you

have anything that you might want to save backed up somehow....

The program is saved as an ASCII/hex file using Gord MacSwain's DOCO.BA

program from DL7, so you will need that program to convert it to

machine code.

To create "OPTEST.CO" from the hex DOcument file, simply load the .DO

file, go to BASIC and type 'CLEAR 256,54000.'  Then run DOCO.BA,

specifying "OPTEST.DO" (or whatever you called it) as the input .DO

file, and "OPTEST" as the .CO output file.  DOCO.BA creates the ML

program and saves it as a .CO file, acessible from MENU.

At this point, I highly recommend that you save the .CO file to disk or

cassette by typing 'SAVEM"x:OPTEST",54000,54988,54000', where "x" is

the device designation (CAS:, 0:, 1:, etc).  Once you have the .CO file

saved you can KILL the hex file that it was created from.

1

RUNNING "OPTEST.CO"

To run OPTEST.CO either go to BASIC and type 'CALL 54000' or run it

from MENU if you have it saved as a .CO file.  When you start the

program you will see a screen that looks like Fig. 1.  You can exit to

MENU at any time by pressing <ESC>.

------------------------------------------

[                                        ]

[ 1) 08    DSUB B     6) D9      SHLX    ]

[ 2) 10    RHR        7) DD yyxx JND     ]

[ 3) 18    RDL        8) ED      LHLX    ]

[ 4) 28 xx DMOV D,H   9) FD yyxx JD      ]

[ 5) 38 xx DMOV D,SP                     ]

[                                        ]

[Press 1 thru 9 to test, <ESC> to quit._ ]

------------------------------------------

Fig. 1.  Entry menu of OPTEST.CO

By pressing a number (1 through 9) you call a routine that initiates

the registers for whichever instruction you selected.  Then a routine

is called that displays the contents of all the registers and the

instruction that is being tested.  For example, the screen for number

1, the DSUB B instruction, should look like Fig. 2.

------------------------------------------

[                                        ]

[     SZDA-PVC  AC       BC      DE      ]

[     00000000  00      0001    0000     ]

[                                        ]

[        HL      SP      PC              ]

[       0000    CEBC     D3E0            ]

[                                        ]

[ DSUB B         Hit any key to test.._  ]

------------------------------------------

Fig. 2.  Registers before execution.

The SP (stack pointer) register may be slightly different, but all

other registers should look like the example shown.  When you press any

key other than the <ESC> key, the program will restore the registers

and return to the selected test routine.  There it will attempt to

execute one instruction, usually the instruction being tested, and call

the register display routine again.  In the case of the DSUB B

instruction the screen should look like Fig. 3.

Now simply compare your results with the results I got, which are

listed in Fig. 4.  For some instructions, the next keystroke will

return you to the entry menu; others require two keystrokes to test.

As always, the <ESC> key is used to exit to MENU.

2

------------------------------------------

[                                        ]

[     SZDA-PVC  AC       BC      DE      ]

[     10100101  00      0001    0000     ]

[                                        ]

[        HL      SP      PC              ]

[       FFFF    CEBC     D3E4            ]

[                                        ]

[ DSUB B         Please verify results._ ]

------------------------------------------

Fig. 3.  Registers after execution.

If your results are radically different than those listed, then your

80C85 probably doesn't work like the one in my machine.  Of course,

your machine may decide to take off on its own; if your micro does

something different, a cold start is usually inevitable.  In the words

of Richard Adams, DON'T PANIC!!  If you saved all your files, etc., the

worst that can happen is that you'll have to reload the program to try

the next instruction.

Since the purpose of running this program is solely to compare your

micro's execution of these instructions with the results obtained by

several other people, please try all the instructions before throwing

up your hands in despair, even if they all bomb.  Negative results are

just as important as positive results, if not more.

EXPECTED RESULTS

Here is a listing of the results I obtained, which are consistent with

the results obtained by other researchers.  The JD and JND instructions

require one extra keystroke, since those routines will increment or

decrement the DE register pair before returning to the display routine

the first time.  Pay particular attention to the D flag (the

dual-register carry flag, bit 5 of the flag byte); this bit should have

toggled from the state it was in beforehand.  Assuming that this

happened, your next keystroke should affect nothing but the PC (program

counter) register pair.  RHR and RDL also require two keystrokes to

verify that the respective bit is shifted into the Carry flag.

In the interest of conserving space and connection time, only the

results are listed here.  To help you identify the results, an asterisk

(*) is placed beside the register that should have a different value

from the original.

1) 08    DSUB B

PSW    AC   BC    DE    HL    SP    PC

*10100101 00  0001  0000 *FFFF  CEBC *D3E4

The contents of BC are subtracted from the contents of HL, results

stored in HL.  ALL flags are affected.

3

2) 10    RHR

PSW    AC   BC    DE    HL    SP    PC

*00000001 00  0000  0000 *C000  CEBC *D407

*00000000 00  0000  0000 *E000  CEBC *D40B

Each time, the contents of HL are rotated right one bit.  Lo bit goes

into the Carry flag, the Hi bit remains the same.

3) 18    RDL

PSW    AC   BC    DE    HL    SP    PC

*00000010 00  0000 *8801  0000  CEBC *D42F

*00000011 00  0000 *1002  0000  CEBC *D433

Each time, the contents of DE are rotated left one bit.  The Hi bit is

copied into the Carry flag, while the old Carry is copied to the Lo bit

of DE.  Why did the V flag (bit 1) change?  Who knows??

4) 28 xx DMOV D,H

PSW    AC   BC    DE    HL    SP    PC

00000000 00  0000 *1200  11FF  CEBC *D457

The sum of the contents of HL plus the byte following the instruction

are copied into DE.  No flags are affected.

5) 38 xx DMOV D,SP

PSW    AC   BC    DE    HL    SP    PC

00000000 00  0000 *CEC6  0000  CEBC *D47B

The sum of the contents of SP plus the byte following the instruction

are copied into DE.  No flags affected.

6) D9      SHLX

No change (except PC, of course).  The message "Ok" (without the

quotes) should be displayed on the screen to the immediate right of the

PC value.  This represents the 2 ASCII character values in HL being

stored in the location pointed to by DE, which is the end of the

message displayed to the LCD.

7) DD yyxx JND

PSW    AC   BC    DE    HL    SP    PC

*00000000 00  0000 *0100  0000  CEBC *D4C6

After one keystroke, DE is incremented by one.  Since this does not

result in a dual-register carry, the D flag is cleared.  No other

registers affected.

4

00000000 00  0000  0100  0000  CBEC *D4EE

After two keystrokes, program execution branches to D4EB due to the JND

instruction.

8) ED      LHLX

PSW    AC   BC    DE    HL    SP    PC

00000000 00  0000  0000 *33C3  CBEC *D4EE

The first two bytes of ROM at location 0000, as pointed to by DE, are

loaded into HL.  If you get a different value, check the first two

bytes in your ROM against your results.

9) FD yyxx JD

PSW    AC   BC    DE    HL    SP    PC

*00100000 00  0000 *FFFF  0000  CBEC *D511

After one keystroke, DE is decremented by one.  This does result in a

dual-register carry, so the D flag is set.

00100000 00  0000  FFFF  0000  CBEC *D4EE

After two keystrokes, program execution branches to D4E3 due to the JD

instruction.

The only unspecified instruction left is RST V (CB), a software

interrupt which causes program execution to branch to location 0040 if

the V flag (two's complement overflow, bit 1) is set.  Since location

0040 is in ROM, and the only thing there is a BASIC lookup table, this

is a particularly useless instruction for Model 100 users.  Try it if

you like, but even proper execution will only send your Model 100 into

never-never land.

5

NOTES FOR PROGRAMMERS AND TINKERERS

Here is a listing of the important subroutines, ROM calls, etc. for

those of you who are interested in such stuff.  If you would like to

try some different things with these instructions, I have left 8 bytes

empty (NOP's) after each instruction's subroutine.  This allows

additional code to be added if you want to try something else with

these "illegal" instructions.

Program Subroutines, Etc.

D2F0 to D327:  Entry and main menu routine.

D328 to D339:  Address table for test subroutines,

in numerical order.

D33A to D3C9:  Register display and number conversion routines.

D3CA to D51F:  Test subroutines.

D520        :  Message pointer stash.

D522        :  SP value stash.

D524 to D6CC:  Messages and screen displays.

ROM Calls, Etc.

12CB:  Wait for a character from keyboard.

423F:  Lock display (no scrolling).

5797:  MENU routine.

5A58:  Print message at HL until a 0 is found.

5A79:  Clear function key definitions.

F638:  Device flag; 00 = LCD output.

F67A:  Current BASIC line number.  Anything but FFFF here

temporarily disables the Auto-off feature.

6