RTTY.100 is a (mostly) BASIC program to allow the RS Model 100

to transmit and receive Baudot 5 level RTTY code.  Speeds

of 60, 66, 75, and 100 WPM are supported.  Both received and

transmitted text can be echoed to a printer, and the RTS control

line can be used for transmitter control.

This program uses the RS-232 port, and requires an external TU

(terminal unit) to convert the receiver audio into RS-232 levels,

and from RS-232 levels back into tones for transmission.  I am

using a Flesher Corp. TU-300, but any TU with RS-232 compatible

signal levels should work.  Note that the TU-300 does NOT support

RS-232 xmit control - external circuitry is required.

Program Operation:

The program begins in receive mode with echo to printer disabled,

at 60 WPM.  Function keys F1 thru F4 set the speed to 60, 66, 75,

and 100 WPM respectively.  F5 toggles in and out of xmit, F6 sets

letters mode and starts a new line, F7 toggles the echo to printer

mode, and F8 returns to Menu.

About thirty characters are buffered in receive mode, and will be

transmitted as soon as transmit is selected.  If any character is

typed which does not exist in Baudot, you get beeped, and nothing

is sent.  Letters may be typed in upper or lower case - they are

converted to upper case before transmission, but are echoed in

lower case (also to the printer, if enabled).  This could be useful

to differentiate received from transmitted text on a printout.

If I understood more about the LABEL line, I would allow the

LABEL key to toggle the last line with a description of the

function keys.  Any takers??

Machine Language Calls

Several M/L calls are used in this program.  For those interested

who do not have access to references, they are listed below.

16969 (4249H) - Turns on cursor

19268 (4B44H) - Print the character in A reg on LCD

27967 (6D3FH) - Print the character in A reg on LST

28210 (6E32H) - Send character in A reg to RS-232 port

In addition, a machine language routine is read into array ML.  The

entry point at ML(0) checks for received characters - if none are

present, it returns with X(0) = 0.  If a character has been received,

X(0)=1, and the character is in X(1).  The entry at ML(9) converts

the character in the A reg to upper case, and places it in the

variable pointed to by HL.  This allows transmitted text to be typed

in lower case and automatically converted to upper case (Baudot has

no lower case).

Steve Alverson WB8HRV 75236,341