Documentation for XMODEM, for TRS-80 Model 100

Version of 1/23/85  by  Rick Perry

This program performs auto-dial-redial, auto-logon, terminal

emulation, and buffered text file transfers (.DO) using either regular

ASCII Xon/Xoff protocol or the Xmodem Christensen checksum protocol.

Communication parameters are 8N1D during Xmodem file transfers and 7I1E

for terminal emulation and Xon/Xoff transfers. The program will operate

using the 300 baud internal modem or an external modem (Novation

Smart-Cat or equivalent) at 300 or 1200 baud. Files may be Up/Down

loaded directly from/to RAM, disk or cassette.

AUTO-DIAL/LOGON:

The program starts with a "Find:" prompt and will attempt to retrieve

a phone number and auto-logon sequence from the ADRS.DO file and logon

to a host computer.

When using the internal modem, if no carrier is detected within 20

seconds after dialing, or if you press any key, the program will hang up

the phone. After waiting 20 seconds more, it will retry the number

unless you press any key.

If the carrier is detected, it will BEEP and proceed through the

auto-logon sequence as specified in the ADRS line, then jump to the

terminal emulation mode of the program.

At the "Find:" prompt, enter a string to search for in the ADRS file

or press F8 (or enter "Menu") to return to the Menu. When the string is

found press "M" to begin auto-dial using the internal modem, or press

"S" to dial through an external Novation Smart-Cat modem at 1200 baud,

or press "3" to use Novation at 300 baud, or press ESC to return to the

"Find:" prompt, or press any other key (like ENTER) to continue

searching.

If you wish to manually dial a telephone number, enter "Dial" at the

"Find:" prompt. After dialing and hearing the carrier tone, press "M",

"S", or "3" to allow the program to Connect, or press any other key to

return to the "Find:" prompt.

For a Novation Smart-Cat modem, "3" actually sets the modem baud rate

to 300, "S" sets it to 1200, and the modem command character is set to

CTRL/V in both cases.  The modem is also set for 8-bit data format and

"unlisten" mode to allow Xmodem transfers.  Note: modem baud rates, cmd

char, etc. are not affected when using "DIAL" option at Find: prompt.

The control sequences used with the Novation modem appear around the

last twelve lines of the program. This code may be modified to work

properly with other smart modems. When dialing through an external

modem, code around the last eight lines of the program allow you to

directly enter commands to the modem and optionally press CTRL/R to

restart the program or CTRL/E to exit to the Menu.

TERMINAL EMULATION:

Once in terminal emulation mode, the program is transparent to all

control characters entered from the keyboard (except CTRL/G as noted

below) and these characters (like CTRL/C/S/Q) will be sent to the host.

But Function key F5 (Brk) may be used to STOP the program if needed.

F4 (Term) is used to jump directly to TELCOM's Term mode while

remaining connected to the host. Once in TELCOM you cannot return to the

program without logging off and disconnecting from the host unless you

are connected through an external modem.

F6 (Free) provides a display of how many bytes of RAM are currently

available.

F7 (Dir) lists the RAM file directory in the same way as the BASIC

FILES command.

F8 (Bye) is used to exit the program and return to the Model 100's

Main Menu.

F2 (Down) will initiate a file download transfer and F3 (Up) will

initiate uploading.

F1 (Prev) will display the previous screen, same as in TELCOM.

FILE TRANSFERS:

After pressing F2 or F3 and entering file name, program prompts

"Append?" if downloading.  Then it prompts "Use XMODEM protocol?" --

press "Y" or "N".  Responding with "N" means that buffered Xon/Xoff

protocol will be used.

For uploading using Xon/Xoff protocol answer "Width?" prompt by

pressing ENTER if no wrapping is desired, otherwise enter desired width

for word wrapping.

During Xon/Xoff download, F3 is blanked and F2=@@@@.  Press F2 to end

download.  During upload, use F8 to abort.

DOWNLOADING WITH XMODEM PROTOCOL:

The host should be ready to send the file before entering this mode.

While downloading, the received file is not displayed on the screen.

When a NAK (negative acknowledge) is sent, the program prints an "X".

When ACK (acknowledge) is sent, the program prints ".".

You can abort the download at any time by pressing F8 (Abrt). The

program itself will automatically abort the download only if one of the

following circumstances occur:

- The host sends a CANcel.

- The received block number is neither the current block number nor the

previous block number.

- An error trap occurs in the program (this could be caused by an "Out

of Memory" condition on the Model 100).

UPLOADING WITH XMODEM PROTOCOL:

The host should be ready to receive the file before entering this

mode. While uploading, the transmitted file is not displayed on the

screen. When a NAK is received, the program prints an "X". When ACK is

received, the program prints ".". If nothing is received after waiting

10 seconds, or if anything other than NAK, ACK or CAN is received, the

program prints a "?".

You can abort the upload at any time by pressing F8 (Abrt). The

program itself will automatically abort the upload only if the host

sends a CANcel or if an error trap occurs (e.g. due to an IO Error when

uploading from cassette).

ALTERNATE CHARACTER SET:

When in terminal mode, press CTRL/G to switch between the regular

character set and an alternate set (pseudo APL). This character

translation is defined in an array, T(*), which is loaded from the last

data statement.  This only affects characters printing on the screen,

not file transfers.

PROGRAM NOTES:

The program itself does not count the number of NAK's sent or

received per block and will not automatically abort if an excessive

number of NAK's occur. It is up to you to use F8 (Abrt) to cancel the

file transfer if you feel that too many NAK's are occuring.

When aborting a file transfer the program sends a single CANcel

character (CTRL/X) to the host and returns to its terminal emulation

mode. If you then notice that the host has not responded to the CANcel,

you may send additional CAN's by pressing CTRL/X. Some hosts expect to

receive three CTRL/C chars in a row to signal an abort; that's dumb, but

anyway, if CTRL/X doesn't work, try that.  Most hosts will automatically

abort an Xmodem transfer if no chars are received for 1 or 2 minutes.

MAXFILES is set to 3 at the beginning and reset to 2 upon normal exit

to the Menu.

Function key definitions are restored to their previous user-defined

values upon exit from the program to the Menu.

The following ROM calls and POKEs are used:

6118 - set serial interface parameters and activate modem.

16969 - turn cursor on.

17061 - set and display temporary function key strings.

17875 - fill previous screen with blanks.

21172 - keep phone off hook.

21179 - hang up phone.

21264 - 2 second delay.

21274 - 1/2 second delay.

21293 - dial a phone number.

21392 - execute auto-logon sequence.

21608 - jump to TELCOM's Term mode.

21795 - display previous screen.

27804 - restore function key definitions.

POKE 63056,64 - enable scrolling into previous screen area and also

disables Break and ON KEY and makes F1...F8 return chars 248...255.

POKE -86,0 - purge keyboard buffer.

PERSONAL NOTE:

This program is customized for my own use, so some of its features

may not be useful to you.  In particular, note that it performs CLEAR

500, MAXRAM and leaves MAXFILES=2.  Feel free to revise the program to

your own liking and share it here and/or elsewhere.

MACHINE LANGUAGE SUBROUTINES:

The integer array M(0)..M(60) is used to hold six position

independent machine language subroutines. Entry points are at M(0),

M(3), M(16), M(21), M(24) & M(59). The assembly source listing follows:

; ENTRY M(0) - check MDM: for carrier detect.

;  calling sequence: CALL VARPTR(M(0)),0,VARPTR(X)

;  where X is an integer variable.

M(0)	call	6EEF	; check for carrier detect

mov	m,a	; X=0 if carrier detected,

ret		; FF if no carrier

nop

; ENTRY M(3) - convert string to uppercase.

;  calling sequence: CALL VARPTR(M(3)),0,VARPTR(A$)

;  where A$ is a string variable.

M(3)	mov	a,m	; check length of string

ora	a	; if zero

rz		;  then return

mov	c,a	; save length in counter C

inx	h	; get string address

mov	a,m	; that's the low byte

inx	h

mov	h,m	; that's the high byte

mov	l,a	; HL --> string

lxi	d,2	; here comes the tricky part

call	31E9	; in ROM there is XTHL, PCHL

$PC1	dad	d	; now HL = loop

pop	d	; and DE --> string

loop	ldax	d	; A = char from string

call	0FE9	; let ROM convert to uppercase

stax	d	; restore converted char

inx	d	; point to next char

dcr	c	; decrement counter

rz		; done when counter = 0

pchl		; otherwise jump to loop

; ENTRY M(16) - check MDM: or COM: queue for received chars.

;  calling sequence: CALL VARPTR(M(16)),0,VARPTR(X(0))

;  where X(0)..X(1) is an integer array.

M(16)	mvi	m,0	; set flag, X(0)=0, for no char

call	6D6D	; check queue for chars

rz		; return if no chars available

inr	m	; set flag, X(0)=1, for char received

inx	h

inx	h	; HL --> received char buffer, X(1)

nop

; ENTRY M(21) - retrieve char from MDM: or COM: queue.

;  if entry was made at M(16) and the queue contained a char

;  then the code will fall through to this point and the

;  received char will be placed in X(1).

;  M(21) may also be called independently of M(16) to receive

;  a char if it's already known that one is there (due to an

;  ON MDM GOSUB.. interrupt for example).

;  calling sequence: CALL VARPTR(M(21)),0,VARPTR(X)

;  where X is an integer variable.

M(21)	call	6D7E	; get char from queue

mov	m,a	; store it

ret

nop

; ENTRY M(24) - timed read of 131 chars from MDM: or COM: queue.

;  calling sequence: CALL VARPTR(M(24)),0,VARPTR(Z(0))

;  where Z(0)..Z(131) is an integer array.

;  Z(0) is used as a flag.

;  Upon return, if Z(0)=0 then a timeout occurred.

;  Z(0)=1 implies successful return and in this case

;  Z(1)..Z(131) will contain the 131 received characters.

x1	equ	39.	; get_ch - $PC2

x2	equ	7.	; ck_que - $PC2

x3	equ	4.	;  init  - $PC2

M(24)	mvi	m,0	; set flag for failure

mov	d,h

mov	e,l	; DE = HL --> flag

call	31E9	; xthl, pchl

$PC2	mvi	b,131.	; char counter

push	b

push	h

; stack now contains:

;

;		address of flag

;		char counter

;    sp  -->	$PC2

init	lxi	h,7282.	; HL = 1 second timeout counter

ck_que	call	6D6D	; check queue

pop	b

push	b

push	b	; extra $PC2 on stack

xthl		; counter on stack, HL = $PC2

lxi	b,x1	; offset to get_ch

dad	b

xthl

rnz		; jnz	get_ch

pop	b	; clear stack

dcx	h	; decrement timeout counter

mov	a,h

ora	l

pop	b

push	b

push	b	; extra $PC2 on stack

xthl		; counter on stack, HL = $PC2

lxi	b,x2	; offset to ck_que

dad	b

xthl

rnz		; jnz	ck_que

pop	b

pop	b

pop	b

pop	b

ret		; timeout exit

get_ch	call	6D7E	; get char from queue

inx	d

inx	d

stax	d	; store it in buffer

pop	h

pop	b

dcr	b	; decrement char counter

push	b

push	h	; HL = $PC2

lxi	b,x3	; offset to init

dad	b

push	h

rnz		; jnz	init

pop	h

pop	h

pop	h

pop	h

inr	m	; set flag for success !

ret

; ENTRY M(59) - delay A*2 milliseconds

;  calling sequence:  CALL VARPTR(M(59),A

;  where A is an integer from 0 to 127.

M(59)	mov	c,a

jmp	531C

TIMING COMPARISONS:

Up/down-loading tests were performed using a 3422 byte text file and

a VAX-11/780 host.  The following table shows the relative time required

for the file transfers normalized to the TELCOM download time at 300

baud (which was 114 secs.).  Alternate character set was not used, nor

was word wrapping, and CAS: times include overhead of opening the file:

-------XMODEM PROGRAM-------

Xon/Xoff	    Xmodem

baud  direction  TELCOM	   RAM	  CAS:	  RAM	 CAS:

300	down	  1.00	   1.25	  1.89	  1.02	 2.16

300	 up	  1.25	   2.28	  2.98	  1.52	 2.14

1200	down	  0.46	   1.27	  1.89	  0.76	 1.38

1200	 up	  0.58	   2.25	  2.93	  0.75	 1.41

CONCLUSIONS:

- In TELCOM, uploads take about 25% longer than downloads.

- 1200 baud transfers take at best only half as long as 300 baud,

partly due to the slow speed of LCD scrolling.  Using Xmodem program

Xon/Xoff protocol, transfer times are independent of baud rate.

- Transfer times for Xmodem protocol using RAM file are slightly longer

than TELCOM times.  Using CAS: file doubles the time in some cases.