CUSTOMIZING TYPEFACES ON THE EPSON FX PRINTER

Jay Holovacs

CIS 74756,413

(Address any comments or questions

by Easyplex mail or contact me at:)

95 King George Rd.

Warren, NJ 07060

An excellent feature of the popular Epson FX series of

printers is support for user-defined typefaces in printer RAM.

For example, once a character set is downloaded into the

printer, text from the TEXT utility or a word processor can be

printed in the alternate typeface, or even switching back and

forth between it and the standard face just as if it were a part

of the printer ROM.  This allows for interesting possibilities

including special typefaces (old English, extra bold, block,

company logo) or even exotic foreign (such as Cyrillic, Hebrew

or even 'Klingon' alphabets).  The characters are defined in a

manner similar to graphics, however character definition is a

separate capability, which can be printed much faster than

constructing characters individually from graphics commands.

Most of the special features, such as super and subscripts,

underlining, emphasized print, proportional print, double strike

and pica/elite will work with user supplied character sets.

When I first started working with this feature of the

printer, I soon found that the major problem with implementing

these features as described in the Epson manual is the tedious

conversion of pencilled graphics into binary codes follwed by

preparation for the actual download into the printer.  To

simplify this drudgery, I began to develop FONTFX.BA, a typeface

editor program.  FONTFX allows for graphic editing, saving,

downloading and modification of typefaces.  Type data can be

stored in RAM, cassette or disk files (using Acroatix or

Travelling Software DOS) for downloading at the start of a

printing session or for future editing.  FONTFX is written for

the M100 but is suitable for M200; users may want to modify

screen placement to take advantage of the larger screen size,

but this is not required.

RUNNING FONTFX:

You will first be prompted for the file name of an existing

file to edit.  If this is a new session, simply hit Enter.

The next screen is the command line.  Hit E for edit.  You

will be prompted for an ASCII value to edit, then for its

charistics: whether it is a descender character (such as 'y' or

'g') and start and end of the character in dot columns.  To

accept default characteristics, simply hit enter at each prompt

(65='A', 'No' 0 and 11 are the defaults).  Once these entries

are completed, the character is drawn on the screen (blank is

default).  The characteristics can be changed later, so normally

it is good to start with the defaults when first developing a

character.

EPSON Typefaces     2

Once in the Edit mode, the following choices are available

as key-commands (use upper case for commands):

EDIT returns you to the top of the edit screen, the ASCII or

characteristics can then be changed.

FILL the current pixel location (as indicated by the pixel

cursors along the top and side of the character frame).

CLEAR the current pixel.

BLANK erases the currently selected character.

MOVE is followed by a choice of 5 subcommands (Up, Down, Right

Left and [enter]=abort).  Entering the key command will move

the entire shape one pixel in the indicated direction.

SAVE the current set of character definitions as a file (can

be RAM, cassette, or with proper DOS, direct-to-disk).

LOAD an existing character set file.  Note: if you have

characters in the program, they will be overwritten by this

command.

DOWNload the current character set to printer.  You will be

given the opportunity to select which ASCII values will be

downloaded, also the opportunity to copy the ROM characters

into RAM first (this allows default to any standard characters

which have not been redefined.)

TEST prompts for a string, which then will be printed out, for

the purpose of evaluating the characters on paper.  You will

be able to choose standard or proportional spacing.

KLONE copies a specified ASCII into another.  This is a useful

starting point for characters that are similar such as Q and O.

QUIT exit program.  If changes have been made since last Save,

you will be notified and given a chance to reconsider.

Use the four ARROW KEYS to manipulate the bit cursor locations.

KEY COMMANDS which prompt for additional information (Save,

Load etc.) can be aborted by entering a null in response to

the prompts.

NOTE: When loading or quitting, a warning message will appear if

the character data has been modified since it was last saved,

providing the opportunity to save the current definitions before

losing them.

USING YOUR NEW TYPEFACE:

PIXEL SPACING: The FX printer places a limitation on dot

EPSON Typefaces     3

placement in that dots cannot be directly adjacent right to

left, therefore a horizontal gap of at least one pixel must

remain between pixels on the same row.  Pixels may be directly

vertically or diagonally adjacent.  Even though the white space

is quite noticeable on the screen, there is considerable overlap

on paper and the result is not nearly so drastic as might be

expected.  It is helpful to frequently download and test

characters to get an accurate feel for the procedure.

Simulating of curved lines and various slopes is not really

as difficult as might be imagined from looking at the large

character frame.  Use the Download feature frequently at first

to get a good feel for the techniques of character description.

Once created and saved, typeface files can be downloaded

before a printing session by running FONTFX, loading the

appropriate file and going directly to the Download command.

Once the data is downloaded to the printer, the alternate

typeface can be summoned by the sequence

CHR$(27);CHR$(37);CHR$(N);CHR$(0).  If N=0 then standard

typeface is used, if N=1 then RAM type is used.  Many word

processing programs provide a means to embed control sequences

like the above directly in your text.  Unfortunately, this

command is not directly available in the standard text dump

built into the M100 because it has no means to embed a CHR$(0),

however the command can be sent from BASIC, then you can switch

over to text to utilize the RAM type.

DESCENDERS: When a descender is specified in the

characteristics, the pixels are entered in the same manner as

usual, however the entire frame will be shifted one pixel down

on paper when the character is printed.

PROPORTIONAL SPACING can be specified when printing with the

usual embedded sequence--CHR$(27);CHR$(112);CHR$(1)--however you

must supply the proportional information to the printer when

editing the typeface.  The characters in the Epson FX consist of

12 columns (numbered 0-11) by 8 rows.  The 12th column is always

blank.  The user can specify the start and end positions which

the printer will use when in porportional mode (in standard mode

the full width is always printed).

Once you get the character visually the way you like it,

center it appropriately in its work area, so that the character

will be centered when printing in nonporportional mode as well.

Hit E to get back to the top of the editing sequence and

re-enter the ASCII.  Determine the first column which will be

considered to start the character (it may include one or two

blank columns in front of the actual character) and enter this

value, then enter the ending column (which must be at least 5

greater than the start position).  The character is now ready

for proportional printing.  The editor will still accept data

for all columns, and this data will be saved in files, but data

beyond the last column specified will be ignored by the printer

when in proportional mode.

BOTH DESCENDER DATA AND LENGTH DATA CAN BE CHANGED AS DESIRED BY

RE-EDITING A CHARACTER.

EPSON Typefaces     4

COMMENTS ON THE PROGRAM:

PROGRAM STRUCTURE: In order to keep the program logic 'wide

open' for user modification, FONTFX as listed here is clearly

not trimmed back in size.  Extensive comments, and mnemonic

(rather than 2 character) variable names are used.  Packing this

program, either by hand or using a packer program will

significantly cut this size if size is critical.  Additionally,

the structure is strongly modular in nature, with the purpose of

each module defined by its comments.  BASIC by its nature does

not enforce structure (as Pascal, C or others), however, taking

the effort to construct the program with a structured philosophy

from the beginning helps significantly as the program expands

beyond its original concept.

To keep the file size compact, I have restricted the

changeable ASCII's to 33-127 which is the range normally used by

text type programs, however the Epson will accept 255 different

characters.  By changing DIMensions of CD and altering the value

subtracted from CHAR in lines 910 and 911, you can modify to

define as many characters as desired.

ERRORS: A significant amount of time goes into the design of

a typeface, therefore it is important to be able to recover in

running condition from an IO error; file errors are fairly

extensively trapped, with error messages; the more common file

errors will not stop the program.

Lines 1615-1620 cover disk access errors (Acroatix, these

lines can be user modified for TS or similar DOS) and can be

deleted if you are not using the portable disk drive.

Line 1690 is a catch-all for miscellaneous errors.  Program

execution stops, and if you can fix the problem by changing

value of an offending variable (NOT by changing a program line

however) and resuming at the same statement, the program can

then be CONTinued without losing the current work.

Note that line 1608 RESUMEs to line 100 rather than to the

subroutine level of the error statement.  From what I can

gather, when a memory overflow error occurs, the subroutine

stack is lost; and the program must resume at its lowest level

(I have only tried this technique on the M100).  All other types

of errors that I have encountered can be RESUMEd on the same

level from which the error occured.