RF PATH CALCULATIONS

The formulas were taken from a training manual, LIVING IN A CROWDED

WORLD, issued by General Electric Co., written by Mr. A.K. Guthrie. I'm

not sure who wrote the program that put everything together.

The program will calculate, in dBM, the signal strength at the receiver

from a transmitter at a known distance. The calculations use the figures

for a 1/2 wave dipole antenna. If you prefer the calculations to be in

dBW, the formulas can be modified or I can upload the complete program.

The answers are the same.

The results can be saved to RAM, to DISK or displayed on the LCD. In any

case, you will be asked at the end of the run if you want a "hard copy"

which will print out the results on the printer.

The program will prompt you all the way through the run to let you know

the input to be made. You must know: Transmitter Power Out, Antenna

gains, antenna system losses, distance between the transmitter and

receiver, the receiver sensitivity and the system frequency.

The program has subroutines to convert the transmitter power output to

dBM or dBW and convert the receiver input voltage to the same dB value.

You will go through these subroutines while entering the data. The

program will step you through. After the dB value is entered, press

<ENTER> to continue.

When the Transmitter or Receiver location is asked for, the name of the

site can be entered. The location can have a maximum of 11 letters. The

site name will be on the final printout and will correspond to the data

entered for that site.

You will have to calculate the antenna line and connector losses

seperately. They will be entered in the antenna system losses.

CAUTION: When entering the "antenna system losses" the figure MUST be

preceded by a "-" minus sign or the loss will be considered a gain. If

there is a 5 dB loss, enter -5.

Before the final calculations are made, the data that was entered will

be displayed to verify that it was entered correctly. If you made a

mistake, continue on with the program and it will take you to the

beginning again where the correct value can be entered.

If everything went well, the results will be displayed when you <ENTER>.

The results will show the free space loss, the signal level above the

receiver threshold and the amount of signal at the receiver port in

microvolts and dB.

If you want to run another path and only change one or more of the

parameters, stay in the program, the values you have entered are still

there. Change the values you want and <ENTER> at the ones you want to

remain the same. They will stay in the program.

9-01-87