in the computer are in different

locations, and that their

Appendix B

Reserved Memory

Reserved memory is a difficult subject if you are unfamiliar

with machine-language programming.  The secret to

understanding it is to understand why you need it.  The first

section below explains the need for reserved memory and

relocation in fairly simple terms.  Then, the following

section explains how POWR-DOS avoids some machine language

compatibility problems.

What is Reserved Memory?

Tandy laptop users are faced with two choices: either accept

mediocre performance, or learn how to best use their

machine's limited resources.  There are two crucial areas of

knowledge: machine-language compatibility and RAM

conservation.  To understand both, it is vital to know a few

things about your computer's memory.

The Model 100/200's memory is divided into a number of bytes.

Each byte represents a value from 0 to 255; you can think of

a byte as a character in a DOcument file.  The number of

bytes depends on the machine and the amount of memory.

Each byte within a bank has a unique address.  (If there is

more than one bank of RAM, the banks are almost completely

independent, and there will be one set of addresses for each

bank.)  Depending on the amount of RAM, not all addresses are

capable of "remembering" the bytes stored to them.  However,

the 8192 addresses from 57344 to 65535 are always valid RAM

addresses.

Your computer keeps very careful track of RAM.  Like a good

cook in a crowded kitchen, it divides the available space

into different sections.  There are four sections in one

division that start from the "bottom up", i.e., from the

lowest RAM address working upward:

1. BASIC Programs.  The start of BASIC programs mark the

start of usable RAM.

2. DOcument files.  The DOcument files and the PASTE buffer

come immediately after the BASIC programs.  THe exact

starting address depends on which BAsic files are in

memory.

Page 71

Appendix B                                          Page 72

3. COmmand files.  The COmmand files come immediately after

the DOcument files.  The exact starting address depends

on which BAsic files and DOcument files are in memory.

4. BASIC Variables.  Whatever BASIC variables have been

declared in a running program go here.

There is another major division, containing five sections,

that builds from the "top down", i.e., starting at highest

RAM address and moving downward:

9. System Memory.  This area is a "scratch pad" for the

computer.  The file names that appear on the main menu,

as well as numerous other data, are stored in system

memory.  All of the System Memory address are fixed.  On

the Model 100, the addresses range from 62960-65535; on

the Tandy 200, the addresses range from 61104-65535.

8. Reserved Memory.  This is the area of memory designed

for RAM-based machine-language programs.  Its upper

limit is the System Memory, which is fixed; its lower

limit is HIMEM, as set in the BASIC CLEAR command.

There is no limit to the number of running

machine-language programs that can reside in this area,

as long as they do not use the exact same addresses.

7. The file buffers.  Each file buffer takes up 265 bytes.

The number of buffers is set by MAXFILES in BASIC.  The

file buffers vary in location; they always end just

below HIMEM.

6. The string area.  This is the area used by BASIC to

store the contents of string-type variables.  The size

of this area is set by the BASIC CLEAR command.

5. The stack.  This is a scratchpad area for the computer's

processor.  It also stores information about BASIC

FOR-NEXT loops and GOSUB statements.

Whenever you do something that uses memory, you cause the

bottom division to grow upward or the top division to grow

downward or both.  For example, when you add a character to a

DOcument file, you cause the bottom division to grow upward

by one byte; when BASIC starts a new FOR-NEXT loop, the stack

grows downward by a few bytes.  You run out of memory when

the top of the bottom division (the end of section 4) runs

into the bottom of the top division (the beginning of section

5).

Appendix B                                          Page 73

You notice that all files in the computer are in different

locations, and that their locations can change.  BASIC

programs and DOcument files never need "relocation", since

they are designed to be independent of their position in RAM.

However, machine-language programs must always run from the

same addresses.  In other words, it is insufficient t specify

that they reside "somewhere" in user memory.  The designers

of the 100/200 operating systems made provision for fixed

locations: the high memory area (section 8, above).  The

lower boundary, called HIMEM, is equal to or lower than the

higher boundary, called MAXRAM; now files, variables, etc.,

will reside between HIMEM and MAXRAM.  Since MAXRAM is fixed,

section 8 has the constant address that machine-language

programs need.

Because the Model 100/200's RAM is scarce, most machine

language programs use the exact same locations (those

directly below MAXRAM) for their "running position".  It is

possible to design two or more specific machine-language

programs to share the reserved memory, but it is impossible

to do this in general.  As a result, if you want to run more

than one machine language program, you are out of luck.

Further, some BASIC programs and ROM programs (such as

Interactive Solutions for the Model 100) use the memory

location in section 8 as a "scratch pad" when they are

running; consequently, they corrupt other machine-language

programs.

The Model 100/200's designers anticipated the clashing of

machine language addresses.  They created "command" (CO)

files.  A CO file exists in the user memory with other files

and never clashes, but it cannot be "run" like a BASIC

program.  Instead, it is a snapshot of reserved memory

containing an image of a runable machine language program.

When you select a CO file on the main menu and press [ENTER],

the computer: (1) Copies the "snapshot" from section 3 into

section 8, and (2) begins execution of the copy in section

8. (NOTE: On the 200, spreadsheet files are CO files also.

This confuses the issue, because they are not "programs".

They do not get copied into high memory.)

There are some implications to CO files:

* HIMEM must be set low enough to load the CO file into

reserved memory before running it.  If it is not, the

computer will just beep and return to the menu.

Appendix B                                          Page 74

* In order to run a CO file from the menu, you must have

enough memory to store it twice: once in the CO file,

and once in reserved memory.

* Once a CO file has been copied into reserved memory, it

is often possible to remove the CO file from RAM and

still execute the program!

Enter POWR-DOS

POWR-DOS is a machine-language program.  So, like all other

machine-language programs, it must run in a fixed address.

However, to be most effective, it must be running constantly,

which makes relocation a difficult issue.  We provided a

relocator with POWR-DISK (a precursor to POWR-DOS), but it

was not an ideal solution.

POWR-DOS take advantage of the only other fixed location in

the Model 100/200: the first BASIC program.  You may have

noticed that the 100/200 stores BASIC programs in the very

lowest part of RAM.  The very first BASIC program does not

move once it is installed.  We "disguised" POWR-DOS as a

BAsic file, even though it is a machine-language program; in

that way, it has a fixed location and is completely out of

reserved memory.

There is one complication to this arrangement.  There are

other programs that use the same scheme! Most notably, some

software by Dr. David Sumner (who gets the credit for this

loading idea originally) and some software by PG Design use

this position.  Other software may appear also.  For this

reason, you can specify POWR-DOS to load as the second,

third, etc. BAsic file by designating an earlier program

"immovable".  See chapter 1 for how you do this.  The only

problem, of course, is that if you remove a BAsic file below

POWR-DOS in memory, you will get into trouble.

Error Codes

Code      No.       Description

NF        1         NEXT without FOR.

SN        2         Syntax Error.

RG        3         RETURN without GOSUB.

OD        4         Out of Data.

FC        5         Illegal Function Call.

OV        6         Overflow.

OM        7         Out of Memory.

UL        8         Undefined Line.

BS        9         Bad Subscript.

DD        10        Doubly Dimensioned Array.

/0        11        Division by Zero.

ID        12        Illegal Direct Statement.

TM        13        Type Mismatch.

OS        14        Out of String Space.

LS        15        String too Long.

ST        16        String Formula too Complex.

CN        17        Can't CONTinue.

IO        18        Input/Output Error or user Break.

NR        19        No RESUME in Error Routine.

RW        20        RESUME without error.

UE        21        Undefined Error Number.

MO        22        Missing operand for command.

IE        50        Illegal Error Code.

BN        51        Bad File Number (MAXFILES too low)

FF        52        File not Found.

AO        53        File already open.

EF        54        Input past End of File.

NM        55        Bad File Name.

DS        56        Direct Statement in File.

FL        57        RAM Directory Full.

CF        58        File not Open.

NR        59        RS232 Not Ready for Disk Access.

DN        60        Drive not Responding.

CM        61        Communications Error.

DA        62        Drive Done Early.

WP        63        Disk is Write Protected.

DF        64        Disk is Full.

ND        65        No Disk in Drive.

HT        66        Miscellaneous Drive Trouble.

AE        67        File already exists; can't overwrite.