DISKIT DOCUMENTATION

Diskit  is  a  sample program to demonstrate how to access PCSG RAM+

banks and the Tandy Disk Drive within the same program on the  Model

100.   RAM+ allows you to read and write files in other banks within

a BASIC program.  PWR-Disk allows you to read and write files on the

TDD within a BASIC program.  But, since each resets each others  ram

hooks, after running one, the other doesn't work.

This  program  sets  the  hooks back and forth inside the program to

enable you to read a file from another bank and write to a disk file

on the TDD, or  visa  versa.   The  CALL  MAXRAM  command  sets  the

Pwr-Disk hooks and can be executed anywhere inside a program without

interrupting  it.  PCSG, on the other hand, normally requires a call

63012 which terminates program execution  and  brings  up  the  RAM+

menu.   PCSG  wouldn't give me the pokes to reinstall the ram hooks,

but a few peeks in the ram hook table after  running  each  revealed

the  following  pokes  required  to  reactivate  RAM+  after running

Pwr-disk.

The poke for RAM+ is as follows:

POKE 64262,18:POKE64263,246:POKE64266,18:POKE64267,246

The CALL MAXRAM command and these  pokes  can  be  used  inside  the

program to toggle back and forth between Pwr-Disk and RAM+ to access

the different banks.  The hooks must be installed before any and all

file handling instructions in the program, i.e. open, close, input#,

print#,  etc.  If you print to a different bank's file you must poke

the Ram hooks first.  If you then read from a disk  file,  you  must

poke  the  disk  ram  hooks (CALL MAXRAM)  To open a file in another

bank poke the hooks, then to open a second file on  the  disk,  poke

the  disk  hooks.   Even  to  close  the  files, you must close them

separately - CLOSE1  CLOSE2 - and poke the appropriate hooks  before

each close.  Get the picture?

I  use the Ram+ pokes at the beginning of any program which accesses

another bank, whether or not I am also accessing the disk,  then  if

Pwr-Disk has been run before running such a program it will crap-out

with  a Bad File Name ?BN Error.  Putting the pokes at the beginning

assures that my program will run no matter what.  (Except if I  have

done  an F3 while on the RAM+ menu since this deinstalls everything,

not just the hooks)  It also allows me to do a  RUN":NAME"  and  run

the program off disk and still access files in other banks.

There  is  a  potential that the wrong poke, or some other cause may

result in a cold start using this method.  I have played around with

it for quite a while now and have yet to  cold-start,  so  it  seems

quite  safe,  but  I  take  no responsibility if your combination of

prgrams results in such.

If you have any other program messing around with  ram  hooks  which

deactivates  your  RAM+  capabilities,  this should work as well for

that.  Just back up before you try it the first time.

By the way, if you use Lucid by PCSG and the 4 Ram Bank product, you

may have noticed that Lucid won't  print  out  a  spreadsheet  until

deinstalling  the RAM+ software.  You deinstall by pressing F3 while

page -1-

in the PCSG menu.  This also places a small  Goto  program  in  each

bank.   I  have found that by using the following poke, I deactivate

RAM+ in the bank I am in, but don't deinstall it in every  bank.   I

can  then print out the Lucid file without the "M100 ERROR! message.

Use the following poke:

POKE 64262,243:POKE64263,127:POKE64266,243:POKE64267,127

Good luck!

Terry Luke  73126,1776

0 REM << TDD/PCSG Ram Bank Transfer Program by Terry Luke 73126,1775

>>

1 REM *********************************

2 REM * Set up the Program and input  *

3 REM * the first file. Instr search  *

4 REM * to check the position of ':'  *

5 REM * 1st position = disk files     *

6 REM * 2nd position = ram bank       *

7 REM * eg:':test'=disk or'A:test'=ram*

8 REM * D=1 for Disk, D=0 for Ram     *

9 REM *********************************

10 CLEAR:MAXFILES=2:PRINT"Input       File:";:INPUTF$:C=INSTR(F$,":")

:IFC=1THEND=1ELSED=0

11 REM

12 REM ********************************

13 REM * Go to subroutine to determine*

14 REM *    which hooks to install    *

15 REM *   Then open the input file   *

16 REM ********************************

16 REM

20 GOSUB90:OPENF$FORINPUTAS1

21 REM

22 REM ********************************

23 REM * Go to Subroutine to determine*

24 REM * which hooks to install for   *

25 REM *         output file          *

26 REM *      (Pwr-Disk or Ram+)      *

27 REM * Determine mode of transfer   *

28 REM ********************************

29 REM

30 GOSUB100:INPUT"Output File:";OF$:OPENOF$FOROUTPUTAS2:INPUT"<L>ine

or <C>haracter Transfer:";Y$:IFY$="L"ORY$="l"THENY=1:GOTO50

31 REM

32 REM ********************************

33 REM *  GoSub to install ram hooks  *

34 REM * Input one character at a time*

34 REM * Character mode is 3-4 times  *

35 REM * slower than line mode but if *

36 REM * the file has more than 255   *

37 REM * char. before a C/R, like a   *

38 REM * paragraph,line mode craps out*

39 REM ********************************

40 GOSUB90:IFNOTEOF(1)THENA$=INPUT$(1,1):GOTO60ELSE9

page -2-

41 REM

42 REM ********************************

43 REM * Gosub to install alternate   *

44 REM * Ram hooks to prepare for out-*

45 REM * put.  Inputs a line at a time*

46 REM * If any line in the file is   *

47 REM * longer then 255 characters,  *

48 REM * you must use character mode  *

49 REM ********************************

50 GOSUB90:IFNOTEOF(1)THENLINEINPUT#1,A$ELSE70

51 REM

52 REM ********************************

53 REM * Gosub to install alternate   *

54 REM * Ram Hooks before printing the*

55 REM * present line or character to *

56 REM * output file and return to the*

57 REM *  proper line for more input  *

58 REM ********************************

59 REM

60 GOSUB100:PRINT#2,A$;:IFY=1THENPRINT#2,:GOTO50ELSEGOTO4

61 REM

62 REM ********************************

63 REM * Gosub to install the appro-  *

64 REM * priate ram hooks to close 1  *

65 REM * then install the other ram   *

66 REM * hooks before closing 2. End  *

67 REM ********************************

68 REM

70 GOSUB90:CLOSE1:GOSUB100:CLOSE2:END

71 REM

72 REM ********************************

73 REM * This line pokes the PCSG     *

74 REM *       RAM+ hooks             *

75 REM ********************************

76 REM

80 POKE 64262,18:POKE64263,246:POKE64266,18:POKE64267,246:RETURN

81 REM

82 REM ********************************

83 REM * This line determines if the  *

84 REM * input file is a Ram Bank or  *

85 REM * a disk file then branches to *

86 REM * the proper subroutine to in- *

87 REM * stall the right ram hooks    *

88 REM ********************************

89 REM

90 IFD=0THENGOSUB80:RETURNELSEGOSUB110:RETURN

91 REM

92 REM ********************************

93 REM * This line determines if the  *

94 REM * output files is a Ram Bank   *

95 REM * or a Disk files and branches *

96 REM * to the proper subroutine to  *

97 REM * install the right ram hooks  *

98 REM ********************************

99 REM

100 IFD=1THENGOSUB80:RETURNELSEGOSUB110:RETURN

page -3-

101 REM *******************************

102 REM * This routine installs the   *

103 REM * Pwr-Disk ram hooks by call- *

104 REM * ing MAXRAM. It assumes PD   *

105 REM *  is already installed and   *

106 REM * only Ram hooks need be poked*

107 REM * Doesn't work after LFILESOFF*

108 REM * command has been executed   *

109 REM *******************************

110 CALLMAXRAM:RETURN

111 REM

112 REM *******************************

113 REM * Note: In character mode it  *

114 REM * sometimes seems like nothing*

115 REM * is happening since the      *

116 REM * characters are stored and   *

117 REM * sent to disk in bunches.    *

118 REM *    Just be patient!         *

119 REM *******************************

page -4-