BASIC Program Emulates Logic Circuits

(Jay Holovacs 74756,413)

When designing logical control circuits (consisting of ANDs

ORs etc.), it is frequently inconvenient to evaluate the logical

operation of the circuit entirely 'by hand', even on  circuits

with just a few dozen elements.  This M100/M200 Basic program

can be used to develop a truth table for such a circuit.

Additionally, the effects of changing a circuit element or

circuit connection can be evaluated quickly.

As set up, it can model any circuit composed of AND, OR,

XOR, NOR, NAND and NOT (inverter) elements connected in normal

manners.  Truth tables for each element are maintained in DATA

statements, other types of elements could be added.  In addition

to the normal T/F inputs, circuit links may be specified as

undetermined.  In evaluating an undetermined input into a node,

the program attempts to ascertain the output of the node

according to predetermined rules (an exmple of a condition which

can be determined despite an undetermined input: False AND

[undefined] --> False as output).  This can help when modelling

sections of circuitry which are in unpredictable states at

power-up.

USING THE PROGRAM:

On a paper drawing of your schematic, number each logical

element (Starting from 1) in an arbitrary sequence.  Number the

links (interconnections between nodes) starting from 1 also.

Sequence is again arbitrary.  A link is defined as any

electrically continuous connection; for example, a signal line

that splits and feeds several elements would carry the same link

number for all its branches.  Include at this time any

questionable elements and links that you may not use; they can

be removed from and added to the circuit during the program run.

When prompted, enter the appropriate counts for nodes and

links, then answer the prompts for each of the nodes.  This

process can be redone at any time by selecting option 1 from the

main menu.

Using option 2 on the menu, set up the status of the links

with any initial logical values you wish to specify:

T--true

F--false

U--undefined

[enter/return]--status unchanged

[esc]--exit option 2 leaving the remainder of the links

unchanged

Option 3 will display status of all links for review.

Option 4 will execute the iterative evaluation procedure, in

which the program attempts to set all the links to achieve an

internally consistent (stable) state.

Option 5 allows changing the hookup or type of a single node

without going through the entire list.

Logic Circuit Emulator         Pg  2

ALGORITHM AND PROGRAM MODIFICATION INFORMATION:

Execution of option 4 automatically steps the program

repeatedly through all nodes.  The sequence finally halts when

either 1) an apparent equilibrium condition is reached in which

each iteration generates no new information or 2) an instability

condition is implied by the fact that total quantity of state

changes indicates nodes are changing repeatedly.  This does not,

however, constitute a mathematical guarantee of instability.  If

there is some suspicion that the circuit may yet 'settle down'

option 4 can be repeatedly called without changing the state of

any links between calls.

As currently configured, the program expects 2 inputs per

logical element (node); consequently multiple-input ORs or ANDs

must be represented by cascaded elements.  If you use these

frequently, they can be added as special cases.  An inverter can

be represented as a NAND with both inputs tied together.

TO CHANGE OR ADD NEW LOGIC COMPONENT DEFINITIONS:

(Only necessary if you want to modify the program for special

purposes)

Data statements defining the performance of each logic element

are stored in lines 990-999.  Truth tables for all elements are

contained in the 3 dimensional integer array TT(element

type-1,input #1-1,input #2-1).  Each specific function consists

of a truth table in the following format:

F  T  U

F x  x  x

T x  x  x

U x  x  x

Where the numeric value of x (1=False; 2=True;

3=Undetermined) represents the state of the output for the

element when the two inputs (horizontal and vertical rows) are

present.

EXAMPLE:

Because there is no easy way to upload a realistic circuit

in ascii, a very useless circuit illustrates how nodes and links

are numbered for LOGIC.BA

____

______________|    \  N1

L1           |AND  \____________

______________|     /      L4

L2    |   L2 |____/

|       ____

|______|    \ N2

L2  | OR  \_____________

_____________|     /      L5

L3          |____/

Logic Circuit Emulator         Pg  3

The text below represents the prompting you will get and the

appropriate answers for our useless circuit

NODE 1     TYPE:UNDEF

0=UNDEF 1=AND 2=OR 3=XOR 4=NAND 5=NOR

TYPE:?1

INPUTS A,B [ 0 0 ]? 1,2

OUTPUT [ 0 ]? 4

NODE 2     TYPE:UNDEF

0=UNDEF 1=AND 2=OR 3=XOR 4=NAND 5=NOR

TYPE:?2

INPUTS A,B [ 0 0 ]? 2,3

OUTPUT [ 0 ]? 5

The values in [square brackets] are the current values, when

you look at these later in the program run they will reflect the

current values established.  At any of the prompts, hitting

'enter' leaves the values unchanged.

Note that it is helpful to assign the lowest numbers to

input links, using the [esc] key to avoid stepping through all

the links for each evaluation.

To remove a node from the circuit, change its type to 0

(undefined).  To effectively remove a link from the circuit,

simply remove it as an input from any node to which it was

connected.