PRT187.DOC

Tandy 200 Port 187

The Tandy 200 uses three input lines that are easily accessed.  The printer

port BUSY (pin 21), the printer port NOT BUSY (pin 25), and the bar code

port RX DATA (pin 2) can be tested easily by the BASIC INP(X) function for

a HIGH or LOW condition (all are normally HIGH).  By using all of the

possible combinations of these data lines, eight discrete combinations are

found.  These combinations are in the following table with the results of

the  function:

V=INP(187).

BAR RXD  BUSY  NOT BUSY = VALUE

0      X      X      =  241

0      X      0      =  243

0      0      X      =  245

0      0      0      =  247

X      X      X      =  249

X      X      0      =  251

X      0      X      =  253

X      0      0      =  255

0=OPEN   X=GROUNDED

The BASIC program PRT187.200 will run on the T200 and display the status of

the lines.  I do not know if the data lines are exactly the same on the

Model 100/102 as I do not currently have access to one for testing.  This

program shows the branching that is possible with the different combinations

of lines pulled LOW.  This is a simple way to use the T200 for external

event driven programming.  Hook it up to something imaginative and useful

like a wind direction indicator or a burglar alarm with three zones.

The pins on the Centronics (printer) end of the printer cable that

correspond to the T200 pins are as follows:

T200           =    Centronics 36 Pin

---------------------------------------

BUSY (pin 21)     =    BUSY (pin 11)

NOT BUSY (pin 25) =  NOT BUSY (pin 13)

Have fun with this and good luck.

Paul R. Jorgenson  KE7HR

70416,460

28 November 1992