FLIGHT.DO

FLIGHT.BA and  the  companion  modules  were  written  using

admittedly inaccurate  assumptions and  approximations,  some  of

which are  discussed below.   I  make no  warranties of any kind,

including warranties of merchantability and fitness for a partic-

ular purpose.   These programs were written for my own entertain-

ment.  Since I cannot prevent their change by some unknown person

before they get to you, since I know that they rely upon approxi-

mations which  are not  totally accurate,  and since,  in fact, I

cannot even  be sure  that garbage  won't sneak in from the phone

lines as  you download  them, I cannot know if they will work for

you, but would be willing to bet that they won't.

FLIGHT.BA and its modules were written for the TRS Model 100

to take  the place  of the  slide rule  type  flight  "computer".

Actually, these  are a  rewrite of  FLTCMP.BA,  which  I  earlier

uploaded to  Compuserve.   The most  obvious difference  is  that

FLTCMP.BA was  a single  program incorporating  all 8 of the pro-

grams included  in FLIGHT.BA  and its  modules.    As  a  result,

FLTCMP.BA was  huge.  FLIGHT.BA is a menu program which loads and

runs various  modules depending  upon the choices you make in the

menu.   I wanted  these programs to do everything the E6B can do,

plus a few things it can't, like weight & balance.

Another major change in FLIGHT.BA is a change in the naviga-

tion calculations.   I  had used  rhumb line  formulas I  got off

AVSIG, and  found that  if I plotted a course of either 90 or 270

true, the did not work.  As the course approached either of those

figures, the distance went off the scale.  I gave up on trying to

get this  correct, and  have assumed that the earth is flat.  The

course and  distance are calculated using simple plane trigonome-

try and a constant of 60.1 nm per degree of latitude.  This seems

to get  close enough  for light plane VFR flying, within a few nm

over 400  nm and  within a few degrees of the charted course.  It

may not  be exact, but it seems to work.  However, I am not about

to take off, leave my charts at home, and simply fly the headings

the program gives me.  This is only intended to be an aid, and in

no way  can substitute  for either  good flight  planning or good

piloting.

There are  a total  of 8  programs included  in this  group.

FLIGHT.BA (checksum  85054) is  the main menu program.  FLITE1.BA

(checksum 171559)  converts from  one measure  to another  (e.g.,

Fahrenheit to  Celsius, Knots to Mph, etc.).  FLITE2.BA (checksum

67256) is a simple standard math calculator.  FLITE3.BA (checksum

197223) is  the Time/Speed/Distance  module.  FLITE4.BA (checksum

167609) is the Altitude and Airspeed module.  FLITE5.BA (checksum

173379) is  the Wind  and  Drift  module.    FLITE6.BA  (checksum

133228) is  the Weight  and Balance  module.  FLITE7.BA (checksum

425416) is  the Navigation  module.   Note that if you do not use

these names for the program modules, the menus won't work.

FLIGHT pretty  much runs itself.  You move through the menus

using the  up and  down arrow  keys.  The data for ident to ident

navigation should  be stored in NAVAID.DO, an example of which is

included at  the end  of this documentation.  The data fields are

Ident, Description, Latitude North, Longitude West, Compass Vari-

ation.   Latitude and longitude data are included in NAVAID.DO as

decimal numbers,  but the  number to  the right of the decimal is

not fractional  degrees, but  rather  the  minutes  of  latitude.

Thus, 35  deg. 30 min. latitude would be included in NAVAID.DO as

"35.30", and  not as  "35.50".   Compass variation,  on the other

hand, is  in degrees  and fractional  degrees.   Thus, 10 deg. 45

min. east  variation would  be stored  as "-10.75", and not as "-

10.45", if  you want  to get that exact with the variations.  The

program will  append new idents to your NAVAID.DO, so you can add

idents that  way.  I have found it convenient to put the lat. and

long. for  enroute checkpoints and other locations for which I do

not have  an ident  into NAVAID.DO using an ident of XXXX.  Note,

however, that  XXXX is  the only  ident that the program does not

expect to  be unique.   For  that reason, for VORs which have the

same ident  as an  airport, but which are not located at the same

lat. and  long., I  add "v"  at the  end of  the ident (e.g., LAX

would be  the airport,  and LAXv  would be the VOR).  The program

does not  require you  to use  "v", and  you can develop your own

method of dealing with similar idents.  Use the up and down arrow

keys to scan through your NAVAID.DO idents.

Each program  is relatively large for its functions, because

of all  the screen  printing and menus.  All of them together are

huge, and  therefore they are not something you would likely want

to leave  stored in  RAM on  a permanent basis.  However, none of

the programs are so large that you can't download them to a M100.

The versions  described here  (with their  checksums) are  packed

versions, which  helps some  on the  RAM usage,  but I  have  not

really tried  to optimize  the code to reduce their size.  I have

also unpacked versions if anyone wants to try to make the changes

suggested below  or wants  to modify  these for an IBM compatible

(such as a Toshiba he says, with lust in his voice).  If you want

me to  upload the unpacked versions for direct download, leave me

a message  and maybe I will do so.  I have not renumbered each of

the program  modules to  the lowest  possible line  numbers  (and

therefore the  lowest possible RAM requirements in ASCII) so that

anyone who  wants to  merge them  all can see how everything fits

together into one program.  However, I have made numerous changes

in each  of  the  programs  since  I  broke  them  apart  out  of

FLTCMP.BA, and  therefore a  straight merge  may  overwrite  some

important coding.   You  might want to look at the Tandy 600 ver-

sion I have written and uploaded to the Tandy 600 data library in

this forum.

If you  like this  program, or have suggestions on modifying

it, let  me know  by leaving  me a message in either the AVSIG or

the M100SIG  on Compuserve.   My  Compuserve ID is 73337,2600.  I

may or  may not  acknowledge your message, as I get on Compuserve

irregularly.  And, I may or may not make modifications based upon

suggestions I  receive.   And, if  I do make the modifications, I

may or  may not  upload the  modified version.   In  fact, if you

think of  good modifications,  please make  them and upload a new

version of this.  Suggestions for improvement:

1.   Code actual  formulas for  pressure  and  density  altitude,

instead of approximations derived from charts.

2.   Combine the navigation and wind triangle functions to give a

screen more  like the standard Pilot's Navigation Log/Flight

Plan form (I did this on the Tandy 600 version).

3.   Rewrite the  weight and  balance portion to use data for the

plane you fly, or even several planes from which you choose,

stored in a .DO file, rather than having to be keyed in each

time.

The formulas  for the  following quantities  were derived as

linear  algebraic  (and  not  differential)  equations  from  the

answers to  problems in  either the Private Pilot Test or the E6B

instruction manual.   For  that reason,  the answers this program

gives for  many of  these values  are only approximate.  At least

some of these values are not linear.  For example, pressure alti-

tude does not decrease at exactly 1000' (or even 910') per 1" Hg.

The decrease changes as the pressure changes, and as a result the

difference between  the correction factors for 28.0" Hg and 29.0"

Hg is  greater than the difference between the correction factors

for 30.0" Hg and 31.0" Hg.  Note lines 970-1020 of FLITE4.BA give

six different formulas for calculating the approximate correction

factor, depending upon the altimeter setting.

The formulas  I have  derived are  those for  the  following

unknowns:

1.   Pressure Altitude-see  lines 970-1020  in FLITE4.BA.   PA  =

Altitude +  ((29.92 - Altimeter Setting) x 910) + 46.8 + (60

x (28.5  - Altimeter  Setting)) if Altimeter Setting is less

than 28.5.

2.   Density Altitude-see  line 1080 in FLITE4.BA.  Density Alti-

tude = (Pressure Altitude x 0.9075) + (66 x Outside Tempera-

ture - Standard Temperature)) + 256

3.   Standard Temperature - see line 1080 in FLITE4.BA.  Standard

Temperature = 59 - ((Pressure Altitude x 3.5)/1000).

If someone  has more  accurate formulas for these values, I would

appreciate getting  them.   Please leave them in a message to me,

along with where you got them.

Although I  have never  tried it, I assume that the programs

will run  on a  M200.   The only calls included in these programs

are to  turn the  cursor on  and off  for esthetic  purposes, and

could be deleted for compatibility's sake.

Of course,  any values which depend upon these numbers (such

as True  Airspeed which  depends upon  Density Altitude, which in

turn depends  upon Pressure  Altitude and  Standard  Temperature)

will also  be only  approximations.   And since some of the vari-

ables used  to compute  the values in these derived equations are

themselves computed using derived equations, the inaccuracies may

very well  compound each  other.  What does this all mean?  IF IT

IS CRITICAL,  CHECK IT  USING A METHOD OF KNOWN ACCURACY.  If you

don't, you will be courting disaster.

NAVAID.DO Example

ABQ,ALBUQUERQUE NM,35.03,106.36,-11.34

AUS,AUSTIN TX,30.18,97.42,-7.1

FLG,FLAGSTAFF AZ,35.09,111.40,-12.87

PHX,PHOENIX SKY HARBOR,33.26,112,-12.41

XXXX,DEL RIO TX,29.22,100.55,-8.43