Documentation for XMDM26, for TRS-80 Model 100

Version 2.6 of 5/23/87  by  Leonard Erickson 70465,203

(from XMODEM.312 by Rick Perry, without which this would never have been done)

"If I seem to see farther than others, it is because I stand upon the

shoulders of giants."

This program performs auto-dial-redial, auto-logon, and buffered text file

transfers (.DO) using either regular ASCII Xon/Xoff protocol or the Xmodem

Christensen checksum protocol. Communication parameters are 8N1D during Xmodem

file transfers and 7I1E for terminal emulation and Xon/Xoff transfers. The

program will operate using the 300 baud internal modem or an external modem

(Hayes compatible) at 300, 1200, or 2400 baud. Files may be Up/Down loaded

directly from/to RAM, disk or cassette.

Graphics characters were used to remove the need for several strange

routines to set up string varibles and function keys. Due to uploading problems

those lines (32, 41, 53, & 67) have been left out. You need to run the

following program which will create a file called MERGE.DO.

10 OPEN"merge"FOROUTPUTAS1:Q$=CHR$(34)

20 PRINT#1,"32 POKE-86,0:D=0:ONERRORGOTO44:POKE63056,64:A$="+Q$+"Pre"+CHR$(246)

+"Dow"+CHR$(238)+"U"+CHR$(240)+"Ter"+CHR$(237)+"Br"+CHR$(235)+"Fr"+CHR$(229)+"D

i"+CHR$(242)+"By"+CHR$(229)+Q$+":GOSUB54"

30 PRINT#1,"41 GOSUB92:IFA$="+Q$+"Y"+Q$+"THENE=0:GOTO7ELSEA$="+Q$+"Pre"+CHR$

(246)+"@@@"+CHR$(192)+CHR$(128)+"Ter"+CHR$(237)+"Br"+CHR$(235)+"Fr"+CHR$(229)+"

Di"+CHR$(242)+"By"+CHR$(229)+Q$+":GOSUB54:N=-1:D=-1:GOTO33"

40 PRINT#1,"53 A$="+Q$+STRING$(7,128)+"Abr"+CHR$(244)+Q$

50 PRINT#1,"67 N$=CHR$(0):G$="+Q$+CHR$(255)+Q$+":F$="+Q$+CHR$(248)+CHR$(249)+CH

R$(250)+CHR$(251)+CHR$(252)+CHR$(253)+CHR$(254)+CHR$(255)+Q$+":U$=chr$(27)+"+Q$

+"A"+Q$+":POKE63056,64:CALL17875:POKE63056,0:CALL27804"

After running this, load XMDM26 into BASIC. Then type MERGE"MERGE". This will

merge the new lines into XMDM26. Now you can save the completed version.

AUTO-DIAL/LOGON:

The program starts with a "Find:" prompt and will attempt to retrieve a

phone number and auto-logon sequence from the ADRS.DO file and logon to a host

computer.

When using the internal modem, if no carrier is detected within 20 seconds

after dialing, or if you press any key, the program will hang up the phone.

After waiting 2 seconds more, it will retry the number unless you press any

key.

If the carrier is detected, it will BEEP and proceed through the auto-logon

sequence as specified in the ADRS line, then jump to the terminal emulation

mode of the program.

At the "Find:" prompt you may:

1. press F1 (or enter "Files") to change the search file. (normally the

ADRS.DO file is used). You will be prompted ("File:") for the file

to use. Both RAM & disk files (DVI and Chipmunk, *not* TDD!) may be

used.

2. press F8 (or enter "Menu") to return to the Menu

3. enter "Dial" if you wish to manually dial a telephone number. After

dialing and hearing the carrier tone, press "M", "3", "5" or "6"

(see below for to details on which to use) to allow the program to

Connect, or press any other key to return to the "Find:" prompt.

4. enter a string to search for in the search file.

ONLY LINES CONTAINING COMPUTER PHONE NUMBERS WILL SEARCHED!

If the string is not found you will be returned to the "Find:"

prompt. If match is found you may:

1. press "M" to begin auto-dial using the internal modem

2. press "3" to dial through an external Hayes-compatible modem

at 300 baud

3. press "5" to use Hayes at 1200 baud

4. press "6" to use Hayes at 2400 baud

5. press ESC to return to the "Find:" prompt

6. press any other key (such as ENTER) to continue searching.

TERMINAL EMULATION:

Once in terminal emulation mode, the program is transparent to all control

characters entered from the keyboard and these characters (like CTRL/C/S/Q)

will be sent to the host. But Function key F5 (Brk) may be used to STOP the

program if needed.

F1 (Prev) will display the previous screen, same as in TELCOM. (unless you

are using a TV or monitor with a DVI)

F2 (Down) will initiate a file download transfer and F3 (Up) will initiate

uploading.

F4 (Term) is used to jump directly to TELCOM's Term mode while remaining

connected to the host. Once in TELCOM you cannot return to the program without

logging off and disconnecting from the host unless you are connected through an

external modem.

F6 (Free) provides a display of how many bytes of RAM are currently

available.

F7 (Dir) lists the RAM file directory in the same way as the BASIC FILES

command.

F8 (Bye) is used to exit the program and return to the Model 100 Main Menu.

FILE TRANSFERS:

After pressing F2 or F3 and entering file name, program prompts "Append?" if

downloading. Then it prompts "Use XMODEM protocol?" -- press "Y" or "N".

Responding with "N" means that buffered Xon/Xoff protocol will be used.

For uploading using Xon/Xoff protocol answer "Width?" prompt by pressing

ENTER if no wrapping is desired, otherwise enter desired width for word

wrapping.

During Xon/Xoff download, F3 is blanked and F2=@@@@.  Press F2 to end

download.  During upload, use F8 to abort.

DOWNLOADING WITH XMODEM PROTOCOL:

The host should be ready to send the file before entering this mode. While

downloading, the received file is not displayed on the screen. When a NAK

(negative acknowledge) is sent, the program prints an "X". When ACK

(acknowledge) is sent, the program prints ".".

You can abort the download at any time by pressing F8 (Abrt). The program

itself will automatically abort the download only if one of the following

circumstances occur:

- The host sends a CANcel.

- The received block number is neither the current block number nor the

previous block number.

- An error trap occurs in the program (this could be caused by an "Out of

Memory" condition on the Model 100).

UPLOADING WITH XMODEM PROTOCOL:

The host should be ready to receive the file before entering this mode.

While uploading, the transmitted file is not displayed on the screen. When a

NAK is received, the program prints an "X". When ACK is received, the program

prints ".". If nothing is received after waiting 10 seconds, or if anything

other than NAK, ACK or CAN is received, the program prints a "?".

You can abort the upload at any time by pressing F8 (Abrt). The program

itself will automatically abort the upload only if the host sends a CANcel or

if an error trap occurs (e.g. due to an IO Error when uploading from cassette).

PROGRAM NOTES:

The program itself does not count the number of NAK's sent or received per

block and will not automatically abort if an excessive number of NAK's occur.

It is up to you to use F8 (Abrt) to cancel the file transfer if you feel that

too many NAK's are occuring.

When aborting a file transfer the program sends a single CANcel character

(CTRL/X) to the host and returns to its terminal emulation mode. If you then

notice that the host has not responded to the CANcel, you may send additional

CAN's by pressing CTRL/X. Some hosts expect to receive three CTRL/C chars in a

row to signal an abort; that's dumb, but anyway, if CTRL/X doesn't work, try

that. Most hosts will automatically abort an Xmodem transfer if no chars are

received for 1 or 2 minutes.

MAXFILES is set to 3 at the beginning and reset to 1 upon normal exit to the

Menu or TELCOM.

Function key definitions are restored to their previous user-defined values

upon exit from the program to the Menu or TELCOM.

The following ROM calls and POKEs are used:

6118 - set serial interface parameters and activate modem.

16969 - turn cursor on.

17061 - set and display temporary function key strings.

17875 - fill previous screen with blanks.

21172 - keep phone off hook.

21179 - hang up phone.

21264 - 2 second delay.

21274 - 1/2 second delay.

21293 - dial a phone number.

21392 - execute auto-logon sequence. (internal modem *only*

21443 - execute auto-logon sequence. (external modem)

21608 - jump to TELCOM's Term mode.

21795 - display previous screen.

27804 - restore function key definitions.

POKE 63056,64 - enable scrolling into previous screen area and also

disables Break and ON KEY and makes F1...F8 return chars 248...255.

POKE -86,0 - purge keyboard buffer.

MACHINE LANGUAGE SUBROUTINES:

The integer array M(0)..M(60) is used to hold six position independent machine language subroutines. Entry points are at M(0), M(3), M(16), M(21), M(24) &

M(59). The assembly source listing follows:

; ENTRY M(0) - check MDM: for carrier detect.

;  calling sequence: CALL VARPTR(M(0)),0,VARPTR(X)

;  where X is an integer variable.

M(0)	call	6EEF	; check for carrier detect

mov	m,a	; X=0 if carrier detected,

ret		; FF if no carrier

nop

; ENTRY M(3) - convert string to uppercase.

;  calling sequence: CALL VARPTR(M(3)),0,VARPTR(A$)

;  where A$ is a string variable.

M(3)	mov	a,m	; check length of string

ora	a	; if zero

rz		;  then return

mov	c,a	; save length in counter C

inx	h	; get string address

mov	a,m	; that's the low byte

inx	h

mov	h,m	; that's the high byte

mov	l,a	; HL --> string

lxi	d,2	; here comes the tricky part

call	31E9	; in ROM there is XTHL, PCHL

$PC1	dad	d	; now HL = loop

pop	d	; and DE --> string

loop	ldax	d	; A = char from string

call	0FE9	; let ROM convert to uppercase

stax	d	; restore converted char

inx	d	; point to next char

dcr	c	; decrement counter

rz		; done when counter = 0

pchl		; otherwise jump to loop

; ENTRY M(16) - check MDM: or COM: queue for received chars.

;  calling sequence: CALL VARPTR(M(16)),0,VARPTR(X(0))

;  where X(0)..X(1) is an integer array.

M(16)	mvi	m,0	; set flag, X(0)=0, for no char

call	6D6D	; check queue for chars

rz		; return if no chars available

inr	m	; set flag, X(0)=1, for char received

inx	h

inx	h	; HL --> received char buffer, X(1)

nop

; ENTRY M(21) - retrieve char from MDM: or COM: queue.

;  if entry was made at M(16) and the queue contained a char

;  then the code will fall through to this point and the

;  received char will be placed in X(1).

;  M(21) may also be called independently of M(16) to receive

;  a char if it's already known that one is there (due to an

;  ON MDM GOSUB.. interrupt for example).

;  calling sequence: CALL VARPTR(M(21)),0,VARPTR(X)

;  where X is an integer variable.

M(21)	call	6D7E	; get char from queue

mov	m,a	; store it

ret

nop

; ENTRY M(24) - timed read of 131 chars from MDM: or COM: queue.

;  calling sequence: CALL VARPTR(M(24)),0,VARPTR(Z(0))

;  where Z(0)..Z(131) is an integer array.

;  Z(0) is used as a flag.

;  Upon return, if Z(0)=0 then a timeout occurred.

;  Z(0)=1 implies successful return and in this case

;  Z(1)..Z(131) will contain the 131 received characters.

x1	equ	39.	; get_ch - $PC2

x2	equ	7.	; ck_que - $PC2

x3	equ	4.	;  init  - $PC2

M(24)	mvi	m,0	; set flag for failure

mov	d,h

mov	e,l	; DE = HL --> flag

call	31E9	; xthl, pchl

$PC2	mvi	b,131.	; char counter

push	b

push	h

; stack now contains:

;

;		address of flag

;		char counter

;    sp  -->	$PC2

init	lxi	h,7282.	; HL = 1 second timeout counter

ck_que	call	6D6D	; check queue

pop	b

push	b

push	b	; extra $PC2 on stack

xthl		; counter on stack, HL = $PC2

lxi	b,x1	; offset to get_ch

dad	b

xthl

rnz		; jnz	get_ch

pop	b	; clear stack

dcx	h	; decrement timeout counter

mov	a,h

ora	l

pop	b

push	b

push	b	; extra $PC2 on stack

xthl		; counter on stack, HL = $PC2

lxi	b,x2	; offset to ck_que

dad	b

xthl

rnz		; jnz	ck_que

pop	b

pop	b

pop	b

pop	b

ret		; timeout exit

get_ch	call	6D7E	; get char from queue

inx	d

inx	d

stax	d	; store it in buffer

pop	h

pop	b

dcr	b	; decrement char counter

push	b

push	h	; HL = $PC2

lxi	b,x3	; offset to init

dad	b

push	h

rnz		; jnz	init

pop	h

pop	h

pop	h

pop	h

inr	m	; set flag for success !

ret

; ENTRY M(59) - delay A*2 milliseconds

;  calling sequence:  CALL VARPTR(M(59),A

;  where A is an integer from 0 to 127.

M(59)	mov	c,a

jmp	531C

---------------------------------------

Sysop Note: The current version of XMDM26 in the library (5-July-87) will work

WITHOUT merging the lines with the graphic characters into the program, as

instructed in the DOC file. You may also remove the statement at the end of

line 67 that says you should download the DOC file.

Sysop Tony 11/3/88