PPPPPPPP     OOOOOOO    WW           WW   RRRRRRRR

PP     PP   OOOOOOOOO   WW           WW   RR     RR

PP     PP   OO     OO   WW           WW   RR     RR

PPPPPPPP    OO     OO   WW    WWW    WW   RRRRRRRR

PP          OO     OO   WW   WW WW   WW   RR  RR   =====

PP          OO     OO   WW   WW WW   WW   RR    RR =====

PP          OOOOOOOOO    WW  WW WW  WW    RR     RR

PP           OOOOOOO      WWWW   WWWW     RR      RR

DDDDDDD      OOOOOOO     SSSSSSSS

DD    DD    OOOOOOOOO   SS

DD     DD   OO     OO   SS

DD     DD   OO     OO    SSSSSSS

DD     DD   OO     OO          SS

DD     DD   OO     OO          SS

DD    DD    OOOOOOOOO          SS

DDDDDDD      OOOOOOO    SSSSSSSS

P O W E R  -  D O S

POWR for Portable Disk Drives

Owner's Manual

Transcribed By Denny Thomas [76701,40]

Exclusively for CompuServe Model 100 Forum Members

(C) 1986 Acroatix Incorporated

All Rights Reserved

Introduction

POWR-DOS is a comprehensive utility package to access the

Tandy Portable Disk Drive (100K Version - Called "PDD" in

this manual).  At the heart of POWR-DOS is an invisible

machine-language program that enhances TEXT and BASIC.  Once

these enhancements are in place, you have complete access to

the PDD using BASIC commands.  Surrounding this invisible

program are a number of BASIC programs.  You can load these

independently of each other, and use them only when needed.

Chapters One through Six in this manual Explain how to use

the POWR-DOS utilities.  Each of these chapters consists of

three sections:

* BASIC Procedures for using the utility.  These

instructions are as brief as we could make them; they

will help you get "up and running" as quickly as

possible.

* Notes and Options to the basic procedures.  Here you

will find more detail and interesting options once you

have mastered the basics.  You may skip this section on

first reading, but you will probably want to come back

to it later.

* Errors and Recovery from the more common problems.  Turn

to this section when something does not go as expected.

We only covered the most common errors; to cover

everything would have taken volumes.

C o n t e n t s

Chapter 1: Loading POWR-DOS.......................1

Basic Procedures.............................2

Choose the Loader Program...............2

Choose POWR-DOS.........................2

Shift the BASIC Programs................3

Notes and Options............................3

When to Use IPL.BA......................3

Using TINY..............................4

Specifying an Immovable Program.........5

Errors and Recovery..........................6

Errors in PL and IPL....................6

Errors in TINY..........................7

Errors While Loading POWR-DOS...........7

Chapter 2: POWR-DOS Basics........................9

Basic Procedures.............................9

Is POWR-DOS Active?.....................9

Loading a BASIC File....................9

Examining Disk and RAM Files............10

Saving BASIC Programs...................11

Loading and Saving Text Files...........11

The Memory Eaters.......................12

Deactivating POWR-DOS...................12

Notes and Options............................13

Side Effect of LFILES V.................13

Options when Loading BASIC Programs.....13

Options with LFILES.....................13

Text File Options.......................14

Errors and Recovery..........................14

Chapter 3: Using DMENU.BA.........................15

Basic Procedures.............................16

Viewing Files...........................16

Saving a File to Disk...................17

Loading a file into RAM.................17

Killing a File..........................17

Leaving DMENU...........................17

Notes and Options............................18

Multiple Pages..........................18

Selecting Multiple Files................18

Operations with Multiple Files..........19

Running Programs from DMENU.............20

Examining Document Files................21

Errors and Recovery..........................22

Chapter 4: Formats and Backups....................24

Basic Procedures.............................24

Formatting..............................24

Backups.................................25

Notes and Options............................29

Using FORMAT.BA.........................29

Errors and Recovery..........................30

Chapter 5: Disk-Based TEXT........................31

Basic Procedures.............................31

Accessing Disk Files....................31

Printing a File.........................32

Editing a Small File....................32

Appending to a Large File...............33

Modifying a Large File..................34

Notes and Options............................36

Adding New Print Formats................36

Editing Options.........................37

Accessing the Disk in TEXT..............37

Errors and Recovery..........................38

Chapter 6: Data Recovery..........................39

Basic Procedures.............................39

Notes and Options............................41

How RECOVR Works........................41

Deciding the File Type..................42

Errors and Recovery..........................43

Chapter 7: Command Descriptions...................44

DSKI$........................................45

DSKO$........................................47

LFILES.......................................49

LFILES FORMAT................................51

LFILES OFF...................................53

LFILES V.....................................54

LOADM ,F.....................................55

MAXRAMC......................................57

SAVEM........................................58

Chapter 8: Programming Tips.......................61

Side Effects.................................61

Re-starting Programs.........................62

Machine-Language Calls.......................63

Error Codes..................................66

Appendix A: Your Portable Disk Drive..............68

Appendix B: Reserved Memory.......................71