EPSON.DOC

EPSON.SET is a versatile, user-friendly, general purpose printer set up program

for the EPSON FX80 printer.  It is command driven with automatic command

completion and verification.  It may be used alone, or as one of the

cornerstones of a powerful customizable wordprocessing system comprised of Jim

Irwin's PRN100, the EPSCHR.GEN custom character genreator, and the EPSCHR.100

program for loading the Model 100's CHR$(128)-CHR$(255) into the FX80's RAM.

This package gives the Model 100 full math/science word processing capabilities

including Greek, math/science characters, custom characters, super/subscripts,

and many other features.  The package takes only about 5K of ram when loaded.

The following files from this SIG are required:

EPSON.SET         EPSCHR.100

EPSCHR.GEN        EPSGR.EEK

plus Jim Irwin's

PRN100.BAS        PRN100.DOC

PRN100.LDR        PRN100.MOD

PRN100.OBJ

INSTALLATION INSTRUCTIONS

1.  Use the PRN100 files to load the machine language print program PRINT.CO as

per Jim Irwin's instructions in PRN100.DOC  Edit line 20 of PRN100.BAS to set

I=59850 so as to allow ample space for your custom printer control codes, then

run PRN100.BAS to enter control codes as described in PRN100.MOD  I suggest the

following codes for math/science word processing applications:

^B+ 27,69                  Bold-on

^B- 27,70                  Bold-off

^D+ 27,71                  Doublestrike-on

^D- 27,72                  Doublestrike-off

^E+ 27,52,27,37,0,0        Italic-on

^E- 27,53                  Italic-off

^F+ 27,83,1,15,27,65,6     Fineprint-on

^F- 27,84,18,27,50       Fineprint-off

^H+ 8,27,106,12,94,27,74,12   Backspace, reverse 1/18" and

place ^ over previous character

^H- 8                      Backspace

^L+ 27,74,18               0.5 linefeed (1/12")

^L- 27,106,18              0.5 reverse linefeed (-1/12")

^R+ 27,37,1,0              Select custom Greek/Math characters

set in printer RAM

^R- 27,37,0,0              Select standard character set in

printer ROM

^S+ 15,27,83               ^S+1 selects subscript and

^S+0 selects superscript

^S- 18,27,84               Sub/super scripts-off

^U+ 27,45,49               Underline-on

^U- 27,45,48               Underline-off

NOTE:  Typestyle is assumed to be pica and linespacing 6 lines/inch for these

printer control codes.  Unpredictable results may result from other other

settings.

2.  Save EPSON.SET as in RAM as SETFX.BA  (it replaces SETFX.BAS if you have

previously downloaded it from this SIG).

3. Load EPSCHR.GEN and save as FXGEN.BA (it replaces FXGEN.100).  Download

EPSGR.EEK into the file CHAR.DO.  It contains data for loading the Greek

alphabet to the FX80.  Create and add any additional custom characters which

you desire to the file CHAR.DO using FXGEN.BA

4.Type the BASIC command CLEAR256,57000 to clear (more than enough) space for

the GREEK/MATH custom character data to be stored as a machine code data file.

Then, run option 6 on the menu of FXGEN.BA to copy the custom character data in

CHAR.DO into a compact machine code data file (requiring only about 13 bytes

per character).  The program will tell you the top address and end address used

by the machine code character data file.  (Note: the end address will be 59849,

just above the top address 59850 of program PRINT.CO created in step 1 above.)

5. Enter the BASIC command

CLEAR256,topaddress

where topaddress is the top of the custom character machine code data file

obtained in step 4.

6. Save PRINT.CO together with the machine language custom character data and

the custom printer control codes to tape in a single file with the command

SAVEM"PRINT",topaddress,62972,60503

where topaddress is again the top address of the custom character data obtained

in step 4.

6.  Download the basic program EPS100.CHR, load and save as MOD100.BA

This program, which is called by one of the options of SETFX.BA, reads the data

for the Model 100's special character set directly from the screen, then

calculates the equivalent FX80 data and loads it into the FX80's RAM as CHR$(

128)-CHR$(255).  It takes only about 90 seconds to do all 128 characters.

If no further custom characters will be generated for awhile, the files CHAR.DO

and FXGEN.BA may be saved to tape and killed now, leaving the the following

files for math/science word-processing with the Model 100:

SETFX.BA       MOD100.BA       PRINT.CO

If you like, you may also kill PRINT.CO as it is now called directly by the

machine code CALL 60503 from the main menu of SETFX.BA

USE OF EPSON.SET AND PRINT.CO FOR MATH/SCIENCE WORDPROCESSING

Mathematical formula including subscripts may be readily entered in TEXT-mode

using ^P (i.e., control-P) to insert the printer control codes to shift between

the printer's RAM and ROM for Greek and special math symbols,to insert super/

subscripts, to forward and reverse linefeed and so forth.  For example, a

text.DO file containing

x^L-^H-.^L+ = f^S+1y^S-(^R+g^R-)

would print as the mathematical equation "x dot equals f-sub-y of gamma" when

the SETFX.BA menu option GREEKCHR$ is used to first load the Greek alphabet

into the printer's RAM and the SETFX.BA option PRINT is used to call Jim

Irwin's machine language PRN100.  The equation "y-hat equals integral from

alpha to beta of f(x)dx" might be entered as

y^H+ = ^R+(GRPH-i)^S+1a^H-^L-b^L+^R-^S-f(x)dx

Mike Safonov

3606 Barry Ave.

Mar Vista, CA 90066