The M100 CTS(5) and DSR(6) pins can be

checked from BASIC for hardware

handshaking during RS-232 (COM:) data

output from the Model 100 to another

device by testing the value returned

by the IMP(187) function.

When CTS is low at the 100, A% in the

expression  A%=INP(187) will be either

242 dec or 243 dec; when CTS is high,

it will be either 226 dec or 227 dec.

For DSR the values are 242 or 243 low

and 210 or 211 high.

A sample routine to print the string

S$ (with COM:xxxxx open on channel 1)

by testing CTS is:

100 FOR I%=1 TO LEN(S$)

110 IF INP(187) AND 16 THEN 110

120 PRINT#1,MID$(S$,I%,1);

130 NEXT

140 PRINT#1," "

150 RETURN

Summary:

IF INP(187) AND 16 = false (=0)

then CTS is high

IF INP(187) AND 16 is true(=16)

then CTS is low

DSR is similar (Test AND 32).

This routine works well for connecting

the Model 100 to a Brother EP22 battery

portable typewriter using 300 baud and

sending lines no longer than 75

characters to the Brother EP22.

The character-by-character transmission

is for safety since INP(187) is only

checked before each print statement.

No doubt some devices will be able to

accept multi-character strings without

problems.

RS-232 connections for Brother EP22:

M100 CTS(5) to DTR(20) Brother

M100  TD(2) to  RD(3)  Brother

M100  SG(7) to  SG(7)  Brother

Other devices should be similar.

Woods Martin [70235,232]