XCOMM.DOC [9/4/89]

Copyright (c)1989 by James Yi [73327,1653]

XCOMM is a simple terminal program with Ascii, Xmodem, Xmodem 1K,

and Ymodem Batch file transfer protocols.

It is 1710 bytes and relocatable as it is loaded by the

Basic loader.  Before running XCOMM.CO, clear HIMEM by going to Basic and

LOADM the file, and then CLEAR 255,Top  where Top is the first address you see

when it's LOADMed.

It is not fully featured, like TELCOM, in order save memory space, for those

times when you might need it.

But it has all the basic neccessary functions.

Except getting the phone connected, if using the internal modem.

You can do that by exiting to Basic while online with TELCOM and

RUNM"XCOMM", or use the following simple Basic driver.

If using external modem, you can just run XCOMM by itself without doing

anything first.  It will connect with the modem all by itself.

1 'Phone dialer for Tandy 200

2 'Connects XCOMM with the internal 300 baud modem.

3 OL=25010:'Pick up phone

4 RO=25032:'Modem relay on

5 W2=25102:'2 second time delay

6 T1=36751:'Enable touch tone dial

7 T0=36737:'Disable touch tone dial

8 TD=36670:'Touch tone dial one digit

9 AC=25046:'Turn on carrier

11 AR=34526:'Activate RS232 port

12 DR=34741:'Deactivate RS232 port

13 DM=25018:'Put phone on hook, turn off modem relay, turn off carrier

20 C$=CHR$(0)+CHR$(205)+CHR$(74)+CHR$(135)+CHR$(119)+CHR$(201):C=VARPTR(C$):C=PEEK(C+1)+PEEK(C+2)*256

30 LINEINPUT "Dial: ";D$

40 CALL OL:CALL RO:CALL W2

45 CALL T1:FOR D=1 TO LEN(D$):CALL TD,ASC(MID$(D$,D,1)):NEXT:CALL T0:PRINT

50 CALL AR,,846:'Activate serial port

60 FOR W=1 TO 2000:CALL C+1,,C:IF ASC(LEFT$(C$,1))=0 THEN 100 ELSE NEXT

90 PRINT "No Carrier":CALL DR:CALL DM:END

100 PRINT "Carrier detected"

110 CALL AC:'Turn on carrier

120 RUNM"xcomm"

125 PRINT:PRINT"Logoff?";:R$=INPUT$(1):IFR$<>"y"THEN120

130 CALL DR:CALL DM

140 END

At the "Baud?" prompt when the program is first ran, type the number

corresponding to the baud rate:

0 = 300 baud, internal modem.

External modem:

3 = 300

4 = 600

5 = 1200

6 = 2400

Function keys:

F2 - Download.

F3 - Upload.

F4 - Toggle Full/Half duplex.

F5 - Toggle Echo to printer.

F6 - Goto Menu without disconnecting modem.  Then enter BASIC,TEXT, go back to

XCOMM, etc.  Warning: Exit from TEXT disconnects modem.

F8 - Exit - RETurns to where it was called from.

The function keys are not visible.

It can download or upload to and from a device that you specify with the file names. (0:file, RAM:file, file, etc.)

Download or upload can be aborted by pressing ESC key.  The program will abort,

but if the other side doesn't abort within reasonable amount of time, try

pressing CTRL-X a few times.

Holding down SHIFT while up/downing lets you see the contents of the blocks

being transferred.

Ymodem is usually faster than Xmodem, and Xmodem 1K sightly faster than Ymodem

because Ymodem transfers extra two blocks containing file name, length, date.

It will display this information when downloading with Ymodem.