VT100.100 is a program designed to emulate a 24x80 full screen terminal.

Two different brands of terminal protocols are supported:  Dec VT100 and

ADM models 2, 3, 31 and 32.

It does this by trapping the protocol control codes to modify a RAM

simulation of the terminal screen.  The LCD of the M100 is used as a window

into the larger screen stored in RAM.

The program remembers where the cursor is located even if the window

is positioned over a different portion of the RAM screen.  The window will

home in on the cursor when F5 is pressed or any other key is pressed to enter

data.  As data is entered, if the next data character is located outside the

limits of the LCD window the display will automatically home in on the new

cursor location.

Due to the size limitations of the LCD, the display will tend to become

very jumbled as the host transmitts a screen to the M100.  This is not a bug

in the program, but a design feature so that you can tell when the host has

completed its transmission.  Attempting to "view" the screen before the host

completes transmitting can result in losing characters.  Upon termination

of the host transmission the function keys can be used to de-jumble the LCD

display.

The following describes use of the function keys:

KEY   Description

---   -----------

F1    Display lines  1-8  columns 1-40

F2    Display lines  9-16 columns 1-40

F3    Display lines 17-24 columns 1-40

F4    Shift display 40 columns (left or right)

F5    Home window on current cursor location

F6    Print the RAM screen

F7    Not used  (see programming notes)

F8    Disconnect phone and return to MENU

After you customize the program for your own use the program is very

easy to use and quite capable of keeping up with the host at 300 baud.  It

should be noted, however, because the program is written in BASIC an external

modem at a higher baud rate would be wasted.

PROGRAMMING NOTES:

The variables IX and IY contain the current cursor location.  KX and

KY are the location of the LCD window.  String EA is used to detect the use of

the cursors, TAB, BKSP, DEL and other special control codes.  EB translates

the detected character to the proper control code.

IE is a switch indicating that CHR$(27) has been detected.  EY is the

control character that indicates cursor positioning data follows.  EJ is the

control code for erase to end of screen from current cursor location.

PH$ is the string used to hold the host phone number for the autodial

ROM routine.

Lines 124-134 and 164-172 may be deleted as they are specific to my

host environment.  PH$ is initialized at line 118 to send a CR on carrier

detect to cause my host modem to shift down to 300 baud.  You may want to

change this to some other character sequence for you application.  NOTE:

do not send anything that would cause the host to transmit characters until

program initialization is complete at line 174.  The ROM routine lockup if

too many characters are recieved because there is no input buffer until the

dial operation is complete.

Line 140 should be modified with you host system phone number.  Lines

152-155 contain the MDM stat parms.

Line 102 was included so that you can define an operation for F7.  The

current instruction sequence is essentially a no-operation.