FASTMODEM - A software modem speed up for the Model 100 : Bob Scott

73125,1437

This file describes the use of FSTMDM.100, a program which allows you to

speed up the M100 modem by 50% to 450 baud.  Some CIS direct dial numbers

support this rate (use GO PHONES to check), however it is NOT supported by

TeleNet or TYMNET.  Of course, two M100's can use this rate to communicate

with each other over a dialup circuit.

Cautions:

This program nastily grabs and clears high memory.  It also reassigns the

TELCOM f7 key to give you a readout of bytes free while you're online.  f6

is reset as described below.  This program CANNOT be used with an autologin

macro of any type (yet).  If you can put up with these "features", it

really does speed up the internal modem.

Theory:

The M100 modem can be pushed to as much as 600 baud over standard phone

lines.  Since CIS thoughtfully supports 450 baud, it is possible to

communicate at that faster rate with an M100. The program FASTMDM loads a

machine language routine above HIMEM and assigns the online TELCOM function

key F6 to call the routine.  When executed, the M/L program resets the

internal baud rate generator to 450 baud.  Since TELCOM sets the rate to

300 when it shifts to online mode, you must press F6 after going online,

but BEFORE typing anything(hence the restriction on login macros).  If you

type anything before pressing F6, CIS will sense that you are at 300 baud,

and will not let you change to 450.

Use:

Clear any machine language programs out of high memory.  Place the cursor

over FASTMDM and press RETURN.  You will see the screen flash "Installing

450 bps" then get the menu back.  You may now enter TELCOM and dial CIS

manually, or with a number from ADRS.DO, just so long as it is followed by

only a "<>" (remember, NO login strings!).  After you get connected,

indicated by the label line chaning to the online mode ("Up Down" etc.) and

BEFORE typing anything press F6.  You'll hear a beep, which indicates that

you are in 450 baud mode. You can then hit "ENTER", login and enjoy.

After you finish your TELCOM session, you can remove the machine language

routine, and free up high memory for other uses, by again executing FSTMDM

from the main menu.  You'll see the legend "Removing 450 bps.." and be

returned to the main menu.  (The TELCOM F6 key will be set to a CLEAR

SCREEN function by the program when it deinstalls).

Reference:

For you hackers, the M/L routine is:

DI

PUSH PSW

MVI A,85

OUT 188

MVI A,65

OUT 189

MVI A,195

OUT 184

CALL 16937

POP PSW

EI

RET

Permission for non-commercial use granted to M100 SIG members.  All other

rights reserved.