Remote PC Terminal Emulation Using the Radio Shack Model 100

with PCAnywhere (Version 2.1)

DISCLAIMER:  THE USER OF THIS FILE ACKNOWLEDGES THAT ALL INFORMATION

OBTAINED FROM THIS FILE IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES

OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE AND THAT THE ENTIRE

COSTS OF ALL NECESSARY REMEDIES IS WITH THOSE WHO CHOOSE TO ACT ON SUCH

INFORMATION AND NOT WITH THE AUTHOR OF THIS FILE.  THE LATTER PROVIDES THIS

INFORMATION SOLELY AS A PUBLIC SERVICE WITHOUT MONETARY COMPENSATION WHATEVER

TO MEMBERS OF THE COMPUSERVE MODEL 100 SIG AND TO THE PUBLIC DOMAIN.  TO THE

BEST OF MY KNOWLEDGE THE INFORMATION BELOW IS ACCURATE, BUT I HAVE MADE

MISTAKES BEFORE SO USE IT AT YOUR OWN RISK.

Within the past year, a new software product, called PCAnywhere (Dynamic

Microprocessor Associates Inc., 1986) has been introduced which is billed as

allowing almost any computer or ASCII terminal to be used as a remote terminal

for an IBM PC compatible computer.  It accomplishes this by translating

the keyboard escape sequences and screen attributes of the remote

computer or terminal so as to emulate an IBM PC keyboard and display, allowing

it to control most PC functions remotely over the telephone lines.  Set up

properly, many non-IBM compatible computer or ASCII terminals can

be made to run popular but memory intensive programs such as Lotus 123,

Dbase III+, Websters New World Spell Checker, Wordstar 3.3, and others.

My motivation for experimenting with the PCAnywhere package was to see if

this communication package could be used with the Radio Shack Model 100,

equipped with Disk Video Interface, to emulate an 80X25 IBM PCXT display and/or

an 80X24 ASCII terminal to run popular MSDOS programs remotely which otherwise

would not run on the Model 100 due to its incompatible 8 bit Z80 based

operating system and its severe memory limitations (ordinarily a max of 32 k).

For this purpose, I have used the following equipment:  a 32k Model 100, DVI,

Worldport 1200 baud external modem & cable; Ultra Computer 640k PCXT clone

equipped with 20 mb Seagate ST225 Hard disk, OMTI controller, 2400 US Robotics

Direct Internal Modem; X10 remote telephone switch and remote responder.  With

this hardware appropriately configured to allow communication over ordinary

telephone lines between the two computers, I have been able to turn my computer

on and off remotely and to produce a PC emulation which seems quite adequate

for my purposes.  Programs for which I have obtained satisfactory results

to date include Lotus 123 (version 2), Dbase III+, the Websters New World

Spellchecker, File Express, Wordstar 3.3, and In Control Project Manager.

Programs which have not worked satisfactorily have included all

telecommunication packages as well as memory resident programs which use

interupt 16 on the PCXT in a manner which is incompatible with its use in the

PCAnywhere program.

In this report, I will be limiting myself solely to addressing special

considerations in customizing the RS Model 100/DVI combination for use as a

remote terminal for a PCXT compatible computer.  The reader is referred to the

PCAnywhere documentation for specific instructions on the theory, operation,

and use of the PCAnywhere program.  In order to use the PCAnywhere

communications package to configure a non MSDOS compatible computer or

terminal, such as the Model 100/DVI combination, to run as a remote terminal

for a PCXT compatible, you must first run the AINSTAll program remotely from

the Model 100 with DVI.  To do this, the M100/DVI must be able to

support one of the numerous terminal emulations supported by PCAnywhere.

Although this fact is nowhere documented in either the Model 100 or DVI

manuals, the DVI apparently supports the DEC VT-52 terminal emulation since the

major DVI escape codes correspond to all the major VT-52 escape sequences.

Therefore, when running AINSTALL on your PC select the VT-52 terminal option.

It is then simply a matter of entering the specific DVI screen attribute and

cursor movement escape sequences from the DVI manual into the appropriate

prompts from the AINSTALL program menu according to the instructions contained

in the PCAnywhere manual (See addendum for the DVI Escape Codes to be used).

In addition to modifying the screen attributes, I recommend modifying the

prearranged PCAnywhere keyboard sequences, which are used to emulate the PC

keys and key sequences, to take full advantage of preexisting keys and key

sequences on the Model 100.  For example, I would recommend customizing the

predefined PCanywhere keyboard combinations to make full use of the pre-

existing M100 cursor movement keys, which the PCanywhere AINSTALL program allows

you to do in its "change keyboard sequence" option.  For example, you can

replace the predefined PCanywhere 3 key sequences for PgUp and PgDn to the

much less awkward 2 key combinations on the Model 100 of ESC - up cursor and

ESC - down cursor respectively.  I also like to substitute the 2 key ESC-tab

combination on the Model 100 for the predefined PCanywhere 3 key sequence in

order to refresh the screen when I occasionally get garbage characters.

Special consideration needs to be paid to a problem posed by the

the way the Model 100 uses the 25th line (Status line) of the DVI display.

Most IBM computers use 25 row displays rather than the 24 row display

used by most non-IBM computers and terminals.  The Radio Shack DVI does support

25 rows; however, the 25th row is typically reserved for the function key

status line which you ordinarily toggle out of by depressing the "label" key.

Unfortunately, simply configuring the AINSTALL program menu for 25 rows and

then toggling out of the status line does not allow the DVI to function

properly.  Nor does configuring PCanywhere for 80 columns and 24 rows

necessarily solve the problem.  Apparently different MS-DOS applications write

to the screen using PCanywhere in ways which use the 25th (bottom status) line

very differently (sometimes even within the same application!).  This often

results in overwrites to the wrong line and resulting garbage on the screen.

While PCAnywhere provides a screen refresh key to eliminate such garbage on

the screen, this can be an annoying and time consuming inconvenience at 1200

baud, which is the baud rate I operate at for reliability.

The simplest away out of this dilemna is to use a DOS shell program in

running PC programs on your hard disk remotely from your M100/DVI.  This

apparently standardizes the screen output in applications run from the

DOS shell program.  I have experimented with the shareware program Hard Disk

Manager II and thus far have been able to write all major applications to

the DVI screen with no further difficulty.  I have discovered, however, that to

do this I must first toggle out of the Tandy function line before enterring

the HDM2 DOS shell program.  I have accomplished this by calling a batch

file from the PCAnywhere initial command line which prompts for a pause

in the execution of the batch file.  At the pause I then toggle out of

the M100 status line by depressing the label key before enterring HDMII

automatically from the batch file.   You might want to take this opportunity

to execute a program like MJOG (memory jogger -- a calendar reminder), as I have

done, before the pause command in your batch file (useful but not necessary).

Other useful functions of the PCanywhere program, as implemented on the

Model 100/DVI combination, include the ability to toggle between the upper 24

lines of the display and the lower 24 lines of the display where necessary

for a particular application, remote printing through the serial port of the

Model 100 (must use 300 baud internal modem for this), and password protection

including the use of password tables so that different users may access your

computer using different passwords, terminal emulators, baud rates etc. (helps

for customer support).  If you get disconnected in an application, PCanywhere

has the ability to return you to the exact point you left off in an

application before you got disconnected by dialing back in and logging back

on.  If you freeze up the remote keyboard, that's where the x10 remote

telephone responder comes in handy for a cold boot.  Although PCanywhere`s

Control-Alt-Del warmboot feature does not work on the Model 100/DVI, it

is a relatively simple matter to call a warm-boot utility directly from a

batch file on your PC which then accomplishes the same task from the PC end.

Although the PCanywhere ASEND remote file transfer program does not work

with nonIBM compatible terminals, it is possible nonetheless to transfer files

over the telephone lines by toggling to another communications program which

contains a host mode.  My personal favorite is PROCOMM 2.42 which has a

host mode which supports x-modem file transfers and DOS system access.  In

order to get from PCanywhere to Procomm, I create a batch file which invokes

another favorite IBM utility of mine called reconfigure.com.  This program

allows you to reconfigure your autoexec.bat and config.sys files after

rebooting (warmbooting) the system.  This batch file warm boots my system,

reconfigures my autoexec.bat and config.sys files, and then automatically

invokes PROCOMM host mode using autologon commands from within

PROCOMM.  I can then transfer my files between my computers, enter DOS from

within PROCOMM, and then invoke a second batch command to reconfigure my

system a second time to warm boot my system and return me to PCanywhere.

This, of course, requires that you hang up the phone each time your reboot.

To date, I have been unable to configure either the m100 or T102 alone,

with or without the Ultimate ROM II 80 column emulator, to emulate

satisfactorily my PCXT clone using the PCAnywhere package.  Sorry, folks, but

the DVI appears to be the best way to use the M100 for this purpose.  I have no

experience with the T200 or T600 and thus have no way of knowing if their VT-52

emulation modes might be used for this purpose.  Also, I have been unable

to get the VT100.100 emulator program to work properly with PCanywhere on M100

at 300 baud (lack of reverse video capability is a real drawback here).

I would appreciate any comments or experiences from others in the SIG

who may be experimenting along similar lines.

ADDENDUM

Suggested escape codes for PCanywhere AINSTALL program options using the

Tandy Model 100/Disk Video Interface combination and other parameters.

*Numbers given refer to ASCII equivalents of Escape codes for various

screen attributes.  You may want to experiment with these as I have.

CURSOR UP                               27  65

CURSOR DOWN                             27  66

CURSOR RIGHT                            27  67

CURSOR LEFT                             27  68

CURSOR HOME                             27  72

CURSOR SCREEN                           27 106

CLEAR TO END OF LINE                    27  75

CLEAR TO END OF PAGE                    27  74

INSERT LINE                             27  76

DELETE LINE                             27  77

POSITION LEAD IN                        27  89

NORMAL VIDEO                            27 113

HIGHLIGHT VIDEO                         27 113

REVERSE VIDEO                           27 112

REVERSE HIGHLIGHT                       27 112

BLINK VIDEO                             27 102

BLINK/HIGHLIGHT                         27 102

BLINK/REVERSE                           27 102

UNDERLINE/BLINK                         27 102

UNDERLINE/BLINK/HIGHLIGHT               27 102

UNDERLINE/BLINK/REVERSE                 27 102

SEND ROW OR COLUMN FIRST                  R

OFFSET ROW                                32

OFFSET COLUMN                             32

TERMINAL WIDTH                            80

TERMINAL HEIGHT                           24

TERMINAL EMULATION                      VT-52

Good luck!

End of file