DISS.BA

Copyright 1985, James M. Irwin

All Rights Reserved

Compuserve ID: 72346,1020

DISS.BA is an ultra-compact (1.3K) BASIC disassembler for the Model 100 or

TANDY 200.  It allows disassembly to the Screen or Printer.  With a slight

modification, you can send the output to RAM or cassette if you desire.

Because all PEEKS,POKES, and CALLS are in decimal, this disassembler operates

only in decimal.  You certainly can add a hex conversion routine if you like.

The program will run with as little as 1.1K free, but runs better with about

1.7K free.  But the beauty of the program is that it only takes 1.3K while it's

not running!  DISS.BA stays in my computers almost all the time.  I can't do

that with a larger program.

I defined the RST instructions with their actual call address.  An Intel RST 2

is RST 16 in this disassembler.  Similarly, RST 7 is RST 56 here.  Also, in the

Tandy ROM, a RST 56 always uses the next byte.  So the disassembler assumes RST

56 does that, too.

PROGRAM OPERATION:

Download SMALL.DIS and load it into BASIC.  (I assume anyone interested in this

program knows how to do that.  If you need help, send me a message or E-Mail.)

CAUTION:  Most of the program lines are longer than 79 characters.  The program

has lines 1-12 in it.  Delete all the stray carriage returns before loading

into BASIC.  The listing below will help you find the ends of the program

lines.

After loading, save the program as DISS.BA (or anything that you like).  Run

the program from the main menu just like any other BASIC program. After the

program starts to run, there will be about a 5 second delay while the program

is initializing.  Then the prompt "Start?" will appear.  Enter the starting

address of your disassembly.  Then the prompt "Print?" will appear.  Enter a Y

if you want to print.  Otherwise, just press ENTER and the output will go to

the screen.

OPTIONS WHILE PROGRAM IS RUNNING:

When sending output to the screen, the program waits for a keypress before

going to the next screen of information.  Printer output is continuous except

for a form feed between pages.  At any time, pressing "R" goes immediately to

the "Start?" prompt.  Then you can enter a new address for disassembly.

Pressing "M" or F8 exits the program and goes to the main menu.

The program needs a "garbage collection" about every 100 locations of

disassembly.  This causes the program to pause for a few seconds.  You can

increase the time between garbage collections by increasing the CLEAR statement

in line 1.

The program has been severely compressed to make it so small.  It's very

difficult to follow.  So I exploded the listing for you and gave a line-by-line

description of the program.

The two modifications for TANDY 200 are described below as well.

If anyone has any ideas to compress this further, then let me know and I will

UPL a new version.  Enjoy!!

Jim Irwin

1 CLEAR 1500:

DEFSTR N-S:

DIM N(255):

GOSUB 9

Line 1 dimensions string space, defines string variables, dimensions N for all

the mnemonics, and executes the subroutine that defines the mnemonics.  Change

CLEAR1500 to CLEAR1000 if you are short on memory.

2 R="":

CLS:

INPUT "Start";I:

M=7:

O="LCD:":

INPUT"Print";S:

IF S="P" OR S="p" THEN O="LPT:":

M=49

Line 2 initializes things.  For TANDY 200 use, change M=7 to M=15.

3 OPEN O FOR OUTPUT AS 1:

L=0:

CALL 16959:

CLS

Line 3 opens output file.  For TANDY 200, change CALL16959 to CALL20318

(prevents scrolling).  You may modify OPENO to OPEN"name" if you want the

output to go someplace other than the screen or printer.

4 GOSUB5:

B=VAL(N(A)):

IF B=0 THEN S=N(A):

GOSUB 7:

GOTO 4 ELSE S=MID$(N(A),2)

+STR$(256*(B-1)*PEEK(I+2)

+PEEK(I+1)):

GOSUB 7:

S="":

FOR C=1 TO B:

GOSUB 5:

GOSUB 7:

NEXT:

GOTO 4

Line 4 does all the work.  GOSUB5 gets the value of the current memory

location.  N(A) is the mnemonic for that value.

If the mnemonic is preceded by a 1 or 2 (see DATA statements 9-11), then the

VAL(N(A)) will be 1 or 2.  Otherwise, the VAL(N(A)) will be zero.  If the

VAL(N(A)) is not zero, then the mnemonic requires data from the next 1 or 2

memory locations.  The ELSES=MID$.... line sets that up.  If B is 2 then you

will get a 2-byte value.  If B is one, then B-1 is zero, and that results in a

1-byte value.  The FOR...NEXT loop simply prints the memory address, contents,

and the CHR$(contents).  Again, it will go thru the loop once or twice

depending on the value of B.

5 A=PEEK(I):

Q="":

IF A>32 AND A<127 THEN Q=CHR$(A)

Line 5 gets the contents of the current memory location.  Q becomes the CHR$(A)

as long as A is at least 33 but less than 127.

6 IF R="R" OR R="r" THEN CLOSE:

GOTO 2:

ELSE IF R="M" OR R="m" THEN MENU

ELSE RETURN

Check R to see if you want a new Start Address or if you want to return to

MENU.

7 R=INKEY$:

PRINT#1,USING"#####   ###

\          \  !";I,A,S,Q:

I=I+1:

L=L+1:

IF L>M THEN L=0:

IF O="LCD:" THEN R=INPUT$(1):CLS

ELSE LPRINT CHR$(12);

Gets R.  Prints Mnemonic.  I is current address, A is contents of I, S is the

mnemonic, and Q is the CHR$(A).  L is the line counter.  If the counter is

greater than M, then two things can happen.  If output is to the screen, then

the program waits for a keypress.  If the output is to the printer, then a Form

Feed command, CHR$(12), is sent to the printer to get to the next page.  No

pausing between pages.  Add R=INKEY$(1) to the end of line 7 if you want to

pause between pages.

8 RETURN

Lines 9-11 are just long DATA statements defining the 8085 mnemonics.  You can

get those from the actual program, so I didn't list them here.  To help you

find extra carriage returns:

Line  9 ends with 1MVI D

Line 10 ends with INR L

Line 11 ends with CMP

12 FOR I=0 TO 63:

READ N(I),N(I+192):

NEXT:

S="BCDEHLMA":

K=64:

FOR I=1 TO 8:

READ R:

FOR J=1 TO 8:

N(K)="MOV "+MID$(S,I,1)

+","+MID$(S,J,1):

N(K+64)=R+" "+MID$(S,J,1):

K=K+1:

NEXT:

NEXT:

N(118)="HLT":

RETURN

Line 12 sets up all the mnemonics.  First, it just reads all the mnemonics in

data statements 9-11.  Then it sets up the MOV, ADD, SUB, etc, mnemonics using

the FOR...NEXT loops.  118 would be MOV M,M but Intel chose to make that the

HALT command.  So I define N(118) and return.