SCRDMP is a graphic screen dump utility for the LP VII, DMP100 or other printer

utilizing the 7 bit graphics character system.  The program is merged at the

end of any program to provide graphics capability, typically you would set it

up to be called by a function key (ex. ON KEY GOSUB 63000).  Please observe the

following notes:

1) Don't duplicate line numbers used in this subroutine.

2) This sub uses RESTORE statement, if you are using DATA statements elsewhere

in your program, be sure you are set up to RESTORE your data pointer after

execution of the routine.

3) The Graphics mode and Print mode control characters work with my LP VII

printer, verify that they are the correct ones for yours.

4) Actually, the program prints the first 63 lines of pixels, if the last row

is that important to you, another pass could be added.

Advise me of any comments or suggestions.  (J. Holovacs,95 King George Rd.,

Warren NJ 07060)