DSKMGR - A Enhanced Disk Manager for the Portable Disk Drive

DSKMGR.CO  is  a  machine-language  program   which  provides  routine

disk/file  management  capabilities for the Model 100 and the Portable

Disk Drive 1. The program resides between 60700-62800 during operation

and  executes  at 60700.  The area between 62800 and MAXRAM (62960) is

used by the program for buffers and temporary storage.

DSKMGR is designed for users keeping a copy in low memory and LOADMing

for actual use only. As a result, DSKMGR CLEARs the area between HIMEM

and MAXRAM on exit to the Model 100's Main Menu.  [ A modification  to

disable this capability can be selected when the .CO file is created.]

Due to the number of ROM calls used  by DSKMGR to reduce program size,

it is currently implemented for the  Model  100  only.  If  the  press

releases  can  be  believed, DSKMGR should work with the new Model 102

but I have not confirmed that myself.   At the  present  time,  DSKMGR

does  not  work  correctly  with the new TDD-2.  If enough interest is

expressed, DSKMGR may be updated later for the TDD-2.

DSKMGR operates much like FLOPPY.CO  (the software  provided  with the

drive) with the following exceptions:

1) DSKMGR  does not  contain  the  Backup  or  Rename  functions.  The

primary  design  consideration of DSKMGR  was  size and the limited

requirements for these utilities led to the decision not to include

them as part of the main program.

2) The program's  Menu  operates  similarly to the system's Main Menu.

Files are selected  for  saving  by  highlighting  with  the  arrow

keys/space bar and then pressing <ENTER> or F3.  One of FLOPPY.CO's

major  drawbacks  is  the  need  to  type in all file names with no

file listings to work from.

3) The List function gives the user an  opportunity at the end of each

screen to name a file  for loading from the disk.  Pressing <ENTER>

alone will continue the listing process.

4) DSKMGR  was  written  to  overcome  some  inherent  weaknesses  in

FLOPPY.CO,   specifically  size  and  a  lack  of   'user-friendly

interface'. However, new user comments  and  message threads  here

and elsewhere have led to two additional significant differences--

1)  .BA file loading by DSKMGR is compatible with SUPERA and other

position-dependent BASIC program files;  2) DSKMGR will not 'grab'

your telephone line, except upon exit to the M100 Main Menu.

DSKMGR FUNCTIONS

================

[F1] List - Lists  the names and sizes of all the files on the current

disk. Files are listed in groups of 10 and the number of free bytes on

the disk is given with the last group.   The user is given a chance at

the end of each group to name a file for loading from disk.   Pressing

<ENTER> alone will continue the listing.

[F2] Load - Transfers a file  from  the  disk  to  RAM.  The  user  is

prompted for the file's disk name  and given an opportunity to give it

a different name for RAM.

DSKMGR v3.02                                                    Page 2

[F3] Save - Transfers  a  RAM  file  to disk.  The file is selected by

highlighting its name on DSKMGR's menu, using the cursor exactly as it

is on the Model 100's Main Menu. The user is then given  a  chance  to

rename the file for storage on disk. Pressing  <ENTER>  alone  at  the

"Name on disk:"  prompt  will cause DSKMGR to use the file's RAM name.

Pressing SHIFT/BREAK or CTRL-C  will  abort the  Save.  <ENTER> may be

used instead of F3 to activate the Save routine.

[F4] Sval - Automatically saves all visible  RAM  files to disk.   The

user is given a chance to rename each file just before it is saved  to

disk.   Pressing <ENTER> alone will cause DSKMGR to use the file's RAM

name.  Pressing SHIFT/BREAK or CTRL-C will abort the rest of the  Save

All routine.

[F5] Eras - Deletes a file from the current disk. The user is prompted

for the name and asked to confirm the action.

[F6] Kill - Kills the RAM file currently highlighted on DSKMGR's menu.

The user is asked to confirm the action before it is carried out.

[F7] Frmt - Formats  the  disk currently in the drive.   The  user  is

warned that any data on the disk will be lost and asked to confirm the

action.

[F8] Menu - Exits DSKMGR and returns to the Model 100's Main Menu.

<ENTER> - Saves the file currently  highlighted on  the  menu.  May be

used instead of F3.

DOWNLOAD & INSTALLATION

=======================

Due to the large number of questions  surrounding  the  hex-conversion

method  of  downloading  previous releases of DSKMGR, I have chosen to

make this version available as a BASIC program which will  create  the

.CO  program  file.  This has the added attraction of allowing certain

user options to be specified at the  time  DSKMGR  is  created.   This

change  should  alleviate  the other major source of problems -- using

options that were previously carried in DSKMGR.ADD.

A quick word here about downloading... I highly recommend using a file

transfer protocol to get DSKMGR.302 rather than a simple ASCII capture

download.   The Christensen protocol  (called XMODEM after the program

where it was first implemented) is  widely available, well-documented,

and  very  reliable.  DSKMGR.302  has  an  internal checksum that will

verify the accuracy of the .CO file.  If it reports a checksum  error,

DO  NOT  try  to  run  DSKMGR.CO anyway!  You will almost certainly be

buying your Model 100 a round-trip ticket to the frozen North and your

memos and spreadsheets will be the baggage that gets lost on the way.

After downloading DSKMGR.302, load it into the BASIC  interpreter  and

run  it.  The  addresses  where the program resides will be printed on

the screen as each byte is moved into place.   When the screen clears,

the program will ask about three optional changes that can be made.

DSKMGR v3.02                                                    Page 3

As written, DSKMGR is self-clearing, i.e., on exit the variable  HIMEM

is  reset to MAXRAM.  This frees the 2.2K of RAM used by DSKMGR during

execution.  As a result of limitations in the  8085  assembly-language

command  set,  .CO  files  in the Model 100 must be loaded to specific

addresses  before  execution.   Since  the  RAM  just below the system

variables is the last to be used for file  storage,  most  programmers

choose this area to assemble their programs.  This also means most .CO

files will conflict with each other.   By clearing 'high memory' after

use,  DSKMGR  insures  its code is not corrupted accidentally by other

programs and then executed.

Some  users  have  memory  management techniques that assume .CO files

will reside permanently above HIMEM.  For those users,  DSKMGR  offers

an option to defeat its self-clearing routine.  To do so, just respond

'N' when asked 'Self-clearing?'.

DSKMGR  uses  BEEPs to signal successful completion of commands and to

announce error reports.  If you  prefer  to  use  DSKMGR  without  the

BEEPs, respond 'N' when asked 'Sound?'.

Since DSKMGR was first released, other vendors have begun distributing

the drive as well.  One major difference between  the  TDD  and  these

other  drives  is  the transmission rate.  The TDD has selectable baud

rates and uses 19200 baud for normal transfers. Some of the others are

hard-wired for 9600 baud. If your drive needs the slower speed, select

'N' when asked 'High-Speed?'.

When  the  options are set, the program returns to the Main Menu.  Two

new files will be there -- DSKMGR.CO and DRIVER.DO.  The .DO file is a

program  line  to  be loaded into BASIC and saved as DSKMGR.BA.  After

creating the BASIC file, the .DO file can be deleted.   When  run, the

new BASIC program will automatically load and execute DSKMGR.CO.

FINAL COMMENTS & ACKNOWLEDGEMENTS

=================================

The reaction to DSKMGR over the past year has been overwhelming. There

have been literally hundreds of Model 100 users who have chosen to use

DSKMGR with their Portable Disk Drive.  To all of you who have written

to me with suggestions,  questions,  and words of praise, I would like

to say thank you.  Without your  help and encouragement,  DSKMGR would

never have become what it is.

However, the cost of supporting DSKMGR on-line has been enormous.  For

that reason,  I am asking users to make a donation.  Unlike  shareware

which specifies a registration fee for continued use after evaluation,

all I am asking is this:  If you are  using  DSKMGR,  please  consider

what its value is to you and then  consider  making  out a check.  The

level  of  support  I  can continue to give to DSKMGR as well as other

new program developments will depend on you.

As  with  previous versions of DSKMGR, this program is provided solely

for personal use by members of this online service.   Any other use or

distribution,  not-for-profit or otherwise,  without  prior permission

is prohibited.   No  warranty  exists,  expressed  or implied,  as  to

the  suitability  of  this  product  for  its  intended  purpose.  The

user assumes all risk arising from the use of DSKMGR.

J.K. Heilman                                   CIS ID  >   71216,1375

4577 Southland Avenue                          DELPHI  >   JKH

Alexandria, Virginia  22312-1621               GENIE   >   J.K.HEILMAN

DSKMGR v3.02                                                    Page 4

DSKMGR ERROR CODES

==================

DSKMGR uses two-letter error codes to reduce memory requirements.  The

following error messages are possible with DSKMGR:

Ok  Flashes  briefly  on  the  message  line  (Line 7)  to  indicate a

function has been successfully completed.

NR  Drive  is  not ready.  This error will be reported if the drive is

not powered-up and  connected  to  the  computer  before  entering

DSKMGR.  Check power switch and cable connection, then try command

again.

CM  Communications  error.  Results  from  a bad connection or similar

transmission problem. Check cable and try again.

AB  Command aborted.  Sent  when  BREAK or CTRL-C is pressed at a user

prompt.

FF  File not found.

AE  File already exists.

OM  Out of space.  Returned  when there is insufficient room to save a

file to disk or load a file into RAM.

WP  Disk is write-protected. The write-protection tab on the disk must

be moved to the OFF position before data can be written to it.

HT  Hardware  error.  This  is the one you don't want to get. Turn the

drive off and on,  then try the command again.  If  the  HT  error

comes up again, you got TROUBLE!

IO  Input/Output error.  This  usually  comes  up when a read/write is

attempted  with  an unformatted disk.  It will also be reported if

the disk has become corrupt.

ND  No disk in drive.

BF  Bad file type.  This error prevents reading or writing a file that

cannot be handled by the Model 100.

FE  File empty.  Prevents saving a null file to disk.