Experience with the original version of LP3OF9.CO and many requests from

fellow SIGers have caused me to rewrite the program and to produce a

version that supports Epson printers.  The graphics control codes for the Epson

printers differ so much from those for the Radio Shack machines that it was

easier to write another version than patch the first one.  All versions of

LP3OF9.CO are copyrighted, but SIG usersare authorized to copy them forpersonal

use.  Please don't use them for commercial purposes or distribute copies

to non-SIG members without written permission.

In order to use these programs to print 3-of-9 bar codes, you need a dot-

matrix printer capable of printing around 100 dots per inch and compatible

with either Radio Shack or Epson control codes.  Radio Shack printers are

placed into the graphics mode by a CHR$(18) and "condensed" resolution is

selected by a CHR$(20).  Epson printers use a CHR$(27);"L" command to go into

"dual density graphics" mode and they require the computer to communicate the

number of graphics bytes to be sent with CHR$(n1);CHR$(n2) where n1 is the LSBY

of the number and n2 is the MSBY, using 16 bit arithmetic.  If you own aprinter

that does not conform to the above requirements, contact me via the SIG or

EMAIL and I'll be happy to try to make the program work with your machine (I

love challenges).

To use the machine language program, first download either LP3OF9.OBJ (for

Radio Shack printers - Checksum=212,739) or EP3OF9.OBJ (for Epson printers -

Checksum=215,515) into a RAM file named "LP3OF9.DO".  These files are Intel

format assembler listings of the machine language programs.  You must then use

HEXLDR.100 (from DL7) or a similar loader to load the M/L into your M100.  All

lines in the listing begin with a colon (":") except the last.  The last two

lines are ":00000000" and"$".  Except for the colons and the "$", the files

should contain only those characters used in hexadecimal notation (0-9 and

A-F). Delete all extraneous characters from the file before you try to load it

with HEXLDR.

Once you have the machine language program loaded, it is convienient to

have a BASIC program to load and call it.  Alternatively, you can SAVEM it

with an Exe address (which is the same as the Top address for these programs),

and call it from the menu.  A short BASIC program that automatically clears

the correct amount of RAM and loads the program, then restores the HIMEM value

to MAXRAM when coding is through is:

10 CLEAR256,610##

20 LOADM"LP3OF9"

30 CALL610##

40 CLEAR256,MAXRAM

where "##" is 40 for the Radio Shack version and 15 for the Epson version.  I

use this approach over the menu call because it manages the memory better and

it preserves the integrity of the operating system.

These programs will print out 3-of-9 barcode from text files that contain

ONLY the characters supported by the 3-of-9 standard.  Any other characters

will produce an error condition.  The text will automatically be limited to 23

characters per line, and a carriage return will force a new line.  Up to 24

lines may be printed per 11 inch page.

LP3OF9 is intended as a demonstration of the barcode capabilities of dot

matrix printers and the Model 100.  It cannot be used by itself to print out

BASIC files, etc., because of its limited character set.  However, you may find

a reason to save the barcode you have printed.  An easy and inexpensive way to

do this is to spray the printed pages with a fixative intended to preserve

charcoal drawings. You can obtain this at art supply stores and many college

campus bookstores.  This treatment prevents the printing from smearing when the

wand is run over it.

I hope these enhanced versions of LP3OF9 will help SIG members become

familiar with the barcode system.  Someday we may be able to use it to

enter programs from the magazines, etc. Just think; no more data lines to type!

Merrill Parks

[70536,1366]