Chapter 6

Data Recovery

From time to time, the PDD will malfunction and ruin a

diskette.  By some derivative of Murphy's law, you can

predict that it will ruin a valuable diskette without a

backup.  RECOVR.BA is a POWR-DOS utility that can recover

some or all of the data.  Even better, if you accidentally

KILL a file, the chances are good that you can recover it -

provided that you realize your mistake immediately.

Basic Procedures

Before running RECOVR, prepare a diskette to receive the

data.  RECOVR moves files from the "damaged" diskette to a

destination diskette, which does not need to be empty, but

should be formatted and have sufficient room to hold the

files.

Once you have the destination diskette ready, run RECOVR.  It

will prompt you:

Insert Source Diskette...press key

Insert the diskette containing the lost data.  After a short

pause, the program will respond with one of two messages.

Either:

Directory is damaged.

or:

Ignore sectors in use (y/n)?

In the first case, the directory sector itself has been

damaged.  This is most often the case when an entire diskette

appears to be "trashed", i.e., you cannot even use LFILES to

examine its contents.  In the second case, you can look at

the diskette contents, but perhaps an important file has been

deleted or cannot be read properly.  Normally, you will

always want to ignore sectors that the directory has marked

"in use", so answer the question with a "y".

Page 39

Chapter 6              Data Recovery               Page 40

The next message will be a series of messages saying

something like "reading sector 1". Eventually, the following

screen will appear:

----------------------------------------

| -------------------------------------- |

||    POWR-DOS Data Recovery Program    ||

| -------------------------------------- |

|    Sectors 2-9    File Length Range:   |

|                   8961 to 10240        |

|    Choose Type:                        |

| ------------------                     |

||BA   DO   CO  Skip| GO                 |

----------------------------------------

RECOVR knows that there is a series of sectors that could

make a file, but it is not smart enough to know what sort of

file it is: DOcument, BAsic, or COmmand.  You can examine

each possibility and select a type, or choose to ignore the

file altogether, using [F1] to [F4].  For each different

file, RECOVR will give some data:

* For DOcument-type files, the first characters in the

file;

* For BAsic-type files, the first several line numbers;

* For COmmand-type files, the TOP, END, and EXE.

Normally, it is simple, using this data, to decide which type

of file.  (If you're not sure, see the notes in the next

section.)

Pressing [F1] to [F4] will do nothing except show you the

option; you can press them indefinitely until you can decide

which file type to choose.  Once you do decide, press [F5] to

GO.  If you choose a file type, RECOVR will ask you for a

file name.  You must specify a full name, i.e., including an

extension.  Then RECOVR will begin reading the lost file into

RAM; eventually, it will prompt you:

Enter Destination Disk...press key

Chapter 6              Data Recovery               Page 41

You may need several swaps before the file is completely

restored to the destination diskette.  Once the transfer has

completed, RECOVR will look for other "lost" files on the

diskette; once all files have been recovered or ignored, it

prints the message:

All sectors accounted for...

Recovered:  nn

Ignored:  nn

Bad:  nn

Allocated:  nn

Empty:  nn

Press Space Bar for Menu

You now have recovered all you can from the diskette.  If

there were any "bad" sectors on the diskette, you should now

consider re-formatting it.  Use it carefully until it proves

reliable, otherwise discard it.

Recovered BAsic and DOcument files will usually have some

"garbage" at the end.  You should examine them to remove this

garbage.

Notes and Options

How RECOVR Works

THe PDD divides disks into 80 sectors.  The first sector is a

directory, which keeps track of the following information:

* The active files on the disk.  For each file, the

directory also stores the beginning and ending sectors,

and the length of the file.

* Which sectors are "in use" on the diskette.  When your

computer tries to write more data to the diskette, the

PDD always uses an unused sector for the data.

Chapter 6              Data Recovery               Page 42

RECOVR takes advantage of a peculiarity of the PDD: It does

not store the precise series of sectors in the directory.

Instead, the directory only contains the number of the first

sector of the file; then, the first sector "points" to the

second, and so on.  The most common way a disks gets

"trashed" is by losing the directory; this is logical,

because the directory is being updated constantly, whenever

any file changes.  Even without the directory, though, the

other sectors (provided they are still readable themselves)

can be used to form a chain which contains a file.

There are several implications to this basic recovery

procedure:

* If a file contains a bad sector in the middle, it

cannot be fully read.  In the case of DOcument and

BAsic files, RECOVR can still construct a partial file.

* RECOVR must assume that the first sector of a file will

have the lowest number.  This is nearly always a good

assumption, except when dealing with certain cases where

a DOcument file has been appended to.

* The precise lengths of DOcument files and BAsic files

are not available.  COmmand file-lengths can be

determined from the TOP, END, and EXE values, stored in

the file itself.

Deciding the File Type

It is usually obvious which file type makes the most sense.

One will look "normal", and the other will be nonsensical or

unfamiliar.  Here are a few quick tip-offs:

* A file is almost never a DOcument file unless it

contains readable text.  Graphics characters are usually

a dead giveaway that a file is not a DOcument file.

* If there is only one BAsic line number in the first

sector of a file, or the numbers are not in ascending

order, the file is almost certainly not a BAsic file.

Chapter 6              Data Recovery               Page 43

* If END is lower than TOP, or EXE falls outside the range

of TOP and END, it probably isn't a COmmand file.  An

exception is PCSG (CT and CA) files, which will have a

TOP of 65535, and an END equal to the file length less

1, and an EXE of 63012 (for the M100).  Unfortunately,

there is no easy way to tell whether it is a CA or a CT

file except by trial and error.

If you cannot decide which type of file a sector series

should be, it is probably best to ignore it.  It might be

some leftover data from an old deleted file.

Errors and Recovery

Data recovery is more of an art than a science.  RECOVR will

work in many cases, but there are cases when it does not.  If

you cannot recover your data, you may want to save the

diskette in anticipation of receiving help from someone else

in the future; however, we cannot guarantee that you will be

able to recover anything at all.