THERML.DOC	09/24/87

[GEORGE FLANDERS 73300,2272]

Copyright 1987 - All Rights Reserved

THERML.CO is an 8085 machine language program for M-100 which quickly

formats and sends .DO files to the Radio Shack TP-10 thermal printer through

the RS-232 serial port. This unlikely, but briefcase-sized pair affords an

easy, inexpensive source of hard copy on the road. The mnemonic file is

THERML.SRC (DL2: checksum 256,902), and an Intel format hexdump of the machine

code for THERML.CO appears in THERML.HEX (DL2: checksum 80250). Use HEX2CO.100

in DL8 to create a run-able .CO version.

Probably quite a few lap-toppers will be interested in this utility,

since the TP-10 thermal printers have proliferated as peripherals for Radio

Shack MC-10 and Color Computers. With correct interfacing, the TP-10 becomes a

viable traveling companion for Model 100 laptops. Although THERML.CO has not

been tested on 102's, etc., with minor (if any) address changes, it may work.

When I bought my TP-10, the difficulties in making the two units

produce neat printouts without word-wrap were apparent. The TP-10 prints 32

characters (remembering two more) and then performs a carriage return and line

feed, during which data transmission must pause until the printer head is in

position for the next line. A "status" signal is available at the TP-10's DIN

pin 2, but that only controls the flow of bytes. Even with XON/XOFF protocol,

parts of words would still "wrap".

I set about to (1) make a proper connecting cable and (2) write a BASIC

program to format text files into lines less than 32 characters long,

interdicting the printer's automatic CHR$(13). I wired a cable with a four-pin

DIN plug on one end for the TP-10 and a small RS-232 connector on the other for

the M-100, running continuities as follows:

TP-10	M-100

-----	-----

RCV	  4	  2	SEND

GRND	  3	7 or 1	GRND

STATUS	  2	  6	DSR - The latter connection just added mechanical

strength; I had no intention of using it.

The BASIC program I experimented with created a new formatted file from

the source file, and then sent the new file to the printer. The TP-10 does

funny things with the M-100 character set below ASCII 32 and above ASCII 126.

Consequently, the sequences for detecting spaces, tabs, hyphens, carriage

returns and "illegal" characters expanded the program beyond belief for such a

simple utility. And,  although a means was provided to my end, it ran very

slowly. The obvious but challenging solution was (drum roll) MACHINE LANGUAGE!

(cymbal crash).

Unfamiliar with the 8085 instruction set and the M-100 ROM, I leaned

heavily on Bob Covington's ROM disassembly on the SIG and devoured a few of the

better M-100 books which we all know and love. Intel's manuals on the 8085

instruction set were also very helpful.

To follow this discussion, you will need a hard copy of the source code

(THERML.SRC - DL2). That file is not annotated, so we'll track through it here,

commenting on its operation. I need to preface all of this with a disclaimer:

There is usually more than one way to get an 8085 job done, given the

flexibility of its instructions. I have probably used more bytes than needed to

do some jobs. But THERML.CO zips through its task with nearly the speed of a

parallel line printer.

Assembly was done with ZBGASM.CO from Radio Shack. Not wanting to mess

up high RAM, and leaving a hundred or so bytes just below MAXRAM for things

like my Chipmunk's CDOS to play with, I chose a starting address of E000/57344.

Most disk drive users, I reasoned, keep RAM fairly open, so THERML.CO minds its

own business while BASIC operates under the new HIMEM, 57344. With the Chipmunk

on line, MAXRAM=E26F/57967. If you don't use a disk drive, MAXRAM=F5F0/62960.

In that case, assembly of THERML.CO could very nicely start higher - say

F390/62352 or even closer to MAXRAM.

EQUATES - Prefacing the actual instructions is a list of labels and

addresses referencing ROM routines used in the program. Assemblers use equate

labels as string variables to be referenced during production of the machine

code. In some cases I used labels which differ from the documented ones, but

which to me better described what action to expect in this application.

INSTRUCTIONS - E000-E004 clears the screen, and homes the cursor by

sending 0C/12 (form feed) and 0B/11 (vertical tab) to the LCD. Next, we want to

list only .DO filenames. At E005, the C register is loaded with C0 (192).

You'll recall that the specification byte of .DO files holds a value of 192

(see Page 75, "Hidden Powers of the TRS-80 Model 100").

At E007, we call ROM (R) 5970, which prints all filenames of the type

loaded in C. E00A points HL to the text string at FI (E1BA), which is the

prompt "FILE:". E00D calls R5791, which prints characters in a buffer until a

null or CHR$(34) (") is found, starting with a carriage return and line feed if

the cursor doesn't happen to be at the beginning of a line. Using print

routines like R5791 and R27B1 which is called later on, instead of one of the

others which only accept a null as the end of a text buffer, makes it possible

to save bytes by embedding the CHR$(34) just ahead of the text delimiter

required by ZBGASM.

E010 points HL to the beginning of the 48 reserved bytes, which run

from FILDIR (E1D5) through the end of WBUF at E201. You won't find E201 in the

source code, but WBUF extends through that address. Specifically, the reserved

buffer block includes two two-byte buffers FILDIR and TOPFIL which store

addresses; a 10-byte buffer NMBUF which holds the name of the file to print;

two one-byte buffers for word length (WL) and line length (LL) and finally a

32-byte buffer WBUF to hold words as we extract them from the text file. These

buffers comprise a block of 30h/48 bytes. At E013 we load that number of bytes

into B; and E015 calls R4F0A which fills B bytes with nulls.

E018 points the DE register pair to NMBUF, and E01B sets a counter of

six in the B register. (Six characters is the maximum for filenames). E01D

(LUP1) starts an involved, but user-friendly sequence, calling R5D64 to fetch a

character from the keyboard and converts it to upper case if appropriate.

Conversion is necessary, since lower case is not recognized in the subsequent

filename search routine.

E020-E024 rejects entries above CHR$(91), foiling users who depart from

the standard alphabet when naming their text files. E025-E02E intercepts

CHR$(13) and CHR$(44) ("."), since in either case we assume the user has

finished entering the filename. To be sure, we jump to a "mugtrap" at E03E-E043

(CK) which hustles us back if NMBUF is empty (in which case the counter B still

holds 6).

E02F-E033 rejects any other characters under CHR$(32). I decided not to

bother handling the DEL/BKSP or arrow keys. How much editing should we have to

do while entering six or fewer characters - with the correct spelling sitting

right there in front of us? Finally, from E034-E037 we print any legal

character (RST 4), store it in NMBUF (STAX D), increment NMBUF (INX D)and

decrement the counter (DCR B). At E038, if the counter has reached zero, we

jump to E044 (EXT). But at E03B, fewer than six legal characters have been

entered (and obviously the user still hasn't pressed ENTER or hit "."), so we

jump back to LUP1 for another character.

A much less involved way to get user input is to CALL INLIN (4644) and

read the resulting buffer at F685, but that wouldn't guarantee either all caps

or the input of the filename extension, as required by the file search routine

to be called. So, we add the .DO extension ourselves at E044-E04B. The call

MOVE (R2542) at E04A takes B bytes starting at the address pointed to by HL and

copies them into DE, which we have incremented to the next open byte in NMBUF.

At E04C, we again point DE to NMBUF. R5AA9 (SEEK), called at E04F,

compares the filename in NMBUF which includes the .DO extension) with names in

the file directory. If a bad filename has been entered, the zero flag is set

when SEEK returns (see E052). The resulting jump to E05B displays an

appropriate message: INVALID!, followed at E061 by a delay allowing time to

read it. Then we jump back to E000 (SQ1) for the user to try again.

Assuming the filename entered is valid (see E055), the DIRECTORY

ADDRESS of that file, having returned in HL, is stored in FILDIR for later

reference. Remember, the filename is also preserved in NMBUF. From E058, we

jump to E067 (SQ2).

HL is again pointed to FILDIR, and at E06A we call R5AE3 (STADR). This

routine returns with the STARTING ADDRESS of the file in HL. That's the

location in RAM where the actual ASCII file is located. At E06D-E072, we store

the starting address in TOPFIL for later use. E070 clears the screen in

preparation for echoing subsequent output.

E073-E077: now it's time to wake up the thermal printer, but not before

loading BAUD rate in H and serial transfer protocol in L. The TP-10 manual says

the device is glued to 600 BAUD. On page 86 of the M-100 manual, "4" is listed

as the code for 600 BAUD. E073 loads the H register. How to determine the

serial word configuration to load into L is explained on page 123 of Carl

Oppedahl's "Inside the Model 100". Matching bits with the requirements of the

TP-10, the needed byte looks like this:

BIT	 7  6  5  4  3  2  1  0

------------------------

CODE	 0  0  0  1  1  1  1* 1

Bits 5-7 are ignored. Bits 3 and 4 are used together to discern whether

characters are 6, 7 or 8 bits long. To get 8 bit length, bits 3 and 4 are both

set. A 1 in bit 2 denotes no parity, which obviates bit 1 (* a zero would serve

as well in bit 1). Bit 0 is set, denoting two stop bits. E075 loads the

resulting binary number 00011111 (1F hex, 31 decimal) into the L register. At

E077, the carry flag is set (STC) to be sure the RS-232 port is used instead of

the internal modem; and R6EA6 (INZCOM) is called at E078, making the TP-10

"listen".

At E07B, we call the subroutine at E114 (SCR) to be sure the print head

is "homed". The LCD echos this action unnecessarily at this time; but in

subsequent calls of SCR we'll want that to happen.

At E07E, we load the file starting address in the HL pair; and at E081

we push it on the stack. We use the stack to remember where we are in the file

during the following SQ3 sequences, because the PUSH and POP instructions are

byte-saving. Remember that the starting address always lives in TOPFIL, and the

directory address is preserved in FILDIR. We'll use both addresses again,

should the user decide to kill the file.

E082: Pointing the BC pair to WBUF, where words will be assembled, we

get down to the business of formatting. You will note that everything sent to

the printer is echoed on the M-100 screen. This keeps the user from having to

squint at the TP-10 to see how things are coming along.

At E085 (SQ3), we set up a counter in the H register to make sure we

never overload WBUF. This would probably never happen in the course of

processing normal text files, but it is useful when printing hexdumps or ASCII

versions of BASIC programs, which usually do not contain many of the spaces and

hyphens THERML.CO recognizes as word delimiters.

At E087 (SQ3A), the current position in the file is popped off the

stack into the DE pair. The character is read into the accumulator (A) at E088,

the source file position is incremented at E089 and the new file position is

pushed on the stack at E08A.

From E08B-E0AB, we conduct a series of tests to handle the ASCII value

in the accumulator. Some characters are intercepted and sent on to "handler"

routines, while others are either rejected or accepted. In E08B-E08F, the

end-of-file character CHR$(26) is dispatched to its handler (EOFQ). In

E090-E094, ASCII values greater than CHR$(126) are rejected because the LP-10

wasn't designed to cope with most of them. E095-E099 routes tabs to their

handler; and E09A-E09E sends carriage returns to theirs. At E09F-E0A3, we send

hyphens to HYQ. E0A4-E0A8 takes care of spaces. At E0A9, ASCII values in A less

than 20h/32 which haven't already been dealt with are rejected, and we jump

back to SQ3A for another character.

However, arriving at E0AC, legitimate characters are added to WBUF

(STAX B). E0AD increments WBUF (INX B). E0AE decrements the counter in H, and

E0AF loops back for more (JMP SQ3A) if the H counter hasn't zeroed out. If it

has, though, under special conditions alluded to above, we drop through to CRQ,

one of our handlers.

HANDLERS - as mentioned, jumps to handler routines are executed

whenever ASCII values of 9 (TAB), 13 (CR), 32 (SPACE), 45 (HYPHEN) or 26 (EOF)

are found in the source file. Until one of those characters appears, we keep

cycling through SQ3, stuffing characters in WBUF. That may sound like an

extensive process, but remember that it's all happening in milliseconds! The

handlers labeled CRQ, HYQ, TABQ, SPCQ and EOFQ have some similarities. Rather

than putting each one under a microscope, abbreviated accounts follow.

CRQ deals with a carriage return. First it checks to see if line length

(LL) plus word length (WL) is =>31. If so, it sends a carriage return, then the

word and then the carriage return asked for. It then does LL and WL

housekeeping before looping back.

HYQ (E0C7-E0D2) - because hyphens often separate a longer than usual

word, we deal with them as potential "word-enders" to be sure we are optimizing

the printer's line length. HYQ calls the subroutine PREP, which puts the hyphen

in WBUF and drops through to CKL, which returns with the flags indicating the

result of comparing WL with 20h/32. How it does that will be discussed below.

If the comparison results in a carry (A<32), HYQ at E0CA jumps to the tail end

of the SPCQ routine labeled DONE. We'll track that later.

TABQ: The M-100 tab default is eight spaces. That is wasteful, given

the TP-10's short line length, so we arbitrarily shorten tabs to four spaces.

This routine puts three spaces in WBUF, and drops into SPCQ with CHR$(32) still

in A. On this occasion, SPCQ adds the fourth space and proceeds to handle the

tab as just another word.

SPCQ (E0DD-E0F4) - recognizes a space as the end of a word. It calls

the subroutine PREP, which places the space character in WBUF (without

incrementing WBUF for now), and drops through to CKL which returns to SPC1,

having compared 20h/32 with the current value of LL (representing the number of

characters which have already been sent to the printer without a carriage

return). Then it tests to see if there is still enough room (including the

final space) for the current unsent word to fit on the current line. Now it

gets a bit complex.

If LL equals 20h/32, the zero flag is set and at E0E0 (SPC1) we jump to

RMVS. This code zeros the accumulator (SUB A results in A-A=0), puts the null

in WBUF (STAX B) and recalls CKL. The comparison done there remains intact when

CKL returns and E0FA jumps back to SPC1. At E0E3, if there's room for the new

word without the space, we jump to DONE, where we would have gone anyway had

there been room for the final space on the earlier comparison, eliminating "JZ

RMVS".

If both comparisons have failed (i.e., the pending word with or without

the added space is too long for the existing line), we end up at E0E6. Now we

send a carriage return (CALL SCR). E0E9 (SPC2) sends the pending word to the

printer, adjusts WL and LL and goes back for a new word. Before getting into

the nitty-gritty of the subroutines, etc., lets complete our discussion of the

special handlers.

EOFQ (E130-E155) - has a relatively simple job. It measures WL and LL

and deals with pending word transmission much as the other handlers do; but

since CHR$(26) signals the end of the source file, EOFQ ends by jumping to the

"get ready to exit" stuff at E143 (RTX).

Now let's track the program through the end, assuming we've processed

the EOF character. We come to RTX (E143-E155), which loops five times, moving

the TP-10 paper up far enough so it can be torn off below the printout. This is

another user-friendly frill at the expense of a few more bytes of code. When

this is done, we drop through to...

ENDQ (E156-E17D) - we pop DE off the stack, where it was pushed the

last time we cycled through SQ3. Failing to do this would louse up subsequent

operations. Next, at E157, we de-activate the printer by calling R6ECB

(CLSCOM). At E15A-E15F we print "KILL "; and at E160-E165, reaching back into

NMBUF, which still holds the name of the file we've printed, we print the name

on the screen and follow it (E166-E168) with a question mark. It is worth

mentioning that this time we call a different routine (R27B1) to print NMBUF

because it does NOT conditionally start with a carriage return/line feed. We

want all the above to appear on the same line: "KILL (name).DO?"

At E169, we wait for the user to hit a key, checking the response at

E16C for CHR$(59h/89) ("Y") if the file is to be killed. Remember that R5D64

converts keypresses to upper case. Any other keystroke jumps to MENU to exit at

E16E.

If a "Y" is received, we drop through the conditional jump, and at

E171-E177 we fetch the starting address from TOPFIL, load it in HL, XCHG it so

the address resides in DE and then load HL with the directory address of the

file from FILDIR. The subsequent call at E178 to R1FBE (KILL) quickly puts the

source file on inactive status. Finally, at E17B we jump to the main MENU, our

job finished. The file has been printed on the TP-10 in legible form (no word

wraps), and the hard copy is ready to tear off. This assumes that we've

remembered to turn the printer on at the outset. If the power light isn't

glowing, not to worry. Turn it on and run the program again. No harm has been

done.

SUBROUTINES/AUXILIARIES - a brief look: The purpose of a subroutine or

auxiliary routine is to condense repetitive instruction sequences in a callable

section of code ending in a RET or appropriate jump in order to save bytes. I

planned some of them, and after early assemblies of THERML.CO, I added others.

Maybe you can find more ways to shorten the program - have at it!

DONE (E0FD-E10C) - this routine sends the pending word to the printer

by repeatedly calling R6E32 (THERM), gets the word length and stores it

temporarily in E. Then CNTSW, a subset of SWAP, is called, followed by SWAP

itself. When SWAP returns, we jump back for a new word at SQ3.

PREP (E10D) - simply loads the character in the accumulator into WBUF

and drops through to...

CKL (E10E-E113) - which calls GETCT, compares LL+WL with 20h/32 and

returns without acting on the comparison.

SCR (E114-E11D) - sends CHR$(0D/13) to the LCD and thermal printer.

After calling LDLY, it returns.

SNDW (E11E-E12F) - points the BC pair to WBUF, and loops internally

until it encounters a null in the buffer. When that happens, the instruction at

E124 (RZ) sends us back to the instruction following the call.

GETCT (E17E-E187): counts the characters in the WBUF. R21FA (COUNT)

does that when HL is pointed to a buffer, which must end in a null. The

resulting count returns in the E register. GETCT copies E into A, stores A in

WL and drops through to...

CNTSW (E188-E18C) - which loads LL in A, adds WL (still in E) and

returns.

SWAP (E18D-E19F) - loads LL in A, zeros A and puts the resulting null

in WL. Then, from E194-E19B SWAP points the HL pair to WBUF, loads B with

20h/32 and calls CLRBF in order to fill WBUF with nulls. Then SWAP returns to

the instruction following the call.

LDLY (E1A0-E1AC) - sets a counter in A. The value 20h/32 indicates that

DELAY will be called 32 times before a return. This routine provides an

appropriate pause while the TP-10 performs a carriage return. The section

E1A5-E1AC (LDYL) simply calls DELAY, decrements A and returns when A zeroes

out.

DELAY (E1AD-E1B9) - provides the short delay used between printable

characters sent to the thermal printer. This is absolutely necessary.

Mechanical peripherals move at a snail's pace compared with the lightning speed

of the microprocessor. I fiddled a good deal with the loop values trying to

make the TP-10 operate as fast as possible without being overrun by the

program.

TEXT STRINGS (E1BA-E1D1) - these are messages to be sent to the LCD as

appropriate. All but "XT" contain a quotation mark to signal the ROM routines

that print them that the end has arrived. XT doesn't need such a flag because

it is "moved" with the aid of a counter loop (review EXT at

E044).		Labels we haven't discussed are simply internal to certain routines and

subroutines.

Once THERML.CO has been loaded into RAM, you may want to set HIMEM to

protect THERML.CO with the direct command	CLEAR256,ORG where ORG=start of

the code for THERML.CO. If you assemble as I did at E000 to honor CDOS,

ORG=57334. If you don't use a Chipmunk drive, ORG could end up being a higher

address corresponding to where you assemble. I'm sorry, but I don't know where

other drives' DOS codes may occupy RAM. Interestingly, even without first

issuing that command, once THERML.CO runs from menu selection you'll find that

HIMEM will equal the strating address of THERML.CO!

Well, that's it. If there are others in the user group who have a

reason to interface the TP-10 and M-100, this has all been worthwhile. And, it

may have been entertaining to others who are perfecting their 8085 machine

language skills. I know I've had a ball writing it, debugging it, finding ways

to shorten it and finally watching it perform. The least fun was editing the

nearly five pages of source code file to include all the hex calculations for

transmission to the SIG. You see, the ZBGASM.CO assembler toggles F675 on the

user's input of a "switch" code (/LP) to print the source to the line printer.

Otherwise, the source is printed to the LCD.