POWR-DISK Owner's Manual

Name	     Format	   Comment

BLAH.BA	     BLAH  BA	   Space padded

D.CO	     D     CO	   Space padded

LFILES.DO    LFILESDO	   Not padded

[Note that periods are not included - ed.]

The best way to get the "address" is with VARPTR.  If you don't know how this

is done, adhere to the following example, where the file "GRUNT.CO" is prepared

for output:

Manual Page 16

--------------

10 B$ = "GRUNT CO"

20 B = VARPTR(B$)

30 CALL MAXRAM+3, 1, PEEK(B+1) + 256*PEEK(B+2)

CALL MAXRAM+6, len, address

This call allows you to read data from a TDD file, previously opened with

MAXRAM+3 with a "mode" of 3.  "Len" is the number of characters to read, which

must be in the range from 1 to 255; "address" is the destination memory

location for the characters.  [similar to a block move in memory - ed.]  Be

warned that if you use a STRING variable, it must be large enough to receive

the characters; this CALL won't change the variable length.  For example, to

read 25 characters into B$:

40 B$ = SPACE$(25) : '  Allocate space

50 B = VARPTR(B$)

60 X = PEEK(B+1) + 256*PEEK(B+2)

70 CALL MAXRAM+6, 25, X

CALL MAXRAM+9, len, address

This call allows you to write to a previously opened file.  You must precede

any writes with a CALL to MAXRAM+3, with a "mode" of 1 or 2.  "Len" can be up

to 255 (0 is illegal); "address" marks the beginning of the characters to

write.  For example, to write the contents of B$ to a file use the following

procedure:

100 B$ = "ABCDEFGHIJKLMNOPQRSTUVWXYZ"  [or whatever...]

110 B = VARPTR(B$) : X = PEEK(B+1) + 256*PEEK(B+2)

120 CALL MAXRAM+9, PEEK(B), X

CALL MAXRAM+12

This command sets the COM settings on the computer to communicate with the TDD

(98N1D).  It's unlikely that this call would be necessary unless you were

attempting to set up a telecommunications program that simultaneously uses the

TDD and the computer's internal modem.

POWR-DISK Owner's Manual

Manual Page 17

--------------

CALL MAXRAM+15, cmd

This call sends a single-code command to the TDD.  In the context of the

currently supported calls, the only sensible value for "cmd" is 2, which closes

a file.  Your programs should perform a "CALL MAXRAM+15, 2" after any file

input/output using the supported calls.  If you forget, files opened for output

will not be changed properly.

CALL MAXRAM+15,6  [command documented in the POWR-DISK Newsletter after the

original software release - ed.]

This call will format new diskettes, or reformat, and completely erase existing

data from a previously used diskette.  To use the command safely, first go into

BASIC and use the LFILES command to determine whether the diskette currently

has data on it, by showing the disk directory.  If you get an ?IO Error, the

disk is unformatted and you can proceed.  Or be sure to use a brand-new, unfor-

matted disk.  Type the following command:

CALL MAXRAM+15,6 <ENTER>

Immediately the red LED light will come on, and the drive will begin formatting

the diskette, erasing any pre-existing data.  When BASIC returns the "Ok"

prompt, the formatting is complete and the LFILES command will indicate 79

sectors free.

Error Codes

POWR-DISK returns error codes when something goes wrong with TDD interaction.

Below is a list of error numbers, along with a brief description of each. NOTE:

These errors will print out in BASIC unless you use the ERROR statement, which

will print out "?UE Error" for any error number greater than 58.

No. Code  Explanation

59   NR   RS-232 Not Ready.  This error code usually occurs if the TDD is

turned off or disconnected. (Yes, we know that the description is the

same as "No Resume".)

60   DN   Drive not responding.  This error happens most often when an earlier

operation ended unusually.  The best solution is usually to turn the

TDD off, then back on. [if another device is connected to the RS-232

port, a ?DN error in response to the LFILES command can be

interpreted as a "No Drive" error - ed.]

61   CM   Communications error.  Your computer and TDD use a system of checks

to make sure no errors in transmission occur.  If you get a CM error,

check the cable for snugness at both ends, and try again.

62   DA   Drive done early.  This error should never occur unless you attempt

to read past the end of a disk file.  It can happen when using the

CALL MAXRAM+6 function.

POWR-DISK Owner's Manual

No.  Code  Explanation

63   WP   Disk is Write Protected.  You are attempting to write to a disk with

the data protection tab open, protecting the disk data by preventing

writing to the disk.

Manual Page 18

--------------

64   DF   Disk Full.  You have exhausted the capacity of the TDD disk, either

by using more than 79 sectors, or by creating more than 40 files on

the disk. [also by attempting to write more than 64K to a file - ed.]

65   ND   No Disk.  There is no disk in the drive.  This differs from the DN

error above.

66   HT   "Hard Trouble".  This error covers any miscellaneous problems with

the TDD.  It rarely occurs except when using the CALL functions.

67   AE   File already exists.  You are attempting to overwrite an existing

file with the SAVEM or LOADM,F commands.

In addition to the above errors, the IO Error (#18) can signify a damaged or

unformatted disk, and the FF Error (#52) means that you are trying to access a

non-existent disk file.

Manual Page 19

--------------

Section Five

Using Other Machine Language Programs

POWR-DISK comprises about 2,000 bytes of machine language (m/l) code.  As

Appendix B discusses, the natural location for all m/l programs tends to be in

the same place, the area of memory just below the system default MAXRAM.  Con-

sequently, there are some considerations you must make when running other m/l

programs like FLOPPY.CO or TMPC.

Further, some BASIC programs and ROM programs (such as Interactive Solutions

for the Model 100) use the memory locations occupied by POWR-DISK as a "scratch

pad" when they are running; consequently, they corrupt POWR-DISK as it comes in

POWR-D.CO.  If you use such a product, you must also take precautions when

using POWR-DISK.  Fortunately, the TDD and POWR-DISK software can accommodate

just about any of these programs conveniently.

Checking for Possible Conflicts

In candor, it is unlikely that a m/l application will not conflict with POWR-

DISK.  In most cases, you will have to modify either POWR-DISK or the m/l

program so that they can run together.  However, it is worthwhile to check for

a conflict anyway, since the information you gain is useful for solving any

problems later.

For m/l programs, you should determine the TOP and END addresses.  There are

several ways to do this:

POWR-DISK Owner's Manual

1.  If the program comes on a cassette tape, use CLOADM to load the file into

BASIC. (NOTE: It's highly recommended to turn LFILES OFF before doing this.

See the discussion in Section One.)  The command will display the TOP, END,

and EXE values just before loading.

2.  If the file exists already on a disk or in RAM, use the [F6] key (Info) in

MENU.BA to display the values.  See the discussion of MENU.BA in Section

Two for more detail.

Manual Page 20

--------------

Note that neither of these techniques may work, for various reasons.  If you

are confused about how to determine the TOP and END values for a particular

application, consult a friend who knows some machine language.

If the END value is less than MAXRAM when POWR-DISK is active [default load of

POWR-D.CO is 58960 -ed.] there is some chance that there is simply no conflict.

However, do not try running the application with POWR-DISK on, until you have

saved all important files.  There is a distinct chance that something will go

awry.

Running Relocated Programs

Some m/l programs can be relocated.  When this is possible, the program docu-

mentation usually explains the procedure. If you want to relocate a m/l program

below POWR-DISK you must determine the offset for the program's relocation; you

should use the difference in MAXRAM with POWR-DISK on and off.

Example 1:  Determining MAXRAM values, starting with POWR-DISK loaded

PRINT MAXRAM <ENTER>

58960

Ok

LFILES OFF:PRINT MAXRAM <ENTER>

61104

Ok

CALL 58960 <ENTER>

Ok

PRINT 61104-58960

2144

Ok

The above example assumes that POWR-DISK is loaded right under the computer's

default MAXRAM at 61104.  If you have other m/l programs loaded, these values

may be different.  The CALL value should be identical to the MAXRAM address

when POWR-DISK is loaded and operational.  In the example above, the "offset"

is 2144.  When you relocate most m/l programs, you will want to lower their TOP

and END addresses by 2144.

Once you have relocated a program, you can access it directly from the disk

without ever loading a CO file into RAM.

POWR-DISK Owner's Manual

Manual Page 21

--------------

The RUNM command, with POWR-DISK active, will access the program directly.  The

most convenient way to use a relocated program is with a small BASIC load

module that clears the necessary memory, then runs the program.

Example 2:  Running a relocated Program

LAPWRD is a m/l print formatter available for the Model 100.  It is supplied to

the customer on cassette tape.  The TOP and END values can be obtained from the

tape with the CLOADM command in BASIC.  [Below is an example of relocating the

Model 100 version of LAPWRD; addresses would be different for a 200 version]

LFILES OFF:CLOADM"CAS:" <ENTER>

TOP: 59675

END: 62960

EXE: 60561

?OM Error

Ok

RUNM"POWR-D" <ENTER>

TOP: 60850

END: 62806

EXE: 60850

Ok

Notice that the above version of POWR-DISK has an offset of 2110 bytes (Model

100 default MAXRAM of 62960 - 60850).

Note that POWR-DISK was turned off before trying a CLOADM, and that POWR-D.CO

was present in RAM to restore POWR-DISK after the attempt.  From this exercise,

we learn that LAPWRD and POWR-DISK use conflicting memory locations.

However, LAPWRD is generously supplied with a relocator.  Without going into

detail, assume in the example above that LAPWRD is relocated with an offset of

2110 bytes (so TOP would be 57565; 59675 - 2110), then can be saved into a file

LAPWRD.CO on the disk.  The following BASIC program can then be written to run

LAPWRD from the disk:

10 CLEAR 0, 57565 : RUNM":LAPWRD" <ENTER>

Save this statement into a BASIC program which appears on the main menu; then

LAPWRD is available from the disk with the minimum RAM overhead and maximum

convenience.  [Running the program directly from the disk eliminates the image

copy which appears on the menu, thus saving the space allocated to that image

file. - ed.]

Manual Page 22

--------------

Relocating POWR-DISK

When another application cannot be relocated, it is necessary to relocate POWR-

DISK to accommodate the other application.  The BASIC program, RELOC.BA is

available to accomplish the relocation.  [See PDSK11.200 -ed.]