GPRINT.BA is a program that can be used for configuring the gemini 10X printer

The program has 4 modes of operation;

1 - Configure Printer - allows the setting of print font, line feed value

character width, type style, print type, right & left margins*, first

line printed*, perf skip, Uni-dir print, and print with paper out.

* These values should be left at default if using "Format Print"

2 - Envelope Mode - Allows addressing of buisness and letter size envelopes.

Line 400 allows you to set a default return address. Default addresses

can be over-ridden simply by typing in a new line. Comma's can be

used on addresses. BKSP is supported.

3 - Quick Print - Opens a .DO file and prints contents sequentially.

4 - Format Print - Opens a .DO file and prints contents. Formats output -

Right justifies (ragged left), sets right, left, top, and bottom margins.

>> Text should be entered in the .DO file. Carriage returns should only

be entered where desired (i.e. end of paragraph, to add blank lines,

etc.)

Variables for setting margins are set in line 2; L=left margin, R=right

margin, T=blank lines on top, B=page lenght.

Imbedding Printer Control Codes:

Format Print allows imbedded printer control codes in text. The Codes are:

&0& Form feed

&1& Select italic character set

&2& Select normal Character set

&3& Select underlined print

&4& Cancel underlined print

&5& Select double-wide print

&6& Cancel double-wide print

&7& Select super-script

&8& Select sub-script

&9& Cancel super/sub-script

To use simply enter code in .DO file. The actual code characters will not be

printed. The gemini printer executes control codes on recieving a carriage

return, therefore it is not possible to select & cancel a code in the same line.