80C85 ASSEMBLER DOCUMENTATION

-----------------------------

This program is offered under the shareware concept.  Please feel free to

try it out and pass it on to your friends.  If you like it, the author hopes

that you will support it by paying a $10 registration fee.  Send the fee to:

Paul Papanek Stork

Don't Pa..Panic Software

E19431 Church Rd.

Aniwa, WI  54408

THANK YOU IN ADVANCE FOR YOUR HONESTY!!

There are two main steps to be taken in using this Assembler for the TRS-

80 Model 100/200.  The FIRST is the creation of a source file using assembly

language and the built in TEXT program in the Model 100.  The SECOND is

running the actual assembler program.  Be aware that this program runs like

Molasses, but it is a lot better than Hand Assembly and a lot cheaper than a

Commercial Assembler.  Now on to STEP #1.

STEP #1

-------

The first step is to use TEXT to write a source file to be assembled.

The source file will use three fields or columns of entry.  Columns 0-7 will

be the Label Field, Columns 8-15 will be the Mnemonic Field, and Columns 16-23

will be the Operand or Address Field.

LABEL FIELD

Any character except a space or TAB may be used in a 7 letter label.

Examples of valid labels are: 'START', 'start', 'AD<LF>', and 'test#1a'.

Examples of invalid labels are: 'test 1' & 'AD LF'.

MNEMONIC FIELD

This assembler uses standard 8085 MNEMONICS and a few Psuedo-Ops.  The

Psuedo-ops supported are:

1. "DB"(DEFINE BYTE)

2. "DW"(DEFINE 16 BIT WORD)

3. "DS"(DEFINE STORAGE)

4. "EQU"(SET LABEL EQUAL TO VALUE)

5. "ORG"(ORIGIN OF PROGRAM)

6. "END"(END ASSEMBLY)

For more information on these Psuedo-Ops and 8085 MNEMONICS see "8080A-8085

Assembly Language Programming" by Lance A. Leventhal.

OPERAND or ADDRESS FIELD

The Operand or Address Field may contain a number of different things.  It may

contain a register identifier like: PSW, A, B, or H; or it may contain a

LABEL; or it may contain data; or some combination of the above.  Data may be

input in a number of forms:

1. Hexadecimal - 1AH,0FFH,etc.

2. Decimal     - 10D,12,etc.

3. Octal       - 08Q,07C,etc.

4. Binary      - 10110011B,etc.

5. String      - 'This is ASCII'

6. Offset      - $+2,$-1,etc.

7. Label       - START,AD<LF>,etc.

Any Column may be used to start a Comment by printing a ";" to start the

Comment.

To move between fields simply hit the [TAB] key.  Once your source code

is written move on to STEP #2 (HINT: If space is a problem you can assemble a

.DO file from TAPE, but be sure to save it TWICE, once for each pass).

STEP #2

-------

Run the Basic Assembler.  On the Title Page you will be asked for the

starting address of the Program to be assembled.  This is so that the program

can set aside memory space for your program to be assembled into.  It will

lower "HIMEM" to the address you specify.  Next the program will display the

files in memory and ask for the name of the file to be assembled.  Enter the

Name which you gave to the source code you wrote with TEXT. (Use of the ".DO"

suffix is unnecessary.)  Or the name of a file to be read in from Cassette

with the prefix "CAS:". (Caution:  File must be read in on each pass.)

Now sit back and wait while the Assembler works.  It takes two passes

through your source code for the Assembly to be complete.  Meanwhile the

display will show which pass you are on and the memory address which is

currently being processed.  The current memory address for Pass#1 will be

displayed in the upper left hand corner, while Pass#2's will be displayed in

the upper right.

When the assembly is complete the computer will beep and display the

pertinent information about the assembled program.  It will also save a ONE

BYTE key file.  This key file is not a full copy of the assembled file.  It is

simply a file which has the same entry point as your assembled file.  By

moving the cursor over this file and pressing return you can run your

assembled file, while not maintaining two complete copies of your machine

program in memory.  To erase your program simply enter BASIC and type in

"CLEAR 256, MAXRAM" and press "ENTER".  To save a full copy simply type in

'SAVEM filename,start address ,end address ,execution address' and HIT

[ENTER].

If you have any problems please feel free to leave a message on the Wausau

Online of Wisconsin Bulletin Board at (715) 842-4850, COMPUSERVE to ID#

75515,1651 or Contact me direct at:

PAUL PAPANEK STORK

(715) 446-3050 voice only