GRAFIX.ASC is a utility program written for the NEC PC8201A

portable computer.  Its purpose when active, is to intercept

outputs to the printer and check for characters which have been

defined as graphics characters using the CHRDEF utility.  It

prints these out as defined.  The program uses the commands

for 480 dots/ line required by my EPSON RX-80 printer, namely

"ESC K n1 n2" (HEX 1B 4B n1 n2).  where:

n1 = (number of data) mod 256;

n2 = int(number of data/256).

If your printer uses a different sequence, the routine can be

modified easily.  If you have downloadable fonts you probab-

ly don't need this utility.

In operation the utility resides at hex F000, so before accessing

it, a CLEAR,61439! must be executed from BASIC.  The loader

does this for you initially, but it will have to be entered after

any cold starts or running of programs that have CLEAR statements

higher that 61,439.  If you hear a beep when attempting to enter the

program from the menu, do the CLEAR statement.

To get the program running, download it and the loader program.

The loader will convert the ASCII into machine code at F000,

and BSAVE it to a .CO file in RAM.

When you run the program, you will see the main menu.  Your

options are:

1. Stop intercepting and exit

2. Begin intercepting and exit

3. Exit with no action taken

With the intercept active, graphics characters(>HEX 82 = decimal

130) cause the intercept routine to check for a graphics font

address at HEX FEBF.  If the address is zero nothing happens.

Otherwise the characters are saved in an decimal 80 character

buffer located above the program.  A non-graphics character will

cause the graphics buffer to be printed out, and then will itself

be printed.  Graphics characters beyond 80 characters will be

ignored, but continuous graphics can be achieved by printing a

carriage return-line feed pair between graphics lines(LPRINT after

80 chrs).

You can easily change graphics fonts between lines by BLOADing

previously defined character sets.

CAUTION!!!  If you don't exercise option 1, and load another

program to the area above 61,439, then try to LPRINT; the new

routine will be entered with unpredictable results.

You can excercise option 1 from BASIC by entering:

POKE 63960,219:POKE 63961,127

Have fun!

-Ray