RAM2PC.DOC  by Kent Nickerson  71061,01650  August 13/90

This file distills most of what I've learned about file transfer between

an IBM-type PC and the TANDY 200.  There are two main sections:

1) Transfer of single files to and from a PC.

2) Single-step transfer of all .DO and .BA files in a RAM bank.

======================================================

TRANFERRING SINGLE FILES TO/FROM A PC (requires no program):

(Note: Only 7 bit ASCII text files may be transferred)

To a PC:

1) Connect the TANDY RS232 port and IBM serial port (say, COM1:) with a

NULL MODEM cable.  Construction of this cable is described below.

2) Set the PC COM port by typing "mode com1: 1200,e,7,1" (for COM1:).

3) On the PC, type "copy com1: file", where 'file' is any valid PC file name.

4) To transfer a .DO file, go into TELCOM, press 'Stat' and type "57e1enn"

to set correct communications paramenters.  Press 'Term', then 'Up'.

You will be prompted for a (.DO) file name, and a width.  The width value

entered, which may be up to 132, determines how long a transmitted line may

be before a return is automatically inserted.  This is useful for sending

text files to be viewed or printed (a good choice of width for 80 column

screen viewing is 78), but not desireable for sending BASIC files which

will not work if a line is broken.  Leaving the width unspecified

by entering nothing will leave carraige returns in the file as is,

but the file transfer will be unsuccesful if there are any lines longer than

255 characters in the file.  When the transfer is finished (the 'Up' key

label returns to normal video), press CTRL-Z (press CTRL, hold it down, and

press 'Z') and ENTER.  The PC should return with a screen prompt.

5) To transfer .BA files (without having to convert to .DO format), go into

BASIC, load the file, and type:

save"com:57e1enn"

FROM a PC:

Perform steps 1) and 2) above.

3) If transferring to a TANDY .DO file, go to TELCOM, set the communication

parameters using 'Stat' and press 'Term', as described in step 4) above.

Press 'Down', and answer the TANDY's prompt with a name you want to give

to the transferred file.

4) If transferring an ASCII BASIC file directly to TANDY BASIC, go into

BASIC on the TANDY and type:

load"com:57e1enn"

5) On the PC, type "copy file com1:", where 'file' is the DOS file to be sent.

Note: A communication rate of 1200 baud is described.  Rates of 2400, 4800, and

9600 baud (among others) may be supported by modifying the PC "mode" command

of step 2) appropriately and changing the TANDY's TELCOM parameters

respectively to  67e1enn, 77e1enn, and 87e1enn.  I've chosen a rate of 1200

because the TANDY has no error checking protocol, and it is the highest rate

at which I've never experienced an error.

Possible Problems:

The files received by the PC may contain "garbage" characters at

the beginning and the end.  These may be edited out.  If the lines

of the file overwrite each other (e.g. only one line is seen), or

new lines start in the column following the end of the last line,

perform step 6) below, and try again.

In the case of transferring files to the PC, if the PC does not

return with a screen prompt after file transfer,

and will not respond to the PC keyboard, it may be waiting for an

end-of-file character (e.g. CTRL-Z), or may not be connected properly

to the TANDY.  If nothing is received in transfers to the TANDY, an

improper connection is the most likely cause.

Improper Connection:

- check that the TANDY's and the PC's communication parameters match.

- make sure the correct PC connector is being used.  If the serial

connector has logical name COM2:, modify the PC commands given above

to reflect this (e.g. copy com2: file).

- make sure a proper NULL MODEM cable is being used (not a normal

RS232 cable!), or a normal cable with a NULL MODEM adapter.

- make sure the connectors are seated properly.

End-of-File Character not received:

If a .DO file was transferred, make sure you have

sent a CTRL-Z as described in step 4).  If you've exited TELCOM by

this time, you may always return to do this.  If this fails, perform

step 6) below, reboot the PC if necessary, and try again.

If a .BA file was transferred, go into TELCOM (the proper

'Stat' should already have been set by the transfer) and type CTRL-Z.

If this fails, try 6) below.

6) Go into BASIC and type "poke 61243,1" (model 200) or "poke 63066,1"

(models 100, 102).  The original value for this location is 0.

WARNING: This is not tested for models 100/102.

Test this procedure first on a RAM bank with files which can be damaged.

Null Modem Cable:

These cables may be purchased or made at home.  The TANDY requires

a DB25 male connector, while the PC will need a DB25 female or DB9 female

(check your PC serial port to see if it has 25 pins or 9).  The

pins will be numbered (in very tiny figures).  The pins named below will

be for a DB25 (25-pin) connector.  Numbers following in brackets will

be for DB9 (9 pin) connectors.  There are 7 conductors between the connectors.

For neatness, one should use a ribbon cable or jacketed cable with 7 or more

wires.

- for each connector, short pins 4 (7) and 5 (8) together.

- connect pin 7 of the TANDY connector to pin 7 (5) of the PC connector.

- as above, connect pin 2 to PC pin 3 (2), 3 to 2(3), 5 to 8(1), 8 to 5(8),

6 to 20(4), and 20 to 6(6).

================================================

TRANSFER OF ENTIRE RAM BANK TO PC:

BK.BA will allow one to transfer all .DO and .BA files in a TANDY 200 RAM

bank to a PC.  This program is a modified form of BACKUP.200 by James Yi

(73327,1653), which saves all .BA and .DO files to cassette tape.  The

program is minimally documented internally to reduce its size.  Use of this

program requires subsequent use of program UNPACK.BAS (included here).

Checksums for BK (in .DO format) and UNPACK, obtained by DOWNCK.WM3,

are 77,076 and 233,309, respectively.

WARNING: This program not tested on models 100 or 102.  Test on a RAM bank

which contains only files which can be lost.  If the program fails on such

models, it may be because a variable assigned in line 45 is wrong.

USE:

BK.BA transfers a single composite text file consisting of a

concatenation of all TANDY files to a PC through the RS232 port

to an IBM type PC.  The individual TANDY files may be restored by running the

GWBASIC program UNPACK.BAS (provided in this document below) on the PC, or

left as is as a single RAM bank backup file.

1)   Connect the TANDY RS232 port to a PC serial RS232 port with a

NULL MODEM cable.

2)   Set the PC serial port's communication parameters to match line 50

of BK.BA (e.g. 1200 baud, 7 bits, 1 stop bit, even parity).  This

is done on the PC for COM1: by typing: "mode com1: 1200,e,7,1".

3)   Type "copy com1: filename" on the PC (assuming you are using com1:),

where "filename" is any DOS file name of your choice.

4)   Run BK on the TANDY.  When finished, the PC should have written a

file to disk.

5)   Run UNPACK in GWBASIC on the PC, specifying the received PC file at

the prompt to restore the TANDY files.

BK will not transfer files with the extension ".CO".  MSPLAN ".CO" files

must first be changed to SYLK format in MSPLAN, and transferred as such.

BK.BA must not be renamed without changing the name at the end of line 180, and

this name cannot exceed two characters, since the TANDY keyboard buffer

will accomodate no more characters than defined in line 180.

DESCRIPTION of PROGRAM:

45 - Location 1 identifies a Model 100 or 200 (PEEK(1)=171 for a 200).

File directory, keyboard buffer, line feed enable locations assigned.

50 - The line feed patch is enabled.  Normally, the TANDY does not issue LF

characters on file transfers.  This patch is useful for most computers,

but some PCs' (HP150) will terminate receiving at the first carraige return

if it is invoked. In such a case, change the

first statement of this line to POKE lf,0.  The RS-232 port is set as an

output device at 1200 baud.  Edit this segment if you want

comm parameters changed.

60 - Location Q serves to count the number of files transferred, and to

supply an index to byte RD, which contains a file type code (128=BASIC

file, 192=text file).  Since there is a maximum of 48 files, any greater

number implies this program has not been run, and the file number is reset.

Q is not an internal because all such variables are lost when the program

calls itself in line 180-210.

70 - Head of program loop.

75 - Code = 255 indicates no more files.  CTRL-Z sent to terminate PC receive.

80 - Provision for empty entry.

90 - Q is file name pointer.

100 - This program handles transfer of text and BASIC files only.  Some

PC's will stop receiving data from this program at the end of transmission of a

BASIC file.  If this is the case, change line 140 to:

140 IFS=128THEN70:' BASIC file        (e.g. jump to 70 instead of 180)

(Lines 180 to 210 inclusive can be removed in this case, but will do no harm otherwise).

If transfer of BASIC files are desired in this event, they must first be saved in ASCII

format before invoking this program.

110 - Prevents transmission of the program file itself (save in .DO format if

file copy desired).

120 - Escape (chr$(27)) sequences cause file name display in reverse video.

130 - Print file separation header to the PC file.  If sequence changed,

modify program UNPACK as well.

180-210: Load keyboard buffer with instructions to load .BA file, and

save in ASCII form to RS232 port, and invoke BK again.  Marker set in 60,70

gives restart point.

PROGRAM BK.BA - Checksum (using program DOWNCK.WM3) = 77,076

10 ' Saves all .BA and .DO type files in the current ram bank to RS232.

20 ' If program stopped, running again will save next file, but POKE Q,0

30 ' will cause it to start over.  WARNING: Run program from .BA format.

40 MAXFILES=2:CLEAR600

45 IFPEEK(1)=171THENSL=62122:KB=64798:LF=61243 ELSESL=63919:KB=65450:LF=63066

50 POKE LF,1:OPEN "COM:57e1enn" FOR OUTPUT AS 2

60 Q=SL+3:F=PEEK(Q):IFF>48THENF=0

70 F=F+1:POKEQ,F:RD=F*11+SL:S=PEEK(RD)

75 IFS=255THENPOKE Q,0: PRINT #2,CHR$(26):PRINT"Done.":BEEP:END

80 IFS=0THEN70

90 F$="":FORM=RD+3TORD+10:F$=F$+CHR$(PEEK(M)):NEXT:FP$=LEFT$(F$,6):F$=FP$+"."+RIGHT$(F$,2)

100 IFS<>192ANDS<>128THENPRINT"File is binary or not recognized -- ";S;F$:GOTO70

110 IFF$="BK    .BA"THEN 70

120 PRINT"Saving:";CHR$(27)+"p"+F$+CHR$(27)+"q"

130 PRINT #2," ":PRINT #2,"!@#$!@#$":PRINT #2,F$:PRINT #2," "

140 IFS=128THEN180:' BASIC file

150 OPEN F$ FOR INPUT AS 1

160 A$=INPUT$(1,1):PRINT #2,A$;

170 IF NOT EOF(1) THEN 160 ELSE CLOSE 1:GOTO 70

180 Q$=CHR$(34):K$=CHR$(127)+"Loadf$"+CHR$(13)+"Save"+Q$+"com:57e1enn"+CHR$(13)+"Run"+Q$+"BK"+CHR$(13)

190 ' Activate Keyboard Buffer

200 FORL=1TOLEN(K$):POKEKB+L*2-1,ASC(MID$(K$,L,1)):NEXT

210 POKE KB,L-1

220 END

The following program, UNPACK.BAS, is used with PC GWBASIC to parse the

composite file transfered by BK.BA into component files on the PC.  Its

checksum (using DOWNCK.WM3) is 233,309.

10 REM ********************  PROGRAM "UNPACK"  *****************************

20 REM

30 REM This program unpacks a composite text file composed of a number of

40 REM  files delimited by a three line header.  The first header line is

50 REM  the string:

60 REM             !@#$!@#$      (e.g. a distinct marker)

70 REM  followed by the file name and a blank line.  These headers are

80 REM  stripped and the separate files reconstituted.

90 REM

100	DIM BUFFER$(1001)  	'A buffer is used to reduce disk file writes.

110 REM

120	ON ERROR GOTO 800

130     INPUT "Enter name of composite file >",C$

140     OPEN C$ FOR INPUT AS #1

150	EOFFL=0: ERRFL=0

160 REM

170 REM If a drive is specified (second character = ":"), set the drive

180 REM  as a filename prefix.  Unpacked files will go to this drive as well.

190 REM

200             PREFIX$=""

210     IF MID$(C$,2,1)=":" THEN PREFIX$=LEFT$(C$,2)

220 REM

230 REM Go through the file line by line until the first file header is found.

240 REM The first header usually is right at the file beginning, but "junk"

250 REM characters may precede it - hence the initial search using INSTR.

260 REM

270		LINE INPUT #1, A$

280	IF INSTR(A$,"!@#$!@#$")<>0 THEN 560

290	IF NOT EOF(1) THEN 270

300		PRINT "End of file reached - No file bounds found":END

310 REM

320 REM Read lines of the composite file until:

330 REM  1) 1000 lines are read

340 REM  2) the "!@#$!@#$" delimiter line is found, or

350 REM  3) end of file is reached.

360 REM In case 2), the counter is reduced by 1 to omit the delimiter line

370 REM

380	FOR I=1 TO 1000

390             LINE INPUT #1, BUFFER$(I)

400        IF NOT EOF(1) GOTO 430

410		EOFFL=1			'End of file flag

420		GOTO 540

430        IF LEFT$(BUFFER$(I),8)="!@#$!@#$" THEN I=I-1: GOTO 540

440	NEXT I

450 REM

460 REM If 1000 lines read, write them to the output file, and resume reading

470 REM

480        FOR J=1 TO 1000: PRINT #2, BUFFER$(J): NEXT J

490     GOTO 380

500 REM

510 REM Case of delimiter encoutered.  Variable FIRST prevents "close #2"

520 REM  statement on first pass, since no output file is open then.

530 REM

540     FOR J=1 TO I: PRINT #2, BUFFER$(J): NEXT J	'Write final data

550		CLOSE #2				'Close file

560	IF EOFFL<>0 GOTO 770		'End program if input file exhausted

570             LINE INPUT #1,F$	'Read file name

580             F$ = PREFIX$+F$		'Add directory

590		OPEN F$ FOR INPUT AS #2	'Attempt to open file

600 REM

610          IF ERRFL<>0 THEN 720	'File does not exist if no error

620             BEEP			'Case of file already existing

630		PRINT "File ";F$;" exists.  Overwrite? (Y,y=yes) >";

640		INPUT ANS$

650	     IF ANS$="y" OR ANS$="Y" THEN 720

660		INPUT "Enter alternative file name (No entry will skip file) >",F$

670 REM

680 REM A null file name will initiate search for the next file header.

690 REM Otherwise, an existence check is performed on the new file name.

700 REM

710	     IF F$="" THEN 150 ELSE GOTO 580

720             ERRFL=0:CLOSE #2

730		OPEN F$ FOR OUTPUT AS #2	'Open new output file

740             PRINT "Restoring file ";F$

750             LINE INPUT #1, A$		'Ignore blank header line

760     GOTO 380

770     CLOSE

780     END

790 REM  Error Handler.  Primarily used to detect an error condition in 520.

800	ERRFL=ERR:RESUME NEXT	'Set error flag in case of error (ERR <> 0)