FIG-FORTH RELEASE 1.1

THIS PUBLICATION HAS BEEN MADE AVAILABLE BY THE

FORTH INTEREST GROUP

PO BOX 1105

SAN CARLOS, CA  94070

IMPLEMENTATION BY JOHN CASSADY

MODIFIED BY KIM HARRIS

THIS VERSION FOR THE RADIO SHACK MODEL 100 BY:

DAVID O ROWELL

3961 CHERRY VLY TPK

MARIETTA, NY 13110

The file FIG4TH.100 is the basic nucleus of a figFORTH

system.  It will run on a 32k Mod 100 as is.  (NEC ?? It

should at this point.)  Since the system is only allowed

(this configuration) to use 16k it probably can be made

to work on a 24k system too.  The origin is set at A100H

(41216 decimal) for two reasons: first to give some room

below it for TEXT or BASIC to work with and secondly to

conform to the figFORTH listing for CP/M which ORG's at

100H.  This way I didn't have to make, and debug, a fat

listing of the figFORTH source.  I've allowed 16K above

that for figFORTH, LIMIT= E000H (57344), to use and I

use another 3k+ above that for an in memory storage of 3

screens.  This gives enough room for DISKMANAGER to be

resident at all times. (Don't know what I'd do without

that program!!)

++

OK, to get started download FIG4TH.100 as a TEXT file,

clean it of any download garbage and verify the checksum

of 741,760.  SAVE the TEXT file to tape (sorry 'bout

that!).  Now CLEAR 256,41216 and run CHANGE.BA to load

the TEXT file into memory as a binary file.  At this

point you have figFORTH installed in memory and can run

it.  But first why not SAVE it so you don't need to do

that again?  Clean out all the junk below 41216 and

SAVEM"FIG4TH",41216,47753,41216, you'll just have room.

Now SAVE it to disk or tape as a .CO file.

I've uploaded this version half finished but useable

since I don't have the time right now to finish up the

disk interface.  As it stands the TDD can be used even

tho' its awkward.  If some kind soul wants to finish it

I can upload a lot more information to help them, just

send me a note on EASYPLEX.  As it stands its a very

nice figFORTH and is highly useful in learning.  There

is some potiential as a control language even with this

version.  THERE IS NO BETTER LANGUAGE TO LEARN ABOUT THE

M100'S GUTS THAN FORTH - the ULTIMATE hacker's

language.

This figFORTH has two entry points, 41216 for a COLD

start and 41219 for a WARM restart.  ->figFORTH IS CASE

SENSITIVE SO PRESS THE SHIFT LOCK AND LEAVE IT THERE!<-

Cold start it (CALL 41216) and you will see the initial

greeting "8085 FIG FORTH 1.1".  Type "-8 OFFSET !" and

ENTER to set up the in memory "SCREENS" properly.  Type

"HEX E000 C00 ERASE (ENTER)".  How's that for speed in

0'ing 3K of memory?  Type "EMPTY-BUFFERS (ENTER)" to

clean up fig's internal text buffers and you're all set

to learn FORTH!

Screens can be stored on disk and brought back and forth

(heh heh) at will, just remember that in memory screen 1

is located between 57344 (E000) and 58367 (E3FF), screen

2 between 58368 (E400) and 59391 (E7FF) and screen 3

between 59392 (E800) and 60416 (EBFF).  Just SAVEM any

screens you want to save and push 'em off to disk.  To

reload 'em put the MENU cursor over the name and press

return, restart figFORTH and the screen is available.

You can use CMOVE to transfer directly to and from a

text file if you are carefull and understand what you're

up to!

How do you learn FORTH?  We're very fortunate in having

a number of excellent books to help.  Without

qualification I'd recommend "STARTING FORTH" by Leo

Brodie, Forth Inc., published by Prentice-Hall.  The

second edition is to be preferred but the first will do

just fine.  When I get time I'll upload my "Brodie to

figFORTH translator" which will help with the first

edition.  Join FIG, the address is up front.  They have

a great "FORTH HANDY REFERENCE" card for fig.  But buy

Brodie - you'll never regret it!

OK, what's different about this implementation of fig?

I've implemented the in memory R/W from the installation

manual in low level forth.  Its buried down with the

rest of the I/O section.  To keep with the memory

adresses of the source listing I've left a big chunk of

0's to fill to TICK.  For readability COUT is 7 bit

only.  I've written a word CALL for use with the ROM

routines.  Stack effect is ( HL DE BC PSW ADR -- HL DE

BC PSW ).  The fig IP is buffered on the way in and

restored coming out.  It works fine!  ^P toggles the

printer on and off.  The screen print routine works.

Next weekend I'll upload the EDITOR that is used in

STARTING FORTH and some utilities to get you started.

ENJOY - David O. Rowell