MORSE.BA -- Morse code trainer for the M100/M102

=================================================

MORSE.BA is a Morse code teaching program designed to

provide practice copy for anyone learning Morse to obtain an

amateur radio operator's license.

The program sends random 5-character strings of code, using

the speaker of the M100, and displays the characters sent on

the screen. Optionally, the characters sent may be captured

in a file for later comparison.

The program allows for full adjustability of the speed and

spacing of the code sent, as well as the pitch of the audio

tone. It supports all letters and numbers, and the

punctuation and prosigns required by the FCC license test.

The program is operated entirely by using the function keys,

as described below:

ACTION KEYS:

============

Send (F4)

---------

This key causes practice code to be sent. The normal mode of

the program is to send 5 random characters, and then stop

and display them on the screen, but a continuous send mode

may be toggled using the Cont option described below.

WPM (F2)

--------

This key allows you to set the words per minute sending

speed to any value from 5 through 30 wpm. WPM is calculated

based on the number of times the word "PARIS" can be sent in

a minute. This convention is described in detail in the ARRL

Handbook.

Tone (F3)

---------

This feature allows you to adjust the audio frequency of the

speaker to your liking. It allows you to send a code test

while making the adjustment, to check the way the code will

sound.

File (F1)

---------

This key toggles the file capture feature of MORSE on and

off.

If no file is currently active, you are prompted for the

name of a document file to capture the copy in. You do not

need enter the '.DO' file extension. If the file you specify

already exists, it will be written over.

If a file is active when this key is pressed, the capture

feature is stopped, and the file is closed.

Test (F7)

---------

This key sends the word "PARIS", using the current settings

of WPM, Tone, and Spce.

Exit (F8)

---------

Ends the program, and returns you to the system menu. If any

capture file is open, it is closed before leaving the

program.

OPTION KEYS:

============

Cont (F5)

---------

This option causes 5-character code groups to be generated

and sent continuously when the option is on. The key remains

active after the Send key (F4) is pressed, and pressing Cont

during continuous sending cancels the option and stops the

sending.

(Note: When the timer is sounding the speaker, the interrupt

generated by pressing the Cont function key is masked, so it

may take a few tries to break into continuous sending. This

is more of a problem at slow code speeds that at higher

ones. Try to press the Cont key at the end of a 5 character

group.)

The normal status of this option is off, that is each press

of the Send key sends 5 characters and halts.

Spce (F6)

---------

The Spce (space) option adjusts the overall formation and

spacing of the Morse code characters. When this option is

off, (the default), the spacing of the dots, dashes, and

character and word pauses follows the conventions set down

for international Morse code. This spacing is sometimes

referred to as "Paris" convention spacing.

With this option on, at WPM settings less than 13 wpm, the

characters are formed and sent at 13wpm, but the spacing

between characters and words is what it would be at the

selected WPM.

This is considered by some to be an aid to learning Morse,

by accustoming your ear to the sound of the characters at a

higher speed, but giving you the time you need at first to

recognize each character sent. This method of sending is

sometimes called "Farnsworth" spacing.

At WPM of 13 or faster, this option has no effect.

Notes on the use of Function Keys

=================================

At any given moment, on those function keys that are

appropriate and may be used appear on the bottom line of the

screen. Pressing a function key when it's label is blank

results in no action.

When an option is on, or a function is active, the key label

appears in reverse video to remind you what the program is

currently doing.

Some final notes:

=================

The information used to create this program was obtained

from the 1987 ARRL Handbook, available from:

American Radio Relay League

225 Main Street

Newington, CT 06111, USA

(These folks are the premier source for all information

relating to getting a ham radio license).

If any problems should present themselves while using

MORSE.BA, please contact me via the M100SIG.

Hope you enjoy the program...

-- Mike Aiello

71066,155