Documentation for MONTOR.100

----------------------------

MONTOR.100 is a Basic utility for inspecting and patching Model

100/102 menu files.  It is adapted from Joel Hassell's LOOKIT.100

in DL4.  But where LOOKIT asks for a start address in Hex,

MONTOR asks for a file name and determines the appropriate start

address (using code borrowed from Tony Anderson's RAMDIR.100,

DL4).  Note that MONTOR.100 is not for the Tandy 200 -- although

it could probably be adapted for the 200 by changes in the

directory search routine.

With MONTOR you can look at the contents of any file on the menu

(visible or invisible), starting at the first byte in the file.

The cursor keys, alone and shifted, are used to move you through

the file, slowly or rapidly -- and can be used to move you beyond

the file.  But for general exploration, LOOKIT is a better tool,

since it accepts specific addresses, even if in ROM.

File names must be complete with '.'  and a two-character

extension, but can be upper or lower case.  If you have a file

with no extension (e.g., 0MENU), end it with '.' followed by two

spaces; the program checks for the '.' and for two following

characters.

With MONTOR you can patch programs on the menu directly.  Typing

in hex values will change the value at the cursor position.  This

can be very handy if you know what you are doing -- but

disasterous otherwise!  Experiment first with a DO file.

At any time while "in a file", pressing ESCape takes you back to

the beginning of the program, ready to examine another file.

There ENTER will take you back to the main menu.

Phil Wheeler [71266,125]