Appendix A

Your Portable Disk Drive

Entire books could be written on the subject of disk drives,

but this short appendix describes only a few matters of

prudence and simple curiosity.

The 3-1/2" Diskette

Technology has advanced considerably in diskettes for

personal computers.  The diskettes used in you r PDD have

more potential storage capacity than 5-1/4" Diskettes (which

are still the standard for desktop personal computers), yet

require less than half the area.  A 3-1/2" diskette holds

about 100 kbytes of data when used in your PDD; other disk

drives can squeeze as much as 1.44 megabytes onto the same

diskettes.

Further, the 3-1/2" diskette is a step forward in ruggedness

and reliability.  No part of the magnetic surface is exposed,

as a spring-loaded metal flap conceals the "window" in the

firm plastic case.  Be warned, however, that these diskettes

are not hermetically sealed: avoid dust and dirt; do not

leave diskettes scattered on a desktop under papers, etc.;

and do not carry them in pockets of clothing that sheds

(sports jackets, etc.)  The other constant enemy of diskette

data is magnetic fields.  The wordless warnings on diskette

boxes show horseshoe magnets, but remember that speakers,

motors, and office paper-clip holders all contain magnets.

You needn't be paranoid, but don't be careless either.

The Disk-Drive Hardware

The PDD comprises two basic moving parts: the motor and the

head.  The motor keeps the diskette spinning at a constant

rate.  The head glides over the diskette to read and write

the data.  The head can be in one of 40 different positions

relative to the edge of the diskette; each of these 40

positions is called a "track".  If you examine the magnetic

surface of a used diskette, you may be able to distinguish

lines of the tracks from friction with the head.

Page 68

Appendix A                                          Page 69

Diskettes must be "formatted" before the drive can

successfully read or write data.  We recommend formatting an

entire box of diskettes as soon as the are purchased.

Formatting uses up some of the diskette's  storage ability

for magnetic timing marks, so that the drive controller

hardware can determine where on the track the head is at any

point.  After formatting, there are 2,560 bytes of capacity

left on each track of a diskette.  Further, it divides each

track into two "sectors" of 1,280 bytes each.  (Other drives,

with more capacity, squeeze more bytes per track).

The PDD Operating System

It may surprise you that your PDD has software built into it

- and contains both RAM and ROM hardware - in order to

control the disk hardware.  The PDD is known as an

"intelligent" disk drive, since it decides on its own which

sectors are available and which are full, whether a file with

a certain name exists on the disk, etc.  Thus, the POWR-DOS

software merely sends codes conforming to those rules to

allow the computer and the drive to communicate.

There are advantages and disadvantages to this arrangement.

The principle advantage is that code in the computer requires

less room.  The principle disadvantage is that the disk drive

is more or less limited by its internal operating system -

What it's designers intended for its use.  In this case,

this limitation means that the files on your PDD must be

primarily sequential rather than random access.  While it is

possible to do random access, it is far less convenient than

sequential access.

There are several other interesting observations about the

PDD operating system:

* The PDD O/S uses a "directory" to keep track of files on

a diskette.  Every diskette has a directory, which makes

up a sector on the outside track.  Thus, an empty

diskette holds 79 more sectors (of 1,280 bytes for a

total of 101,120 bytes)

* The directory cannot hold more than 40 names, limiting

the diskette to 40 files maximum, regardless of size.

* Each file uses at least one sector, no matter how small

the file is.

Appendix A                                          Page 70

* The PDD does not create "empty" files, i.e., files of

zero length.  If you open a file for output and fail to

write to it, it will not appear in the directory after

closing it.

* The PDD can only deal with one file at a time, whether

for input or for output.

* PDD file names can be longer than six characters and an

extension.  The actual limit appears to be something

more like 24 characters, which can also include lower

case.  However, POWR-DOS limits names to six characters

for compatibility with your computer's files, and to

reduce the amount of space required for the POWR-DOS

software.