Copyright 1989 Mike Audleman (73577,3041)

Distribution restricted to Model 100 Forum members

SPEAKER.DVI   Apr 26, 1989  Mike Audleman  73577,3041

What is SPEAKER.DVI?

SPEAKER.DVI is a utility  to  aid  an  individual  in

designing the optimum speaker enclosure.  The user needs

to have the specification sheet commonly  provided  with

the speaker or the specs are  sometimes  listed  in  the

manufacture's catalog.   SPEAKER.DVI  will  calculate  a

varieity of boxes with  different  frequency  charastics

and  the  user  must  choose   one   that   fits   their

application.  SPEAKER.DVI then will step the  height  or

depth through a range entered by the user and  show  the

combinations which will produce the best enclosure.  The

user  must  have  some  knowledge  in  building  speaker

cabinets.   SPEAKER.DVI  will   not   give   "carpentry"

information, just the INSIDE dimentions  that  the  user

can use to construct the box.

What SPEAKER.DVI must know.

The information that it must have is usually  provided

with most quality  speakers.   The  speaker's  free  air

resonance (Fo), equivalent air volume (Vas), Diameter of

the  speaker  (D),  and  the  total  Q  (Qts)  are   the

information  points  usually  provided   and   are   all

SPEAKER.DVI need to perform its calculations.

What will SPEAKER.DVI tell me?

SPEAKER.DVI  will,   after   entering   the   required

information, calculate  the  optimum  width,  depth  and

height for an array of combinations  varying  in  volume

and frequency responce.  It will also,  if  selected  by

the  user,  send  all  results  to  the  printer  for  a

perminant record.  The prelimenary information  provided

is the Q of the box (Qb), the internal volume of the box

(Vb), the Q ratio (Qr) between the box Q and the speaker

Q, volume ratio (Vr), resonant frequency of the  speaker

mounted in the box  (Fsb),  the  3dB  low  end  roll-off

frequency (F3dB), and the width, depth, and  the  height

based on the optimum w-d-h ratio.   The  user  needs  to

select a combination that fits their  application  based

on  the  roll-off  frequency,  resonant  frequency   and

possibly  size  limitations  that  may  be  involved  in

placing the cabinet in a  crowded  room  or  on  a  book

shelf.  Using SPEAKER.DVI you can sit  with  a  list  of

specs  out  of  a  catalog  and  run  through  literally

hundreds of calculations that would take hours to do  on

a calculator.  SPEAKER.DVI also plots a graph on the LCD

screen showing the ratio of the box volume (Vb) and  the

3 dB  roll-off  frequency  so   you   have   a   graphic

representation of the information  provided  in  numeric

form on the CRT or printer.

If the printer option is selected,  SPEAKER.DVI  will

try to call XCOPY to dump the LCD to a Radio  Shack  DMP

series printer.  If you don't have this program or don't

have a DMP printer, you will have to edit the line  that

loadms and calls XCOPY to call a LCD screen dump program

for  your  printer  or   simply   remove   the   feature

alltogather.  XCOPY.CO is created by DMP.DMP  and  loads

into the alternate LCD buffer so it  doesn't  require  a

HIMEM setting or protected  memory  so  will  work  with

almost anything in himem.

After  you  choose  a  combination  that   fits   your

application, SPEAKER.DVI will ask wheather you  wish  to

step through Width & Depth  (and  calculate  Height)  or

Width & Height (and  calculate  Depth).   You  are  then

prompted for the ranges for  both  Width  and  Depth  or

Height depending on which one you  select.   SPEAKER.DVI

will then step through the combinations  and  point  out

the ones that will  fit  the  optimum  ratio.   All  the

dimentions that are presented in this  table  will  work

and their internal volumes are all the  same,  only  the

width, depth and height dimentions will change.  It will

calculate the ratios and display them also.

Other options of SPEAKER.DVI are to  calculate  values

for a Ported reflex or an Acoustic suspention enclosure.

It will also calculate the width, depth and height based

on the optimum ratio for a volume entered by  the  user.

Also, SPEAKER.DVI will calculate the internal volume  of

an enclosure with dimentions provided by the user.

What else will you need besides SPEAKER.DVI?

To complete the Ported Bass Reflex enclosure, you will

have to use the tables in the Radio Shack book "Building

Speaker Systems", catalog # 62-1087  as  of  this  date,

listed at $5.95, well worth it if you are  serious  abot

making your own  speaker  cabinets.   If  you  have  any

carpenter in you, the book probably will not  be  needed

for the acoustic suspention box  but  before  purchasing

any components, material or  wood,  I  highly  reccomend

that you obtain a copy of the book and read through  the

construction information.

How does SPEAKER.DVI work?

The formulas that are in the  Radio  Shack  book  have

implemented in SPEAKER.DVI using  the  loops  and  other

conditional statements provided  by  BASIC.   You  could

design your cabinet  without  SPEAKER.DVI,  but  to  run

through various combinations on a hand calculator  would

be  very  cumbersome  and  time  consuming.    Also,   I

developed a formula to compute the optimum  W-D-H  ratio

of a given volume.  This formula is  not  in  the  book.

The ratio provided in the book is 1:.6:1.6.  This  means

that the depth should be .6  times  the  width  and  the

height should be 1.6  times  the  width.   Deriving  the

formula:

V=Volume,H=Height,D=Depth,W=Width

V  = W x D x H

V  = W x .6W x 1.6W

V  = W x .6W x 1.6W

---   --------------

W^3   W  x  W  x  W

V  = 1 x .6 x 1.6

---

W^3

V  = .96 W^3

V  = W^3

---

.96

______

3 /  V

\  /  ---    =  W

\/   .96

D=.6W

H=1.6H

And is Basic it is:

W=(V*.96)^(1/3):D=.6*W:H=1.6*W

Since most computers don't have a cube  root,   simply

rase the figure to the 1/3rd power.

You are welcome to make modifications to the  program.

If you do, please make them  obvoius  and  if  possible,

upload a copy with  a  changed  version  number  to  the

Compuserve Model 100 SIG, library 4  (Applications).   I

do request that you leave my  name  and  number  in  the

program and menus but you may add yours if you like.

If you have any recomendations or questions,  you  may

leave E-mail to me on CIS, 73577,3041.