ADSM.DOC by James Yi 73327,1653 July 27, 1988

ADSM.200 and ADSM.100 are downloaded as hexadecimal coded text files.  They

should be converted back to a binary file by using HXFER.200(in DL10) or

HXFER.100(in DL8).

HXFER also lets you relocate the program as it is being converted.  To relocate

it, enter the relocation address at the "Load at?(<CR=default>" prompt in

HXFER, else just press ENTER to load it at a preset address.  If you relocate

it, make sure that the new address is lower than the preset address, else the

system variables above MAXRAM will be written over when ADSM.CO is LOADM'ed!

The preset loading address is 56601 for M200, 58603 for M100.

Once it's converted to ADSM.CO, HIMEM has to be cleared from Basic with the

CLEAR,256,xxxxx statement.

The value for xxxxx is the same as the preset loading addresses, but if you

relocated it, the value will of course be different.

ADSM.CO can now be RUNM'ed from Basic or executed from Menu.

As an alternative to the above HIMEM loading procedure, the following is the

LOMEM loading procedure:(M200 users only.)

Instead of running it HIMEM, it is possible to locate ADSM into LOMEM, area

below the file storage space, and access it through ADDRSS, SCHEDL, or MSPLAN

directory slot.  The result is a saving of 4.5 K bytes, since the .CO file is

not needed, whereas if you run it in HIMEM, the .CO file is needed in order to

reload it every time it runs, since other HIMEM machine language programs may

have been run in the same location just before and erased ADSM.  But .CO files

cannot be LOADM'ed below the HIMEM area, so there's no worry of writing over a

program in LOMEM by a loading a .CO file.

To install it in LOMEM, you need ADMINS.200 in DL10.  It also is a hex file and

needs converting to a .CO file with HXFER.  To install it, just run ADMINS.CO.

When installing, ADSM.CO file must also be present.  It will not space-conflict

with other LOMEM programs you may have there already.  But take proper caution

if you have .BA programs that have ML codes poked into the REM statesments,

since all files are moved up when the space is created in LOMEM, or moved down

when it's removed.  The ML codes in the REM statements have to be relocated

every time LOMEM shrinks and expands.

After it's installed, both files can be deleted, and ADSM accessed from Menu.

To remove it, run ADMINS.CO again.

;--------------------------------------

* Monitor:

When you run ADSM, there's a hexadecimal number at the bottom left corner of

the screen, and a flashing cursor beside it.  This number is the current

address and entering data(opcodes, DB, etc.) at this address will store it

there. Initially it's 0F856H(M200) or 0FCC0H(M100), TELCOM's PREVious screen

buffer, used by monitor as the default workspace.  This area will be written

over when you use TELCOM, but otherwise it's a safe place for temporary data.

To change the current address, just enter the address that you want to go to.

It has to be a number starting with 0-9, so if you want to go to F800H, enter

0F800H.  If you enter a label here, the label is defined as the value of the

current address.  So if you are at 8989H and typed ABCD, ABCD becomes a label

defining the number 8989H.

NOTE: From this point, whenever a number is referred to, it can also be a label

or an ascii chr(in single quotes(')), or a combination of these.

For example, the following are all equivalent of LXI H,$43A0:

1)	LXI	H,43A0H

2)	LXI	H,$43*256+$A0

3)	LXI	H,A0H+(43H*256)

4)	LXI	H,'C'*256+$A0

5)	LBLB	EQU	'C'

LXI	H,0A0H+(LBLB*$100)

6)	LXI	H,67*256+10100000B

To step forward or backward certain number of bytes, enter a number preceded by

a positive or negative sign.

To see the disassembly of the current address, press <ENTER>.  The content will

be displayed and the address will step forward to the next instruction.

To go back to the previous address, press <SHIFT> with <ENTER>.  Upto 16

locations can be retraced.  Pressing <CTRL> will trace forward, instead of

backward.

To display the next few addresses, enter commas.  One comma fills one screen.

To stop listing, press <CTRL>, or to pause, press <SHIFT>.

You can read and write to text files from the monitor.  Select the file name by

entering a '=' followed by the file name.  A device name can be specified with

it.  The default name is NOTES.  To read the file, enter a '<'.  To write to

it, enter a '>' followed by a string.  The string will be appended to the file.

Press <SHIFT> to pause reading, or <CTRL> to abort.

Commands mentioned thus far are sign chrs entered as the first chr in a command

line.  They are -,+,,,<,>,=

Rest of the commands consist of one or more alphabets.

The command input in monitor is much like a line in a source code.  There are

LABEL, OPCODE, OPERAND,and COMMENT fields, each seperated by a tab, space, or a

comma.  The command input can be commands, assembler directives or opcodes,

which assemble immediately at the current address.  Commands are entered in

OPCODE field.

The monitor is initially in hexadecimal mode, i.e., all outputs are in base 16.

It's changed with:

MD Byte

Don't forget that commands are entered in opcode field, so a TAB or SPACE

should be inserted before MD, and Byte goes in operand field, so another TAB or

SPACE should be in between MD and Byte.

Byte is any base from 2 to 255.

Any base other than 2,8,10, and 16 will have '#' sign suffix.

You can only ENTER numbers in base 2,8,10, and 16.

Use letter 'B' suffix to indicate base 2,

'Q' or 'O' for base 8,

'D' or none for base 10,

and 'H' for base 16.

A000H, 0A000H, $A000, $A000H

are all equivalent ways of entering a hexadecimal number.  Because hex numbers

can begin with a letter, some restrictions are set on labels.  You cannot have

a label that looks exactly like an ordinary hex number.  For example, you

cannot have a label named A000H, but you can have A000 as a label.  Also, avoid

using the following as labels: B, D, Q, O, H.

Disassembly of an address consists of a mnemonic and numbers or chrs in the

comment field.  The mnemonic can be suppressed with:

CD Byte

If Byte is 0, mnemonics are displayed, but if Byte is 1-32, each line will

contain only comments, For example, CD 11 causes the display to be 11 numbers

or chrs as comments.

What appears in the comment field can be controlled with:

DD Byte

Byte=0 for none,

1 for ascii chrs only,

2 for numbers only,

3 for both ascii and numbers.

RST instructions are normally one byte instructions, but Model 100/200, uses an

extra byte for RST 1 and RST 7.

RFIX Byte,Byte

controls how many bytes a particular RST instruction represents.  The first

Byte chooses which RST(0-7) and the second Byte is the length( 1 or 2).

E.g., to make RST 7 disassemble in two bytes, enter:

RFIX 7,2

As a default, RST 1 and 7 are set to 2 bytes.

If you specify a value after RST, like

:

RST 7,20H

RST is assembled in two bytes, the single byte RST instruction, and 20H after

it.

Commands MD, DD, CD, and RFIX control the disassembly outputs of both the

monitor and the disassembler.

The symbol table contains the currently defined labels.  It is preserved when

you exit the program, if you reenter it without editing any text files,

deleting, or creating new files, or doing other tasks that change the amount of

free memory.

To list the table, enter:

SMB String

and specify an output file to dump the listing.  To view it on screen, enter

LCD: as the file name, LPT: to print it out.  Press <SHIFT> to pause listing,

and <CTRL> to abort.  A beep sounds when finished.

String is optional, used to selectively list only those that begin with String,

e.g., to list only those beginning with "LI", enter:

SMB LI

To delete a label from the symbol table, use:

DEL Label

Or:

DEL *

to clear the table.

To calculate a formula, use:

GET Expression

Expression can be a mixture of numbers of different bases, labels, single ascii

characters(closed in single quotes(')).

Boolean(!(xor), .(and), "(or)) and algebraic(-,+,*,/) operations are allowed.

Only one parenthesis can be opened, e.g., ((8+9)*2) cannot be done, but (34+2)*

(2+3) is ok.  To make things simple, all operations have same priority and

evaluated from left to right, so in order to get a correct answer to 2+9*9,

which is 83, you would have to enter:

GET 2+(9*9)

Without the parenthesis, the result would be 99.

To convert a number to a different base, use:

CNV Number,Byte

Byte is the base.  If only Number is entered, it is converted to an ascii chr.

To search for a sequence of numbers or strings, use:

FIND List

The search begins at the current address.

For example, to look for the string 'No file' followed by a chr$(0), enter:

FIND 'No file',0

To find 195,20301, enter:

FIND 195,20301

Press <SHIFT> to pause the scrolling, or press <CTRL> to abort.

To move to another address, a number beginning with 0-9 can be entered in the

label field, as explained before, but to use a label, enter:

SET Label

To move a block of memory:

MOVE Beg,End,Destiny

To PEEK:

RE Addr

The displayed result is:

Byte in Addr, Byte in Addr+1, and the 16 bit word formed by the two bytes.

To POKE one or two locations at once:

WR Addr,Number

If Number is >$FF, the high byte is poked into Addr+1

To examine an IO port:

IP Port

To write to a port:

OP Port,Byte

To execute a machine language routine:

GO Addr,Parameter

Only one parameter can be specified and it's loaded into BC,DE,HL, and PSW(LSB

in A, MSB in flag byte).  eg, if you enter

GO 32,$FFFE

HL,BC,DE are loaded with $FFFE, and A register is loaded with $FE, flag byte

with $FF, then 32 is called.

To exit the program and return to the menu, enter:

MENU

;--------------------------------------

* Disassembler:

It's called from the monitor by entering:

DSM

Then enter the .CO file to disassemble, or to disassemble an area of ROM or

RAM, enter the begin and end addresses, seperated by a comma.

Then specify the disassembly output file.  You can specify an output device as

well, such as LPT:, LCD:, RAM:, CAS:, R:, 0:, etc.  To disassemble to 200's

option banks, use R1:, R2:, or R3: as device names.

The disassembler will use the labels stored in the symbol table.  A number not

in the table will be shown as Lxxxx or Ixxxx.  'L' stands for a label and

indicates that reference is made to that address by one or more instructions

within the program. 'I' stands for "interval", which is just put there in

intervals as a reference for the user.

Any instruction disassembled in DB means that the reference was made to the

instruction, but to the middle of it, its operand.

A beep will sound when finished.

Press <CTRL> to abort, or <SHIFT> to pause.

;--------------------------------------

* Assembler:

It's called by entering:

ASM

Then enter the name of source code, followed by the name of .CO file to be

created.

A source file in another bank(200 option banks) can be specified by R1:, R2:,

or R3: as the device name.

The assembler must read the source file twice.  After the first pass, it

displays the Top, End, Exe values.  If the program's starting location is in

HIMEM and its end steps above MAXRAM, lower the Top value, so that when it

runs, it will not write over the system variable area above MAXRAM.

If "0" is entered for the object file name, the object file is not created, and

the it stops after the first pass.  This is when you only want to see the

symbols or the Top, End, Exe values.

If error occurrs or user-aborted during the 2nd pass, object (.CO) file is

trashed and should not be run.

Pressing <CTRL> aborts.

ENT directive specifies the Exe addr of the .CO file.  If ENT is not used, Exe

is same as ORG.  If ORG is not specified, it is taken as $F856 for M200, $FCC0

for M100, which are addr of the Prev screen buffer.

SET(also a monitor command) is used to setup a variable space outside the .CO

file.  It must be used at the very beginning of the source code, before

specifying ORG.

As an example, the following sets up 65370 to 65373 as program variables.

SET	65370

var1	dw	0

var2	db	1

var3	db	0

ORG	60000

lxi	h,$0000

shld	var1

mvi	a,1

sta	var2

... etc.

IF and ENDIF are used for conditional assembly.

Example:

COND	equ	2

IF	cond=1

call	beep

ENDIF

IF	cond=2

call	whistle

ENDIF

Since COND=2, only 'call whistle' instruction is assembled.

IF statesments can be nested:

if	C1=1

call	SUB1

if	C2=2

call	SUB2

endif

endif

The following results according to C1 and C2:

C1=1,C2=2:

call	sub1

call	sub2

C1=1,C2<>2:

call	sub1

C1<>1,C2=2:

nothing assembled

C1<>1,C2<>2:

nothing assembled

Source code can be LINKed, so that one can write something much bigger than

what 200's memory can hold.

LINK Filename

to connect two or more pieces of a file together, e.g., if you were to cut a

large file into 3 manageable pieces, and save each to a disk, cassette, option

bank, or whatever,

at the end of piece 1, put:

LINK Piece2

At the end of piece 2, put:

LINK Piece3

No other changes are neccesary.

If you assemble Piece1, Piece 1,2,3 will assemble as if they were a whole file.

In addition to the standard 8085 opcodes, there are 10 undocumented opcodes.

See OPCODE.DOC in DL8 for detailed info on these.  They are:

DSUB B or HLMBC: Subtract BC from HL

RHR: Shift HL right, with bit 7 of H remaining unchanged.  Bit 0 of L goes into

the carry bit.

RDL: Shift DE left.  Carry bit shifts into bit 0 of E, and bit 7 of D goes into

the carry bit.

LHLX or LHLI: Load DE from addr pointed by HL

SHLX or SHLI: Save DE to addr pointed by HL

DEHL or DMOV D,HL+: Load DE with HL+byte

DESP or DMOV D,SP+: Load DE with SP+byte

JND: Jump if bit 5 of flag byte is off

JD:  Jump if bit 5 of flag byte is on

___: Call 40H if overflow flag is on.

List of ADSM directives:

SET	Setup external variables

ORG	Set Top,Exe

ENT	Set Exe

EQU	Define labels

DB	Enter Data

DW	Enter a two byte data, a word

DM	Enter a string

DS 	Reserve space

LINK	Connect files

IF	Assemble following if true

ENDIF	Until endif

END	End of source code

;--------------------------------------

* Possible differences from other assemblers:

The meaning of SET, ENT maybe different

A comment begins with ';' symbol and it can begin at any field, not just the

comment field.

More than one data item can be specified with the DB directive.  Seperate each

item with a comma, tab, or space.

If an item is a number >255, the MSB will also be stored.  For example,

LABELX	EQU	$AA55

DB	'HI' 0,$EEDD,LABELX

is same as

DB	'H','I',0,$DD,$EE,$55,$AA

Hexadecimal numbers can start with a letter.  For example, FAB0H, 0FAB0H, and

$FAB0 are all valid.  As a trade off, labels that resemble a hexadecimal

numbers cannot exist.  For example, if you forget and do this:

DH	EQU	$1098

CALL	DH

The call is made to $000D, instead of $1098 that was meant to be.

;--------------------------------------

* List of monitor commands:

MD: Output base

CD: Suppress mnemonics

DD: Display ascii or number in comment

RFIX: Change length of RST instructions

GET: Evaluate a numeric expression

CNV: Display a different base,or ascii

FIND: Search for string

GO: Execute a ML routine

SET: Go to a different address

MOVE: Move a memory block

RE: Peek

WR: Poke

IP: Read IO port

OP: Write to IO port

MENU: Exit

ASM: Assemble

DSM: Disassemble

LINK: Link source files

While in SMB, ASM, DSM, press <CTRL-C> or <ENTER> for the file name prompts, to

abort.

;--------------------------------------

* Error messages:

When an error occurrs, a beep sounds and the error code is displayed.  Most

error codes are same as the Basic's error codes, so if you run across an error

that is not listed here, consult the Basic manual.

1: ENDIF encountered without IF statement preceding.

2: Syntax error - unrecognizable opcode , command, or register

6: In MOVE Beg,End,Dest, End maybe lower than Beg

7: Out of memory

8: Undefined label

9: Repeatedly defined label

10:Bad label

12: The instruction can only be used in monitor.

18: IO Error

22: Missing operand

52: File not found

55: Bad file name

57: Ram directory is full; cannot create any more files

;--------------------------------------

<End of DOC>