.Screen dump to a dot matrix printer.  Configurable.

.Copyright Polar Engineering and Consulting 1985,1986 [72136,1443]

.

.LCDPLT.101 checksum:  71,623 (screen dump to a dot matrix printer)

.

.LCDPLT.101 creates LCDPLT.CO.  This machine code program can be called to

produce a screen dump onto most dot matrix printers.  The example below shows

the setup for an OKIdata 92/93/192/193 printer.  This routine was originally

developed for the dumping histograms to a dot matrix printer.  See HISTO.DOC.

.

.10 LOADM"LCDPLT"

.20 'SETUP FOR OKIDATA 92/93/192/193

.30 PL!=PEEK(64704)+PEEK(64705)*256'lcdplt's low address

.40 PL$(0)=CHR$(27)+"*`P:"+CHR$(27)+"%P"+CHR$(28)+CHR$(3)

.                          'printer graphics begin

.50 PL$(1)=CHR$(3)+CHR$(14)'move to next printer line (graphics mode)

.60 PL$(2)=CHR$(3)+CHR$(2)'printer graphics end

.70 PL$(3)=CHR$(3)+CHR$(3)+CHR$(3)'3 must be duplicated

.80 PL$(4)=""'no other exceptions

.90 PL%=1'quadruple pixels when printing (use 0 for single)

.

To dump the screen to the printer during a BASIC program use:

.

.CALLPL!,PL%,VARPTR(PL$(0))

.

.

.The setup for other printers would be established as follows:

.- PL$(0) contains the character sequence which places the printer in a

graphics mode ready to accept 8 bit vertical graphics elements.  If PL% is 1

then each screen pixel is a two by two square on the printer.  If PL% is 0 then

each screen pixel is a single printer dot.

.- PL$(1) contains the character sequence which sends the printer to the next

line.  The printer should move to the left margin and down eight printer dots.

.- PL$(2) contains the character sequence which takes the printer out of

graphics mode and back into normal text mode.

.- PL$(3) this is the first exception character's replacement string.  One or

more graphics characters (8 bit vertical elements) will have to be treated

specially.  The first character in this string is the exception character.  The

remainder is the string is sent to printer instead of the exception character.

In the case of the OKIdata the char 3 decimal must be sent as two 3's to the

printer.  The pair of 3's tell the printer to print a single 3 and not exit

graphics mode.  (OKIdata's PL$(3) is CHR$(3)+CHR$(3)+CHR$(3).)

.- PL$(4..) addition exceptions if needed.

.- PL$(last) must contain a null string.  This string terminates the exception

list.

.

.Anyone wishing to share setup values for printers they use is welcome to

submit the information to me.  I will maintain a setup cross-reference here if

enough interest is generated.

.

.Tom Bennett, Polar Engineering and Consulting