Weather Maps to Go

by Alexander B. Trevor

5 September 1983

Weather Maps to Go               Page 2

Sometimes the applications  for  a  new

device  are  so obvious that you hardly

have to point them out.   For  example,

set  a  Model-100  in front of a pilot,

and it won't take long for him (or her)

to  realize  that  this little portable

powerhouse  could  be  very  handy  for

pre-flight    and   in-flight   chores.

Although there have been other machines

that   one  could  tote  along  in  the

cockpit, the 100 is the first one  with

a  screen  large enough to be useful in

flight.   Not  only   can   the   pilot

retrieve   flight   plans  and  textual

weather  information  obtained   before

takeoff from an information service (or

entered  manually),  but  the  100   is

capable  of  pretty decent graphics - a

capability that can be used to  display

weather radar maps.

Graphical   weather    maps    are

available from a number of sources, but

one of  the  least  expensive  ways  to

obtain  maps  that are easily displayed

on the Model 100  is  to  download  the

Vidtex (tm) Radar Maps from CompuServe.

To do this, one must have a  CompuServe

Information  Service  account,  and  at

least 16K of  memory.   The  downloaded

maps  can  consume  up to 4K.  The maps

are   transmitted   in   "run    length

encoding",  which  reduces transmission

time as well as minimizing  the  amount

of   RAM   required   to   store  them.

However, without a  program  to  decode

the  run length encoding, the maps make

about as much sense as the listing of a

.CO file.

Two different graphic  resolutions

are  used:  "high resolution", which is

256  pixels  wide  by  192  high,   and

"medium   resolution"   (128   x   96).

Listing 1 is a program that  will  plot

either  high  or medium resolution "run

length encoded"  graphics  on  the  LCD

screen.   Since the Model 100 screen is

only 240 x 64 pixels, high res maps are

displayed in three sections, and medium

res in two.

Weather Maps to Go               Page 3

Although  anyone   interested   in

weather  should find these maps useful,

they  are  of  particular  interest  to

aircraft  pilots, especially since maps

can be displayed at any time (including

in  flight).   They  provide  a  useful

reminder  of  general  weather  trends,

although  they  are  not  practical for

detailed  storm  avoidance  like   true

airborne  radar,  since  the data is at

least  a  half  hour   old,   and   the

resolution  is  too coarse.  Each pixel

represents an area 20 x 20 miles.

The run-length encoding spans  all

characters  from  30  (decimal) through

511, so eight  bit  reception  must  be

enabled  on  your  Model  100.  This is

done by setting the  Telcom  status  to

M8N1E.   However,  CompuServe  normally

sends 7 bit data with even  parity,  so

you  will  see  some strange garbage if

you log  in  with  this  setting.   The

solution  is  to  log  on  with default

status (M7I1E) and change your terminal

parameters  on  CompuServe  to  set the

parity bit to  zero.   This  is  easily

accomplished by logging on, then typing

"GO CIS-9" at the "!" prompt or running

DEFALT   from  the  programmers'  area.

From the DEFALT menu, select 5 (View or

Change  Current  Terminal  Parameters).

Next, select 8 (Parity) and set  it  to

zero.  Finally, exit the DEFALT program

and request that changes  be  effective

for future sessions.  Before logging in

the next  time,  change  your  terminal

parameters  to  M8N1E  in  Telcom.  Now

when you log on you will see  a  little

bit  of  strange stuff until you get to

the Password:  prompt.  This is because

until  you  identify yourself, the host

computer does not  know  what  terminal

parameters you desire.  So, the initial

"User ID:" prompt  is  sent  with  even

parity,  and  those characters with the

parity bit set to one (s,I  and  space)

will  appear  as graphics characters on

the 100.

Weather Maps to Go               Page 4

At  this  point  eveything  should

appear  normal,  and  you can go to the

Aviation Weather section,  or  directly

to  the  weather  maps  (GO AWX4).  The

program will correctly inform you  that

Telcom   is  not  equipped  to  display

graphics,  and  they  may   appear   as

meaningless  characters,  but  you  can

ignore this.  When you see:

CompuServe Vidtex Radar Maps

choose 1 - Prepare a map  for  display.

Next,  select  2 - Prepare current U.S.

radar reports.  The  system  will  list

the three letter id of each radar site,

and the GMT time of  the  report  being

collected.    After   all  reports  are

collected, choose 2 - Display  prepared

map.   You  may select any of the maps,

but before you press ENTER  after  your

choice,  you must first press F2 (Down)

and specify the name you  wish  to  use

for   the   downloaded  file  (MAP1  or

whatever).  THEN, press ENTER, and  you

will be treated to several screen fulls

of  utter  garbage,  but  don't  panic!

Just  wait  until you hear a BEEP, then

press  F2  (Down)  to   terminate   the

download,  and  ENTER  to tell the host

computer to procede.  You can  now  log

off   or  continue  with  your  on-line

session.

Once off-line, disconnect  and  go

back  to  the  menu.  From here you can

run the GRAPHR.BA program.  Give it the

file  name you specified to Telcom, and

watch the current weather  map  appear!

An outline of the state boundaries will

be   drawn    with    radar    "echoes"

superimposed   on   top.    "Weak"  and

"moderate"   echos   will   appear   as

horizonal   shading,   while   stronger

echoes appear solid  black.   When  the

screen  is  full,  press any key to see

the next section.

A design "feature"  of  the  Model

100  will very occasionally cause a map

to  be  partially  trashed.   For  some

reason,  the Model 100 will not store a

delete character (&H7F) in a  RAM  file

Weather Maps to Go               Page 5

without  some  sleight of hand, and one

does not get an opportunity to do  much

magic  in  TELCOM.   The problem can be

avoided by writing  a  special  purpose

communications  program  which replaces

the  delete  character  with  something

else.

One   final   remark   for   those

contemplating  the  use of this program

in aircraft:  pilots need not be overly

concerned  about  operating a Model 100

in  flight.    Although   the   Federal

Aviation Regulations (FAR's) saddle the

pilot    in    command     with     the

responsibility  of  determining whether

or not an electronic  device  (such  as

the   Model  100)  will  cause  harmful

interference with  your  navigation  or

communications  radios,  I  have  found

nothing to suggest a problem.  My Model

100 has many hours of instrument flight

time in my Cherokee Challenger,  and  I

have  never noticed any ill effects.  I

hear the same thing from  other  pilots

who  regularly fly 172's, King Airs and

such with their little white boxes.

For your convenience, this program

may  be  downloaded  from the Model 100

SIG XA1 Data Base on  CompuServe.   The

file name is:

GRAPHR.100[70000,130]

OK