Documentation for CHDUMP.BA

(Excerpted from Portable 100 magazine, May 1990)

Copyright 1990 Tri-Mike Network East -- All rights reserved.

IMPORTANT FIRST STEP!

Line 10 should be all one line, with just a single CRLF at the end.

In downloading, it may be broken up. Be sure to remove all extraneous CRLF's

except for the one at the end of line 10. All other lines should be okay as

delivered.

ABOUT THE PROGRAM

CHDUMP.BA is derived from George Flanders' screen dump program,

DMP100.102 elsewhere the data libraries (and published in the May 1990 issue of

Portable 100, as was this program). It simply prints a copy of the Model

100/102 screen to an IBM-compatible printer.

This program works much like George's original, using the same

keystrokes to start and to disable the screen dump. The major difference is

that, while his program is a .CO file that operates high RAM, mine is machine

language packed inside a .BA file, which resides in low RAM, thus minimizing

conflicts with other .CO files, like print formatters, disk operating systems

(e.g., FLOPPY), screen enhancement software (e.g., Ultrascreen, View-80), and

what-have-you.

To use CHDUMP.BA, type it in and save it as CHDUMP.BA (it expects that

name). To save memory, you may leave out unnecessary spaces and delete remarks,

but you must use all existing line numbers, and remark lines must contain at

least the line number and the apostrophe. If in doubt, just type it in exactly

as shown. This creates the BASIC installer program. Save it to tape or disk, so

you'll never have to type it in again.

Before running the installer, first kill any .BA files that you may

later want to remove. Some BASIC programs don't show a .BA extension (for

example, UR-2, RANDOM, T-WORD+). When in doubt about a file, kill it. You can

reload it later.

CAUTION: Mistakes made in typing CHDUMP.BA can have potentially

disastrous results. The first time you run it, back up any important files

first!

Now you can run CHDUMP.BA. When run, it installs the machine code into

line 10, and then deletes any unneeded lines, shrinking itself from 3K to 277

bytes. CHDUMP.BA is now ready to use. (If you get a Load Error message, see the

LOAD ERROR section below.)

To arm the program (i.e., to make it ready to do a screen dump) just

run it from the main menu like any other .BA program. The Model 100/102 main

menu quickly returns, and CHDUMP.BA is missing. It's been made invisible so

that it can't be killed. Killing it while it's armed can cause a cold start. To

indicate that it's armed, BASIC on the main menu is now named BASIC+, and any

time you press the ESC/GRPH key combination, it will initiate a screen dump to

your printer. If the printer is not connected and ready, the computer will beep

to inform you.

To disable CHDUMP, press ESC/CODE. When you return to the main menu,

BASIC has lost the + symbol, and CHDUMP.BA is back on the menu. It is now

disarmed and can be safely killed. However, being so small, it is usually

easier to leave it on the menu, making it convenient and handy whenever needed.

CAUTIONS AND CLARIFICATIONS

First, never edit the installed CHDUMP. The embedded machine code in

line 10 may run afoul of the computer's editor, and it may refuse to restore

itself to .BA form, forcing you to kill it and start over.

Second, if you disable it from the main menu, CHDUMP.BA won't reappear

(and BASIC+ won't lose the +) until you've left the main menu and returned. A

bit of a quirk, perhaps, but what do you want from a 277-byte program?

Third--and this applies to George Flanders' original version as well--

when pressing ESC/GRPH or ESC/CODE, you must first hold down GRPH or CODE, and

then press ESC. If you press ESC first, nothing happens. It has to do with the

way the M100/102 scans for keys; nothing we can do about it.

And finally, don't bother saving the installed (277-byte) CHDUMP.BA to

tape or disk, intending to reload it later to avoid hassling with the normal

installation procedure. Upon reloading, it won't be able to find itself in the

directory, so it won't work properly. In other words, if you ever remove

CHDUMP.BA, always reinstall it using the uninstalled (3K) version, as described

above.

LOAD ERROR

Packing machine language into a .BA program (as CHDUMP does) can be a

complex affair, because you can't predict where the program will reside in

memory. Since the values of certain bytes depend on the program's location,

sometimes a byte contains a value BASIC misinterprets. The Load Error message

warns you of such a byte. You must move the code higher in memory, a byte at a

time, until all bytes are acceptable to BASIC. Here's how:

1. Type NEW and press ENTER. Type KILL "CHDUMP.BA" and press ENTER.

2. Load a fresh copy of CHDUMP.BA from disk or tape, and save it to RAM as

CHDUMP.BA.

3. Edit line 10 (type EDIT 10 and press ENTER). Insert an X right after the

REM, making it REMX. Then press F8.

4. Edit line 100 (type EDIT 100 and press ENTER). Change X=0 to X=1. Then press

F8.

5. Edit line 870 (type EDIT 870 and press ENTER). Change the +5 to +6. Then

press F8.

You've just moved the program code up one byte in memory. Try running

the program again. If it still fails, repeat steps 1-5, inserting another X in

line 10 (e.g., REMXX) adding 1 more to the numbers in lines 100 and 870, and

then run it again. One or two X's should do the trick. If you've added 5 or 6

of them without success, carefully check your program listing for errors.

COMPATIBILITY

As described in George Flanders' documentation, the program uses the

CHGET hook in the Model 100/102 keyboard ROM routine. Some other programs may

alter that hook without first checking to see if it's in use, and/or without

restoring its contents afterward. See George's docs for further details. Until

you've experimented a bit and are sure of its compatibility with your other

programs, make frequent backups. (Good advice when using any new software.)

HAPPY TRAILS

Well, that's it. Use and enjoy. Catch ya! -- Nuge --

Afterthoughts: I've learned that the Gemini X10 printer uses different codes

to set up the left margin and linefeed spacing. Change last three DATA values

in line 810 from 27,108,10 (ESC"l"10) to 27,77,10 (ESC"M"10) for left margin.

Also change the first three DATA values in line 820 from 27,51,24 (ESC"3"24)

to 27,51,16 (ESC"3"16) which sets the linefeed spacing to more closely suit

the Gemini. Experiment with different values near 16 until you find one that's

pleasing to your printer.