Chapter 5              Disk-Based TEXT              Page 35

----------------------------------------

|; code for POWR-DISK                    |

|        org     60850                   |

|start:                                  |

|        jmp     psu:    ;pointer setup  |

|        jmp     xopn:   ;special open   |

|        jmp     xrd:    ;special read   |

|        jmp     xwr                     |

|<Key> for More, <ENTER> to START        |

----------------------------------------

The text at the top of the screen shows where the edit

portion will start.  When you see this screen, you must

decide from the text presented whether this is an appropriate

place to "Start" the edit portion.  If it is, press [ENTER];

if not, press any other key, in which case D-TEXT will show

you the next possible place to "Start" the edit portion.

Similarly, if you have opted to "set" the stop point for the

edit portion, a screen similar to the following will appear:

----------------------------------------

|                                        |

|        sta     scr3:                   |

|        call    tod:    ;set up         |

|        mvi     a,1                     |

|        sta     scr2:                   |

|        call    4bb8h   ;new ln if nec. |

|        mvi     m,scr1  ;next 8 files   |

|<Key> for More, <ENTER> to END          |

----------------------------------------

In this case, the text shows the last characters that will

appear in the edit portion.  If they are an appropriate place

to stop, press [ENTER]; otherwise, press any other key.

D-TEXT will continue showing you more possible places to stop

the edit portion until you either press [ENTER], reach the

end of the file, or reach the limit of the RAM.

After you have selected the limits, D-TEXT automatically puts

you into TEXT.  When you are done editing the edit portion,

press [F8] to leave TEXT.  D-TEXT will ask you "Save Changes

(y/n)?"  If you answer "n", you will return immediately to

the D-TEXT menu; if you answer "y", D-TEXT will update the

disk file with the contents of the edit portion.  This

process can take a while for a large file, so be patient.

Chapter 5              Disk-Based TEXT              Page 36

Important Note: The D-TEXT program chooses the Start and Stop

points arbitrarily.  In many cases you will see words split;

on occasion, you may even see Control-J or Control-M as the

first or last charter in an edit portion.  Do not try to

"fix" these breaks!  When D-TEXT puts the edit portion back

into the disk file, it will restore any breaks automatically.

If you "fix" them while in TEXT, whey will not go together

properly.

Notes and Options

Adding New Print Formats

It is possible to add new print formats to D-TEXT.  Each

format is in a data statement; you can add as many additional

data statements as necessary.  The DATA statement contains a

wide variety of information.  In order to simplify the

creation of a new DATA statement, we provide a small program

called PRFORM.BA, which asks the following questions:

* The number of formats already available in D-TEXT.  If

you are not sure, use [F1] to count the different

formats.  It is important to correctly answer this

question; if you answer with less than the correct

number, PRFORM will replace one of the old formats with

the new one.  (You can use this behavior to replace or

modify existing formats if you wish).

* The name of this new format.  Use 10 charters or less.

* The left margin.  How many characters from the left

should the printout start?

* The width of text.  How many characters wide should a

printed line be, maximum?

* Printed Lines per Page.  How many lines of text should

be in the body, which excludes top and bottom margins?

* Spacing.  Answer 1 for single spacing, 2 for double

spacing, etc.

* Print a header?  THe header always consists of the date,

time, file name, and page number.  If you answer yes,

you can specify the starting page number.  Often, page 2

is the first header page.

Chapter 5              Disk-Based TEXT              Page 37

* CRLF Pairs?  If your printer requires an additional line

feed with a carriage return, answer "yes".  Answer "no"

for Radio Shack Printers.

* Lines at top of page.  How many blank lines will be at

the tip of each page.  Note that this number is

independent of single spacing or double spacing.

* Does the printer accept form feed? if not, how may blank

lines at the end of the page?

* Page Pause?  Does the printing process need to pause at

the end of each page?

Once you have answered all of these questions, PRFORM will

ask you to confirm that you like your responses.  If you do,

it will automatically modify D-TEXT to add the new printer

format.

Editing Options

The file status screen gives you several options when editing

the text file.  Wrap lets you turn the "word-wrap" feature of

TEXT off for a given editing session.  Normally, you almost

always will want word wrap on, but for some data and program

files, it is nicer to work without it.  Buffer allows you to

reserve a minimum amount of RAM for adding to a files.  This

feature can come in handy if you are dealing with large files

and you want to "reserve" room for an anticipated addition.

Finally, the "Top" and "End" settings can be changed for any

file, not just large files.  Using this feature, you can

exercise considerable control over your DOcument files.

Accessing the Disk in TEXT

One highly convenient feature of D-TEXT is your access to

disk files from within the TEXT program itself.  [F2] and

[F3] can access disk files as described in Chapter 2.  This

may seem a minor convenience, but it has many interesting

applications.  Two examples:

Chapter 5              Disk-Based TEXT              Page 38

* You can use D-TEXT to concatenate large files.  Simply

use D-TEXT to "Append" onto one disk file, then loading

a second disk file onto the end using [F2].

* You can CUT and PASTE very large file segments, larger

than you could manage in RAM, by saving them to

temporary files from within TEXT.

Errors and Recovery

D-TEXT handles errors in a very similar fashion to the "menu"

phase of DMENU.  The error will be displayed on the message

line, and you must press a key to continue the program.  The

only difference is that for some errors, you will return

automatically to DMENU (or to your computer's menu) after the

error.

D-TEXT must access the disk many times when it reads and

updates files.  If an error occurs before you enter the TEXT

program, no change will have occurred to the disk, so it is

best to start D-TEXT over.  If the error persists, perhaps

the file has gotten an error in it.  If an error occurs while

you are "saving changes" to a files, there are several

possibilities.  Part of the DOcument files may be in RAM, and

the DOcument file on disk may contain some or all of the

original data.  Also, RECOVR (see Chapter 6) may be able to

find additional data on the disk.