SECTREAD

SECTREAD is a BASIC utility program to read 512-byte sectors directly

from a 3-1/2" disk in drive A: using HH/OS system interrupt &H57 (87

decimal), as described in the Tandy 600 Programmers Reference Guide.

SECTREAD calls routines in the UTILTY.LIB library file, and that

library file must be present in RAM.

SECTREAD creates a temporary file, "sector.tmp", in order to use the

512 byte file I/O buffer for storage of the sector data.  The UTILTY.LIB

DSVARPTR routine obtains the RAM location of this 512 byte buffer, and the

DBSYSINT routine uses the buffer address as the transfer address for disk

sector data.  The file is closed and killed when the program is

terminated.

To use SECTREAD, place a disk in drive A:, and RUN the program from

BASIC.  SECTREAD will display the program name and version, followed by

the message, "Initializing (30 seconds)".  Roughly 30 seconds later, the

program will prompt for the "Absolute Sector Number" which you wish to

have displayed.

The disk sector number must be entered in decimal, and should be

limited to the range: 0<=sector<=719.  The program DOES NOT validate the

sector number, although entering a number LESS THAN ZERO will end the

program.  Requesting a sector greater than the maximum absolute sector

number on the disk will cause an error.

The 512-byte sector information is displayed in hex and in ASCII.

The leftmost side of the display contains the absolute sector number, the

track number and the relative sector number (i.e. the sector within the

track, range 0-8) in hex.  The first column to the right contains the

offset within the sector, followed by sixteen columns of two-digit hex

numbers which represent the contents of individual bytes within the

sector.  The hex columns are followed by an ASCII representation of the

same data.  Two examples follow:

Tandy 600 Boot Sector (Track 0, Sector 0)

000  00 00 00 54 61 6E 64 79 36 30 30 00 02 02 01 00   ...Tandy600.....

010  02 70 00 D0 02 F8 02 00 09 00 01 00 00 00 90 90   .p..............

ABS  020  00 00 00 00 00 00 00 00 00 00 00 00 00 00 2B 0C   ..............+.

SEC  030  41 24 14 00 00 00 00 00 32 00 00 00 B2 0D 02 00   A$......2.......

000  040  00 08 14 00 14 00 7C 1D A8 09 7C 1D A8 09 41 24   ......|...|...A$

050  41 24 AD 0C B1 E5 03 F2 AE 09 00 00 AE 09 00 00   A$..............

060  00 00 00 00 C4 03 01 80 5E 0B 00 00 00 00 00 00   ........^.......

TRK  070  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

000  080  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

090  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

0A0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

REL  0B0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

SEC  0C0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

000  0D0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

0E0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

0F0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

Tandy 600 Directory Sector (Track 0, Sector 5)

000  46 4F 52 4D 41 54 20 20 21 36 30 00 00 00 00 00   FORMAT  !60.....

010  00 00 00 00 00 00 12 99 AA 0A 02 00 00 06 00 00   ................

ABS  020  49 4E 53 54 41 4C 4C 20 21 37 30 20 00 00 00 00   INSTALL !70 ....

SEC  030  00 00 00 00 00 00 28 7A CE 0A 04 00 00 04 00 00   ......(z........

005  040  44 49 53 4B 43 4F 50 59 21 39 30 20 00 00 00 00   DISKCOPY!90 ....

050  00 00 00 00 00 00 04 7A CE 0A 05 00 80 07 00 00   .......z........

060  50 52 49 4E 54 45 52 20 44 56 52 20 00 00 00 00   PRINTER DVR ....

TRK  070  00 00 00 00 00 00 B0 9A 21 0A 07 00 3D 00 00 00   ........!...=...

000  080  44 42 43 41 4C 4C 53 20 4C 49 42 00 00 00 00 00   DBCALLS LIB.....

090  00 00 00 00 00 00 79 85 93 0A 08 00 00 0C 00 00   ......y.........

0A0  42 41 53 49 43 20 20 20 21 35 35 00 00 00 00 00   BASIC   !55.....

REL  0B0  00 00 00 00 00 00 5C 51 9A 0A 0B 00 CD 84 00 00   ......\Q........

SEC  0C0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

005  0D0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

0E0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

0F0  00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00   ................

Tandy 600 3-1/2" disks are organized into 80 tracks, numbered 0

through 79.  Each track contains nine sectors, numbered 0 through 8.  Each

sector contains 512 bytes.  There are 720 absolute sectors on the disk,

numbered 0 through 719.

Track 0 Sector 0 (absolute sector 0) is the boot sector, and may

contain a bootstrap loader for the 600.  This is what loads if you perform

an "uncooperative system boot" (ESC/ALT/LABEL at power-on or reset).

Track 0 Sectors 1 through 4 (absolute sectors 1 through 4) contain

the File Allocation Tables (FAT's).

Track 0 Sectors 5 through 11 (absolute sectors 5 through 11) contain

the disk's directory entries.  The directory entries may be used to

determine the absolute sector locations of the various files on the disk.

Each 32-byte directory entry contains the following information:

Offset    Length         Content

--------------------------------------------------

&H00       08.     Filename

&H08       03.     File extension

&H0B       11.     ???

&H16       04.     Date and Time of File Creation

&H1A       02.   * Starting location of file

&H1C       02.     Length of file in bytes

&H1E       02.     ???

*  The starting location of a file is a cluster number (or whatever the

MSDOS term is).  The absolute sector may be calculated by the following

formula:

(Cluster Number x Cluster Size) + Offset = Absolute sector

or

(Cluster Number x 2) + 8 = Absolute sector number

/

//

ex