1 December 1987

LPTCOM

======

Redirect LPT I/O to COM Port

Copyright (c)  1987  by Stanley Wong

Note:  The  LPTCOM  program and associated documentation are  re-

leased  for  personal  use by Compuserve  M100SIG  members  only.

Contact the author for additional permissions.

LPTCOM  is a program which will send data written to the LPT port

of the M100/102 to the COM port.  LPTCOM is useful for people who

have serial printers.   Most Basic programs use LPRINT  statments

to  write  to the print device.   It is inconvenient to find  and

change all the print statements to use the serial port.   This is

impossible  if the program is written in machine language  (m/l).

I wrote LPTCOM because I have two parallel printers at home and a

serial  printer  at work.   I wrote a simple utility F2COM  which

prints  a text file on a serial printer but some packages  insist

on a parallel printer.   The listing for F2COM is included at the

end of this document.

LPTCOM  does not performs true I/O redirection as in CP/M or  DOS

machines.  The routine which sends data to the printer port is in

ROM  and  can  not  be changed.   If you are  interested  in  the

programming techniques used please read the file LPTCOM.NOT.

LPTCOM  requires  that a shorting plug for the  printer  port  be

built.   This  is  described  in the section  below.   Since  the

printer routine is in ROM we have to fool the M100 into  thinking

that a printer is hooked up and ready.   In fact if you hook up a

serial AND a parallel printer at the same time,  your output will

be printed on both machines.

The program F2LPT can be used to test your setup.  The listing is

included at the end of this document.

Create  a  LPTCOM.BA  file and run  in  the  usual  fashion.  The

comments lines 1-6,15,19,39,  and 89 can be deleted while in your

machine.   The  program is 1317 bytes long with comments and 1023

bytes without.

The main LPTCOM menu offers four choices:

I)nitialize

L)PT-to-COM

C)OM LF

T)op Menu

LPTCOM						   Page 2

Initialize

----------

If  the  MDM or COM ports have been used for  any  other  purpose

since  Initialize  was last run you must re-initialize the  port.

The port settings are in line 22.   It is currently set to 88N1E.

Change  it  to  suit your serial device.   LPTCOM also  uses  the

memory  reserved for the F7 function key.   You can not  use  any

program  that defines F7 and LPTCOM.   If a program that you  use

sets  F7  and  later  you want to print  to  COM,  you  must  run

Initialize again.

LPT-to-COM

----------

Select  this  option  to enable or disable  the  I/O  redirection

feature.

COM LF

------

If  your serial printer (or device) requires that a LF  character

be sent after every CR, enable this option.

Top Menu

--------

Return to the M100/102 main menu.

Shorting Plug

=============

Connect  a wire jumper between pin 21 (BUSY) and any of the  even

number  pins 2-24.   Pin number 21 is the third from the left  on

the  top  row  as you look at the printer  connector.   The  even

number  pins  2-26 are on the bottom row.   Any  pin  except  the

leftmost  one  (pin  26)  will do.   There is a  picture  of  the

connector in your users manual.

Below  is a list of possible ways to construct the shorting plug.

It  is  not  meant to be a  comprehensive  list.   The  suggested

solutions are given in order of "most permanent" solutions.

** To create a permanent shorting plug buy a 26-pin connector and

add a jumper wire.   To aid in removing the plug you may want  to

add  a short length of ribbon cable to the plug and then tie wire

21 to ground (even pins 2-24).

**   Use  the  standard printer cable and add  a  jumper  at  the

Centronics  plug  end of the cable.   The printer cable  for  the

LPTCOM						   Page 3

TTXpress  printer has serial DB25 connector even though it  is  a

parallel  printer.   It is easier to hook a jumper to serial pins

that Centronics pins.

**  Add a jumper to the printer port connector.   I used a  short

push-on  jumper of the type used for wirewrap posts.   This makes

for a handy "emergency" jumper if you are on the road and need to

use a serial printer.  Be careful though,  if you bump the jumper

wire you can bend the connector posts (I did).

**  Use a paper clip on the printer connector.   It's awkward but

it works.  I used this method for initial concept testing.

F2LPT Listing

-------------

F2LPT  is a simple program to help you check out your setup.   It

prints a text file to the printer via Basic LPRINT statements.

1 'F2LPT - Test LPTCOM setup

2 'Stan Wong CIS [70346,1267]

10 MAXFILES=2:CLS

20 PRINT:PRINT"Text File? ";:LINEINPUTA$

30 OPENA$FORINPUTAS1

40 IFEOF(1)THEN100:LINEINPUT#1,B$:LPRINTB$:GOTO40

100 CLOSE:MENU

F2COM Listing

-------------

F2COM  is  a simple Basic program which prints a text file  to  a

serial printer through the serial port.   The port parameters are

set  in  line  30.   The current setting is  88N1E  (9600  baud).

Change it to suit your particular configuration.

1 'F2COM - Print a TEXT file on a serial printer

2 'Stan Wong - CIS [70346,1267]

10 MAXFILES=2:CLS

20 PRINT:PRINT"Text File? ":LINEINPUTA$

30 OPEN"COM:88N1E"FOROUTPUTAS1

40 OPENA$FORINPUTAS2

50 IFEOF(2)THEN100:LINEINPUT#2,B$:PRINT#1,B$:GOTO50

100 CLOSE:MENU

=======================================

Acknowledgements:  The  LPTCOM program borrows several ideas from

Phil Wheeler's LFUTL.PW1 program.   In particular the idea to use

the function key definition area for the m/l code is from LFUTL.

Please   send  comments  and  bug  reports  to  Stan   Wong   CIS

[70346,1267].

######################################################################################