# Hardware Scrolling Patch for the Model 100

S. Adolph

March 2022

## Introduction

This patch is intended to reduce the scrolling times for the Model 100 and Tandy 102 laptops. The 10 LCD drivers used (HD44102) support hardware scrolling.  Implementation of hardware scrolling should reduce the time to scroll the screen, since currently the computer manually updates every location in the LCD RAM as well as the drivers themselves.  Hardware scrolling should reduce the time it takes to copy data into the LCD drivers.

## Theory of Operation

Currently, the model 100 scrolls the screen manually, by copying data line by line to both the LCD RAM as well as the LCD drivers.

The HD44102 device is capable of a hardware scrolling mechanism that allows lines to be quickly shifted, leaving only the last line requiring new data.  Please refer to the datasheet for the HD44102 device.  This device displays data vertically on the LCD according to the contents of the internal page register.  In the M100, this register is always 00.  However, writing (00, 01, 10, 11) allows the order of display of the 4 vertical pages to be shifted.

Further, the page register can be assigned simultaneously for all 10 LCD drivers.  Once the pages have been shifted, there remains only 1 line of data to update with new text per group of 5 LCD drivers.  The M100 display is organized such that 5 LCD drivers control the top half, while the remaining 5 drivers control the bottom half.

The approach to improved scrolling will be as follows:

- Changes will be implemented in the main ROM by using a pre-existing modification that clears about 150 bytes for new code (the “base patch”)

- the page register will be to scroll the LCD driver data

- the current “top of page” indication will be stored in RAM, using data bytes in “Hayash” directory entry (avoiding TS-DOS use of this space as well)

- Several internal ROM routines will be redirected into the 150 byte space, where the necessary changes to the routines will be made.

When the entire screen is scrolled, 2 hardware scrolls, one line erase, and one copy are required.  Screen scroll is implemented primarily by intercepting the scroll up and scroll down control code sequences, ESC-M and ESC-L.   When a full screen scroll is detected, the general process that is followed is described here:

- ESC-M or ESC-L is detected

- The register contents indicate if the entire screen is to be scrolled.  If not the full screen then the routine operates unmodified.

  - When less than the entire screen is being scrolled, hardware scrolling may actually bring benefits but (1) it is much harder to implement and (2) in general the benefit is reduced, many more lines must be copied.

  - A way to defeat hardware scrolling is to enable the LABEL LINE.  This patch is disabled whenever LABELS are displayed

- For the first half of the screen scroll,  the routines that copy data from LCD RAM to the LCD drivers is disabled, and half of the LCD drivers are hardware scrolled

- In order to copy 1 line of data from one set of drivers to the other, this copy routine is re-enabled for the needed line copy only

- Then this routine is again disabled for the 2nd half of the screen scroll, and the remaining drivers are then hardware scrolled.

- Lastly the existing routine erases the new line.

The page register values in use in the LCD drivers must be stored in memory, as it cannot be read back from the LCD drivers themselves.  The screen update ROM routines must be aware of the state of the LCD drivers in order to update individual characters correctly.

## Compatibility with existing M100 software

This patch should be compatible with the standard model 100 and T102.  This patch will not be compatible with any of the enhanced screen utilities.  Changes to these rom routines will not affect DVI usage.  Testing and bug reports are always appreciated.

## Proposed RAM location for Controller page register data

Since the page register is write-only, it will be necessary to track the state of the page register, so that software may identify actual screen lines with pages within the controller’s memory.  This memory location should survive power cycles, but should be reset to ‘00’ on resets, and warm/cold boots.  It is required that this memory location should survive unchanged during power cycles, since the controllers themselves will not restart to the same page register state, and since programs themselves continue to operate through power cycling.  The screen data in memory must be displayed identically before and after a power cycle.

On cold or warm restart, the screen is completely re-freshed, so simply restoring page to ‘00’ is sufficient.

One memory location will be used to store the page register state.  This is needed to facilitate the separate scroll process for top and bottom LCD driver groups.

Location F9AD is the current proposal.  This byte is the s in the name “Hayash” stored in ROM at 6C3C.  This byte needs to be patched to 00 in the ROM at 6C3C also.  They appear to survive correctly and not affect the operation of the machine.  Additional data is stored in the name, so in total 2 bytes in “Hayash” will be used.  There is always a chance that some other software will make use of this location; testing is required to understand any conflicts.

The page byte may only take on the values 00H, 40H, 80H, C0H.  No error checking will be provided to ensure this is adhered to.

## Analysis of scrolling functions

As stated above, hardware scrolling is only implemented in the case where the entire screen is scrolled.  The below tables show in general how the lines are moved during scrolling, for some of the common scroll conditions.

In general, the scroll routines shift lines either up or down depending on where on the screen the escape code is sent.

Opportunities to hardware scroll either top or bottom are indicated in blue. In these cases three lines can be shifted by hardware, with the last line being copied in.

Situations that must remain as software scrolls are indicated in pink.  All lines must be copied via software, with no hardware scrolling

Situations where a variant of hardware scroll might bring benefit are indicated in orange.  In such cases 2 lines may be hardware scrolled, while 2 lines must be software copied.

When the LABEL line is activated, the bottom half of the screen can never benefit from a full hardware scroll.

Different scenarios for Scroll Up and Scroll Down are captured below.

Scroll Up

| hw scroll | sw scroll | special case |
| --- | --- | --- |

- LABEL line active

|  |  | Line | line | Line | line | Line | line | Line | line |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scroll Up | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  | line 8 std |  |  |  |  |  |  |  |  |
| line L | Original state |  |  |  |  |  |  |  |  |
| 0 | A | B | A | A | A | A | A | A | A |
| 1 | B | C | C | B | B | B | B | B | B |
| 2 | C | D | D | D | C | C | C | C | C |
| 3 | D | E | E | E | E | D | D | D | D |
| 4 | E | F | F | F | F | F | E | E | E |
| 5 | F | G | G | G | G | G | G | F | F |
| 6 | G | H | H | H | H | H | H | H | G |
| 7 | H | blank | blank | blank | blank | blank | blank | blank | blank |
|  |  | 2 HW | 1 HW, | 1 HW, | 1 HW | 1 HW |  |  |  |
|  |  | 1 copy | 3 copies | 2 copies | 1 copy |  | 2 copies | 1 copy |  |

- LABEL line inactive

|  |  |  | line | Line | line | Line | line | Line | line |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scroll Up |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
|  | line 8 lock |  |  |  |  |  |  |  |  |
| line L | Original state |  |  |  |  |  |  |  |  |
| 0 | A | B | A | A | A | A | A | A |  |
| 1 | B | C | C | B | B | B | B | B |  |
| 2 | C | D | D | D | C | C | C | C |  |
| 3 | D | E | E | E | E | D | D | D |  |
| 4 | E | F | F | F | F | F | E | E |  |
| 5 | F | G | G | G | G | G | G | F |  |
| 6 | G | blank | blank | blank | blank | blank | blank | blank |  |
| 7 | H | H | H | H | H | H | H | H |  |
|  |  |  | 1 HW, | 1 HW, | 1 HW | 1 HW |  |  |  |
|  |  |  | 3 copies | 2 copies | 1 copy |  | 2 copies | 1 copy |  |

Scroll Down

- LABEL line active

|  |  | Line | line | Line | line | Line | line | Line | line |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scroll Down | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  | line 8 std |  |  |  |  |  |  |  |  |
| line L | Original state |  |  |  |  |  |  |  |  |
| 0 | A | blank | A | A | A | A | A | A | A |
| 1 | B | A | blank | B | B | B | B | B | B |
| 2 | C | B | B | blank | C | C | C | C | C |
| 3 | D | C | C | C | blank | D | D | D | D |
| 4 | E | D | D | D | D | blank | E | E | E |
| 5 | F | E | E | E | E | E | blank | F | F |
| 6 | G | F | F | F | F | F | F | blank | G |
| 7 | H | G | G | G | G | G | G | G | blank |
|  |  | 2 HW | 1 HW, | 1 HW, | 1 HW | 1 HW |  |  |  |
|  |  | 1 copy | 3 copies | 2 copies | 1 copy |  | 2 copies | 1 copy |  |
|  |  |  | line | Line | line | Line | line | Line | line |

- LABEL line inactive

|  | Scroll Down | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | line 8 lock |  |  |  |  |  |  |  |  |
| line L | Original state |  |  |  |  |  |  |  |  |
| 0 | A | blank | A | A | A | A | A | A |  |
| 1 | B | A | blank | B | B | B | B | B |  |
| 2 | C | B | B | blank | C | C | C | C |  |
| 3 | D | C | C | C | blank | D | D | D |  |
| 4 | E | D | D | D | D | blank | E | E |  |
| 5 | F | E | E | E | E | E | blank | F |  |
| 6 | G | F | F | F | F | F | F | blank |  |
| 7 | H | H | H | H | H | H | H | H |  |
|  |  | 1 HW, |  |  |  |  |  |  |  |
|  |  | 3 copies | 5 copies | 4 copies | 3 copy | 2 copies | 1 copy |  |  |

## LCD ports on the M100

185, B9  LCD driver select, bits 7..0

186, BA  LCD driver select, bits 0,1

254, FE  LCD driver control and status – bits 7,6 – bank select -  bits 5-0 byte select

255, FF  LCD driver data

## LCD controller physical mapping by bit number

0	1	2	3	4

5	6	7	0	1

port 185, port 186

## Hardware Scrolling Functional Description / Patches

Routines that deal with scrolling directly:

44C4H 	Escape M routine

44D2H		Scroll up

44EAH	Escape L routine

44FAH	Scroll down

4566H		Character plotting

Routines that are affected by introducing LCD controller page register state

74F7H 	Byte plotting

752BH		Set Page

## Patch Descriptions

Escape-M @ 44C4H

- Hardware scroll condition is detected

- Scroll active flag is set

- hardware scroll for top and bottom drivers at the same time

- return to 44D2H routine

Scroll up @ 44D2H

- here, the return from software scrolling is intercepted

- scroll active flag is disabled here

Escape-L @ 44EAH

- Hardware scroll condition is detected

- Scroll active flag is set

- hardware scroll for top and bottom drivers at the same time

- return to 44EAH routine

Scroll down @ 44FAH

- here, the return from software scrolling is intercepted

- scroll active flag is disabled here

Character plotting @ 4566H

- here the scroll active flag is tested

- if a scroll is active then we look for the special condition where the line being “software scrolled” is occurring

- when this happens, we actually employ the routine at 73EEH to copy the new line into the LCD drivers, the source being Video RAM

- if scroll is not active, then carry on to 73EEh

Changes to “Hayash” @ 6C73H

- 1 byte for page register storage

- 1 byte for scroll active flag

- These 2 bytes are copied to RAM.

Byte plotting @ 74F7H

- The value of the b register must be corrected to use the page register

Set Page @ 752BH

- The page setting routine is modified to use the page register

- In addition, this routine is used to set page for the ESC-M and ESC-L routines.

## Patch Details

The main ROM is intercepted in the following locations

Escape-M @ 44C4H

|  | original | patch |
| --- | --- | --- |
| 44CFH | JZ 4535H | JMP TRAP_M |

Scroll up @ 44D2H

|  | original | patch |
| --- | --- | --- |
| 44E7H | JMP 4535H | JMP RET_M |

Escape-L @ 44EAH

|  | original | patch |
| --- | --- | --- |
| 44F9H | JZ 4535H | JMP TRAP_L |

Scroll down @ 44FAH

|  | original | patch |
| --- | --- | --- |
| 44FAH | JMP 4535H | JMP RET_L |

Character plotting @ 4566H

|  | original | patch |
| --- | --- | --- |
| 4573H | CALL 73EEH | CALL stop_access |

Changes to “Hayash” @ 6C3CH

|  | original | patch |
| --- | --- | --- |
| 6C3CH | “sh” | .db 00h, 00h |

Byte plotting @ 74F7H

|  | original | patch |
| --- | --- | --- |
| 74F7H | PUSH D PUSH PSW MOV A,B | CALLcorrect_b |

Set Page @ 752BH

|  | original | patch |
| --- | --- | --- |
| 752BH | CALL 7533H MVI A,3EH | call set_page ori 3EH |

Patch code @ 75AB

;---------------------------------------------------------------------

; Hardware Scroll Patch

;---------------------------------------------------------------------

page_loc:		.equ	0F9ADH

scroll_active:	.equ	0F9AEH

.org	0000

;---------------------------------------------------------------------

; hook into Scroll down

;---------------------------------------------------------------------

.org	044CFH

call	TRAP_M

.org	44E7H

jmp	RET_M

;---------------------------------------------------------------------

; hook into Scroll up

;---------------------------------------------------------------------

.org	044F6H

call	TRAP_L

.org 	0450FH

jmp	RET_L

;---------------------------------------------------------------------

; hook into 4566 routine

;---------------------------------------------------------------------

.org	4573H

call	stop_access

;---------------------------------------------------------------------

; change to Hayash default value

;---------------------------------------------------------------------

.org	06C3CH

.db	00h		; page_loc data 6C3C -->F9AD

.db	00h		; scroll status 6C3D -->F9AE

;---------------------------------------------------------------------

; hook into byte plotting

;---------------------------------------------------------------------

.org	74F7H

jmp	correct_b

;---------------------------------------------------------------------

; hook into set page

;---------------------------------------------------------------------

.org	0752BH

call	set_page

ori	3EH		; called from set_page

out	0FEH

ret

;---------------------------------------------------------------------

; Patch code  - place in hole at 75AB - 763F

;---------------------------------------------------------------------

.org	75ABH

;---------------------------------------------------------------------

; TRAP_M (11 bytes)

;---------------------------------------------------------------------

TRAP_M:				; A holds # of scrolls, L is the line number to process first

; return vector on stack

pop	d

jz	4535H		; if zero set, erase current line and done.. return to caller

push	d

; do a scroll up on upper and lower LCD drivers

lxi	d,0C028h	; d= C0h, e = 40d, 28H

jmp	TRAP_C

;---------------------------------------------------------------------

; TRAP_L  (22 bytes)

;---------------------------------------------------------------------

TRAP_L:				; A holds # of scrolls, L is the line number to process first

; return vector on stack

pop	d

jz	4535H		; if zero set, erase current line and done.. return to caller

push	d

; scroll down on upper and lower LCD drivers

lxi	d,04027h	; d= 40h, e = 39d, 27H

;	jmp	TRAP_C

TRAP_C:

cpi	07d

rnz 			; if a <>7 then return and process normally

push	psw

mov	a,e

sta	scroll_active

push	h

call	set_new_page

pop	h

pop	psw

ret

;---------------------------------------------------------------------

; RET_M  RET_L (6 bytes)

;---------------------------------------------------------------------

; a is always zero on jump in.

; use this to just clear the scroll active flag

RET_M:

RET_L:	;xra	a

sta	scroll_active	; disable scroll active

jmp	4535H

;---------------------------------------------------------------------

;  stop_access  (14 bytes)

;---------------------------------------------------------------------

; prevent character plotting level 7 when hardware scrolling

stop_access:

lda	scroll_active

ora	a

jz	073EEh		; if scroll not active then continue with character plotting

; scroll is active

add	e		; sum of e and a

cpi	44d		; some number out of normal range but same for both

jz	073EEh		; process the line if we are in the copy condition

ret			; if scroll not active then continue with character plotting

;---------------------------------------------------------------------

; set_new_page (6 bytes)

;---------------------------------------------------------------------

set_new_page:			; d holds subtract amount, 40 for down and C0 for up

lxi	h, page_loc		; set location

mov	a,m

sub	d

mov	m,a

;---------------------------------------------------------------------

; set_page  (15 bytes)

;---------------------------------------------------------------------

set_page: 	;  752EH is on stack

mvi	c,03d

call	7657H		; short delay

lxi	h,7641H

call	0753BH		; enable LCD drivers

lda	page_loc	; load page data

ret

;---------------------------------------------------------------------

; correct_b   (13 bytes)

;---------------------------------------------------------------------

correct_b:		; all registers in use

push	d

push	psw		; store PSW

lda	page_loc	; ok to just use the one page value as they should be the same

Add	b		; EXCEPT during a page scroll

mov	c,a		; temp store

pop	psw

push	psw		; make sure flags are the same as initial

mov	a,c		; restore, hopefully C is not used

jmp	074FAH		; jump back into routine

; just leave B as is; manipulate A to be the corrected page

Initialization of page.

Routine at 752BH initializes the page to 00.

This routine is called at:

- warm start routine at 6D0CH calls 752BH which initializes page register to 00

- character plotting level 7 calls 752BH which initializes page to 00

Need to confirm that on power up, this is configured.   Should be.  Do we care?  Whatever the value is, all that matters maybe is that for any one session it is only changed to capture scrolls.

Therefore, a patch at the routine 752BH is required to load the contents of F9AAH and send that to the LCD drivers.

Setting page.

Page register is set via write to port 254 of values (62, 126, 190, 254) (3E, 7E, BE, FE) – probably 62 = 00111110

OUTput xx11 1110 binary on port FE as per 752BH, where xx is page. 00, 01, 10, 11.

Any routine that writes to the LCD at a low level must adjust how data is written to accommodate the page setting.

Note:  it is probably ok to power cycle, and have the page be set correctly simply through this one patch.  On power up, the page value will be read back and applied.

## The “Base Patch”

The hardware scrolling patch builds on the so-called Base patch, which provides about 150 free bytes to use for modifications to the M100 ROM.

The basis of this patch is the observation that the PIO data table could be reduced from 240 bytes to 80 bytes.  This change forces a rewrite of a short section of a routine.

## Changes to 8155 PIO bit patterns for upper/lower LCD drivers at 7551H

New LCD data table at 7551H to 75A0H.  Table is reduced from 240 bytes to 80 bytes.  This table was reduced by noting the following-

- the data was redundant with the table at 7643H, so the repetition could be removed (240 160 bytes).

- the upper and lower LCD table differences can be managed via the table at 7643H also. (160 80 bytes)

The original 2 tables are organizes as triplets.  The first two bytes are redundant with table 7643H (and so can be replaced by a pointer), while the remaining bytes is the column offset data for preparing the command byte for the LCD driver.

In the new table, the data is organized as pairs.  The first byte in the pair provides an offset to table 7643H, for providing programming data for port 185 and 186.  The next byte in the pair is the column offset data for preparing the command byte for the LCD driver.

This reduction in data space comes at the expense of about 10 bytes of code, and changes in subroutine 74A2H – byte plotting.  The space from 75ABH to 763F1H is free for use by routines.

The original table at 7551H was about 240 bytes:

Upper LCD driver data – 7551 to 75C8 – 15x8 = 120 bytes

Lower LCD driver data – 75C9 to 7639 – 15x8 = 120 bytes

The new table at 7551H is from 7551 to 75A0 – 10x8 = 80 bytes :

7551H	00H,00H,00H,06H,00H,0CH,00H,12H,00H,18H,00H,1EH,00H,24H,00H,2AH,00H,30H

02H,04H,02H,0AH,02H,10H,02H,16H,02H,1CH,2H,22H,02H,28H,02H,2EH

04H,02H,04H,08H,04H,0EH,04H,14H,04H,1AH,04H,20H,04H,26H,04H,2CH

06H,00H,06H,06H,06H,0CH,06H,12H,06H,18H,06H,1EH,06H,24H,06H,2AH,06H,30H		08H,04H,08H,0AH,08H,10H,08H,16H,08H,1CH,08H,22H

To use the new table, changes are needed to the routine PLOT/UNPLOT at 744DH.

At 7467H, swap this code

|  | Original | patch |
| --- | --- | --- |
| 7467H | JNC 746DH | LXI H,7643H |
| 746AH | LXI H,764DH | CALL 75A1H |
| 746DH | DAD B | NOP |
| 746EH | MOV B,A | NOP |

Which loads HL with a pointer to the table at 7643H, and then calls a patch routine at 75A1H.

| 75A1H | JNC 75A6H |
| --- | --- |
| 75A4H | MVI L,4DH |
| 75A6H | MVI B,00H |
| 75A8H | DAD B |
| 75A9H | MOV B,A |
| 75AAH | RET |

Here, the pointer is corrected depending on the carry flag.  B is assigned the contents of A, and HL points to the correct data in the table at 7643H.

In fact this patch is not needed for the PLOT/UNPLOT routine directly, but it allows for the next change to be more efficiently accomplished.  In effect this is a hook into this routine for future use.

The Byte plot routine at 74A2H is the main modification.  At 74BBH, the routine is rewritten to be the following.

| 74BBH | RLC | ROTATE ACCUMULATOR LEFT (*2) |  |
| --- | --- | --- | --- |
| 74BCH | MOV C,A | PLACE RESULT IN C |  |
| 74BDH | MOV B,00H | LOAD B WITH 00 |  |
| 74BFH | LXI H,7551H | POINT TO TABLE |  |
| 74C2H | DAD B | ADD OFFSET |  |
| 74C3H | MOV C,M | LOAD NEXT TABLE OFFSET BYTE IN C |  |
| 74C4H | INX | INCREMENT HL |  |
| 74C5H | MOV A,M | LOAD COMMAND DATA IN A |  |
| 74C6H | PUSH PSW | push command data |  |
| 74C7H | LDA FFF4H | LOAD A WITH ROW DATA |  |
| 74CAH | RAR | ROTATE RIGHT |  |
| 74CBH | RAR | ROTATE RIGHT |  |
| 74CCH | RAR | ROTATE RIGHT |  |
| 74CDH | LXI H,7643H | Point to table |  |
| 74D0H | CALL 75A1H | Adjust pointer to correct location |  |
| 74D3H | CALL 753BH | ; Enable LCD drivers after short delay |  |
| 74D6H | SHLD FFF6H | STORE POINTER TO LCD TABLE |  |
| 74D9H | POP PSW | reload A with command data |  |
| 74DAH | ORA B | FORM THE DATA WORD |  |
| 74DBH | MOV B,A | PLACE WORD IN B |  |
| 74DCH | POP H |  |  |
| 74DDH | DCR D |  |  |
| 74DEH | CALL 74F7H | this routine sends a byte as a command, and reads or writes E values depending on D |  |
| 74E1H | INR D | restore D to original value |  |
| 74E2H | MVI A,06H | load A with 06 |  |
| 74E4H | SUB E |  |  |
| 74E5H | RZ | if 6 bytes were intended to be written, then return, else the remainder must be written to the adjacent driver |  |
| 74E6H | MOV E,A | put the remainder into E |  |
| 74E7H | PUSH H |  |  |
| 74E8H | LHLD FFF6H |  |  |